3GPP SA WG2#169	S2-2505039

Fukuoka, Japan, 19-23 May, 2025

Source:	Tejas Networks Limited

Title:	Solution for KI#1: System Architecture to support sensing

Spec:	3GPP TR 23.700-14

Agenda item:	20.2.1

Work Item / Release: FS_Sensing_ARC/ Rel-20

Document for:	Discussion and decision

Abstract of the contribution: This contribution proposes a new solution to Key Issue#1: system architecture for sensing.

1. Introduction

A new SID on architecture enhancement to support Integrated Sensing and Communication was approved in SP-250401 with several objectives. A key objective of this SID as presented in WT#1 is the study of architectural aspects for sensing comprising new and/or exisiting functions in the core network and their interactions for supporting sensing.

This paper proposes a new solution for integrating sensing capabilities in the 5G system by introducing new Network Functions(NFs) within the core network and enhancing capabilities of some of the existing 5GC NFs.

2. Text Proposal

It is proposed to adopt the following text within TR 23.700-14.

* * * * First Change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> | <Key Issue #4> | <Key Issue #5> | <Key Issue #6> |
| X | X |  |  |  |  |  |

* * * * Next Change (All New Text) * * * *

## 6.X	Solution #X: 5G architecture enhancements to support sensing

### 6.X.1	Description

Introduction of sensing in 5G networks enables the acquisition of information regarding the characteristics of the environment and/or objects within the environment. This enhances the capabilities of 3GPP 5G system and makes it possible to support a multitude of sensing use-cases as described in 3GPP TR 22.137 within the cellular network infrastructure.

Using the 5GC specified in TS 23.501 as a starting point, three new network functions viz., Sensing Control Function (SCF), Sensing Data Function (SDF) and Sensing Data Repository (SDR) as illustrated in Figure 1 are introduced to equip the 5G system with sensing capabilities.

Figure 1:Architectural enhancements to support sensing.

The proposed enhancements are in alignment with the Control and User Plane separation (CUPS) paradigm, facilitating the independent scaling of each of the NFs based on the requirements. Note that, although three different NFs have been proposed, flexibility is also provided to the network operator to deploy SCF, SDR and SF as a single network function i.e., Sensing Function (SF) as a single-box solution for the sake of convenience.

Details on the functional description of the newly introduced NFs are provided below:

Sensing Control Function (SCF): The SCF handles control plane signaling related to sensing. It is the first NF contacted by the sensing application through the NEF to initiate the sensing task. More than one SCF may be present within the system to enable sensing over large geographical areas. The SCF is also responsible for selecting the set of NFs which will participate in the sensing activity. The selection of various NFs is carried out based on several factors not limited to nature of the sensed object, geographical area of interest, available system resources, duration of the sensing task, required sensing accuracy etc. The sensing control function supports a service-based interface known as Nscf and can interact with other functions such as Network Exposure Function (NEF), Network Data Analytics Function (NWDAF) etc. It is also responsible for routing the sensing data to be either stored in the Sensing Data Repository (SDR) or to be sent towards a data network after being processed by a Sensing Data Function (SDF). SCF is responsible for selecting sensing modes and the participants in the sensing activity in co-ordination with other NFs based on the details provided by the sensing application.

Sensing Data Function (SDF): The SDF is a data plane function that is responsible for processing the sensing data within the core network. It is responsible for routing the sensing data and also for providing an identifier to the tracked object/entity. This function is also responsible for ensuring that the sensing metrics are met.

Sensing Data Repository (SDR): SDR is a database which stores multiple types of data related to sensing. One such dataset would include an object type-characteristics map which may be used to identify a sensed object. The SDR may also be used to store sensed data which can be used by other NFs such as NWDAF for further processing.

Within the proposed 5G system, sensing is provided as service. Therefore, an application function initiates and updates/revokes the sensing task. The clients requesting sensing services are authenticated by the sensing application and authorized to access the sensing data. The application function is also responsible for providing the requisite metrics such required Quality of Service (QoS), required horizontal and vertical accuracy, response time etc. to the the 5G system.

* * * * Next Change (All New Text) * * * *

### 6.X.3	Impacts on Services, Entities and Interfaces

The following impacts are envisioned on the existing network functions:

NEF:

- For supporting sensing services, NEF performs functions such as exposure of capabilities and events for other NFs, translation of information between an AF and another NF, support of non-IP data delivery etc.

NRF:

- NRF maintains the NF profile of available NF instances related to sensing (e.g., SCF, SDF and SDR) and their supported services

NWDAF:

- For a given sensing task, NWDAF may support data collection from SDR, SDF and/or SCF, whenever required. It also supports all tasks related to computation of analytics on sensing data such as data collection, model provisioning and training, accuracy information computation etc.

* * * * End of Changes * * * *