3GPP SA WG2 Meeting #169	S2-2505049

Fukuoka, Japan, 19th May 2025 - 23th May 2025	(revision of S2-250xxxx)

Source: 	LG Electronics

Title: 	KI#1: New Sol: Optimisation of IMS voice call initiation over NB-IoT (GEO) when the UE is in Idle

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper proposes a new solution to KI#1 to TR 23.700-19.

1	Discussion

Regarding Key Issue #1: Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC captured in clause 5.1 of TR 23.700-19, this paper proposes a new solution.

2	Text Proposal

It is proposed to agree the following changes into TR 23.700-19.

* * * * Start of 1st Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X | X |  |  |  |

* * * * Start of 2nd Change * * * *

!! All New Texts !!

## 6.X	Solution #X:  Optimisation of IMS voice call initiation over NB-IoT (GEO) when the UE is in Idle

### 6.X.1	High Level Principles

The high level principles of the solution are:

-	The UE and Network Support both CP CIoT EPS optimisation and S1-U data transfer.

-	The UE uses IMS user plane for IMS signalling and voice when the UE is in ECM-CONNECTED.

-	The UE indicates active flag in Control Plane Service Request and also includes SIP INVITE in ESM message container when UE the initiate voice call in ECM-IDLE. This activates S1-U bearer and user plane radio bearers for IMS PDN connection.

-	The rest of IMS procedure continues with user plane.

### 6.X.2	Description

The UE triggering IMS voice call over CP have advantage of saving call setup time because Control Plane Service Request message can contain the INVITE message, therefore the waiting time for service request and service accept from the network can be saved. To support QoS of IMS voice data, dedicated GBR bearer is needed over NB-IoT RAT.

This solution describes the enhancement based on the CP CIoT EPS optimisation and establishment of S1-U bearer during Data Transport in CP CIoT EPS Optimisation specified in clause 5.3.4B.4 of TS 23.401 [5]. Basically, the UE uses user plane default bearer for IMS signalling and dedicated bearer for IMS voice when the UE is in ECM-CONNECTED. But when the UE is in ECM-IDLE, the UE use the Control Plan Service Request to send SIP INVITE message with indicating active flag. The MME forwards the SIP INVTE encapsulated in the ESM message to SGW via S11-U interface and establishes S1-U bearer. Then the rest of IMS SIP signalling and voice are transferred via user plane default and dedicated bearers as existing procedure.

NOTE:	Since the NB-IoT UE only support maximum 2 data radio bearers, the MME need to set Control Plane Only Indicator for all other PDNs except for IMS PDN.

After the voice call end the eNB may trigger the S1 release procedure (e.g. due to user inactivity), the MME releases UE context to the eNB and release the radio bearers.

Since the proposed solution only optimize the call initiation part when the UE in ECM-IDLE, it can be used together with other solutions.

### 6.X.3	Procedures

Figure 6.X.3-1: High-level procedure for IMS voice over NB-IoT (GEO)

0.	A GEO satellite of a NB-IoT access broadcasts that it supports IMS voice service over NB-IoT in SIB.

1.	UE provides Attach Request to the NB-IoT (GEO) satellite including Voice domain preference indicates IMS PS Voice only and UE's usage setting indicates voice centric, the Preferred Network Behaviour indicates both Control Plane CIoT EPS Optimisation and S1-U data transfer are supported. The UE may use IMS PDN as attach PDN.

The MME proceeds the Attach request from the UE. If the attach is successful, it sends an Attach Accept message to the UE. The MME indicates both Control Plane CIoT EPS Optimisation and S1-U data transfer are supported in the Supported Network Behaviour information. The MME also indicates IMS voice over NB-IoT Support indication.

If the UE is not attached using the IMS PDN, the UE establishes the IMS PDN Connection.

2.	After Attach or PDN Connection procedure, the UE in ECM-CONNECTED performs IMS registration procedure over the IMS PDN Connection (user plane).

3.	The eNB may trigger S1 release procedure (e.g. due to user inactivity) and the RRC Connection is released.

4.	The UE initiates a voice call using SIP INVITE message over the Control Plane using CP CIoT EPS Optimisation to the MME. The UE includes the SIP INVITE in the ESM DATA TRANSPORT message of ESM message container in the Control Plane Service Request and indicates active flag towards the MME.

5.	If the S11-U connection is not established, the MME sends and receive Modify Bearer Request/Response with SGW.

6.	The MME forwards the uplink data (SIP INVITE message) from the UE through S11-U bearer.

7-9.	Based on the Control Plane Service Request with active flag, the MME sends S1-AP Initial Context Setup Request message to the eNodeB for the IMS PDN connection to activate user plane radio bearers and S1-U bearers.

10.	The MME sends a Modify Bearer Request message to setup S1-U User Plane.

As the user plane radio bearer is setup, the UE uses user plane bearer to transfer IMS signalling data..

11.	SIP 100 Trying signalling is transferred over the default EPS bearer (UP)

12-14.	P-CSCF sends to PCRF the AA-Request to activate a dedicated bearer for voice media. Dedicated bearer activation procedure specified in clause 5.4.1 of TS 23.401 [5] is performed, and the PCRF responds to P-CSCF with the AA-Answer.

15.	P-CSCF forwards the SIP 183 progress over the default EPS bearer (UP).

16.	The procedures for IMS voice call setup continues over the default EPS bearer (UP).

17.	IMS voice media is transferred over the dedicated GBR EPS bearer (UP).

18.	The voice call end.

19.	The eNB may trigger S1 release procedure (e.g. due to user inactivity) and the RRC Connection is released.

### 6.X.4	Impacts on services, entities and interfaces

UE:

-	Support dedicated bearer over NB-IoT access for GBR.

-	Indicates active flag and includes ESM message container to MME using Control Plane Service Request when voice call start in ECM-IDLE.

eNB:

-	Indicate support of IMS voice over NB-IoT GEO satellite in SIB.

-	Support dedicated bearer over NB-IoT (GEO) access for GBR.

MME:

-	Indicate to the UE the support of IMS voice over NB-IoT GEO satellite

-	Support dedicated bearer over NB-IoT (GEO) access for GBR.

PGW:

-	Support dedicated bearer over NB-IoT (GEO) access for GBR.

* * * * End of Changes * * * *