3GPP TSG-SA WG2 Meeting #169	S2-2505062

Fukuoka, Japan, May 19 – 23, 2025

Source:	Samsung

Title:	AIOTF Service Update

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The contribution proposes to update AIOTF Service for AIoT.

1. Discussion

AIOTF Service procedure needs to be described in the normative text in TS 23.369 regarding the following editor’s notes.

Editor's note:	It is FFS how to support the service between AMF and AIOTF in order to report and update the information provided by the RAN for indirect connectivity.

For the above EN, AMF sends the Namf_AIoT_Notify message (AIoT Transfer Container) to the AIOTF.

Editor's note:	It is FFS whether to use separate service operation to respectively support read, write or disable command procedure.

It is agreed that command type is used to discriminate service operations like read, write or disable. Hence, we need to delete the EN.

Editor’s note:	The AIoT Data specific for the Read/Write/Disable command is FFS.

For the above EN, AIoT data is as specified in clase 6.2.3.

Editor’s note:	The Command specific parameters for Read/Write/Disable command is FFS.

For the above EN, we use Command Type with AIoT data specific to Command Type. Hence, we can remove the EN.

Editor’s note:	It is FFS whether the Approximate message size from the AIoT Device can also be provided by the AF for the other command operations.

For the above EN, we cannot estimate the size from the AIoT Device for Write or Disable, even though the results will include success or failure status and optionally an error cause. Hence, we can remove the EN.

Editor’s note:	It is FFS whether that fail to discovery any AIoT device is normal operation result or failure case.

For Inventory, of which the main purpose is  searching for the target device(s). So, if there are no target devices for a target area, that would be a normal result. For Command, the main purpose is reading/writing/disabling. So, if there are no target devices for a target area, that would be a falure as the purpose is not achived. We need to add the above descriptions, removing the above EN.

Editor’s note:	It is FFS how to make AF be aware of the report is finished in case there are multiple notify for the same AIoT operation.

For the above EN, the last report indication in the final Naiotf_AIoT_Notify message indicates it is the last report for the operation.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369.

* * * Start of Change * * * *

7.2.1	General

The AIOTF supports to expose AIoT services towards the AF or the NEF as described in Table 7.2.1-1.

Table 7.2.1-1: NF services provided by the AIOTF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naiotf_AIoT | Inventory | Request/Response | NEF, AF |
|  | Command | Request/Response | NEF, AF |
|  | Notify | Subscribe/Notify | NEF, AF |

NOTE:	AMF sends the Namf_AIoT_Notify message (AIoT Transfer Container) to the AIOTF in order to report and update the information provided by the RAN for indirect connectivity.

7.2.3	Naiotf_AIoT_Command service operation

Service operation name: Naiotf_AIoT_Command.

Description: The NF consumer requests a command operation for one or multiple AIoT Device(s).

Inputs, Required:

1)	AF ID.

2)	At least one of the following parameters are included:

-	Target area information for the command operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID(s) or the filtering information for multiple AIoT Devices.

4)	Command Type: Read, Write or Permenant Disable.

5)	Command specific parameters: AIoT data (as specified in clase 6.2.3).

Inputs, Optional:

1)	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

-	Approximate message size from the AIoT Device for Read Operation.

Outputs, Required: Transaction ID, Result indication (Success or Failure), Failure Cause in case of Failure.

Outputs, Optional: None.

7.2.4	Naiotf_AIoT_Notify service operation

Service operation name: Naiotf_AIoT_Notify

Description: The AIOTF uses this service operation to notify the results or status of the service operation towards the NF consumers. If the NF consumer invokes the Naiotf_AIoT_Inventory, or Naiotf_AIoT_Command service operation, the NF consumer implicitly subscribes to the results of the requested service operation. For Inventory, the main purpose is searching for the target device(s). So, if there are no target devices for a target area, that would be a normal result. For Command, the main purpose is reading/writing/disabling. So, if there are no target devices for a target area, that would be a failure as the purpose is not achieved.

Inputs, Required:

1)	Common report information: Transaction ID, List of AIoT Device ID or Failure Cause in case of Failure).

2)	Read command specific report information: Information obtained from each target AIoT Device.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: The last report indication that indicates the message is the final message for the operation.

* * * End of Change * * * *