3GPP TSG-SA WG2 Meeting #169	S2-2505066

Fukuoka, Japan, May 19 – 23, 2025

Source:	Samsung

Title:	AMF Service Update

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The contribution proposes to update AMF Service for AIoT.

1. Discussion

AMF Service needs to be described in the normative text in TS 23.369 regarding the following editor’s notes.

Editor’s note:	It is FFS whether the Correlation identifier or AIOTF ID is included in the Namf_AIoT_MessageDelivery.

For the aboe EN, we need to update the text based on LS from RAN3 (R3-252481) which mentions the following.

-	Within all Command related NGAP messages:

	The AIOTF Identifier is included outside the AIOTF information container.

	Working assumption: The Correlation identifier is included inside and outside the AIOTF information container.

Editor's note:	It is FFS whether the Correlation identifier is included in the Namf_AIoT_Notify.

For the above EN, Correlation identifier is needed for notification between AMF and AIOTF, as AIOTF needs to identify the related AIoT Request by Correlation identifier.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369.

* * * Start of Change * * * *

7.3	AMF services

7.3.1	General

AMF supports to expose AIoT services towards the AIOTF as described in Table 7.3.1-1. The Namf_AIoT AMF service is used when NG-RAN and the AIoTF communicate indirectly via an AMF.

Table 7.3.1-1: NF services provided by the AMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Namf_AIoT | MessageDelivery | Request/Response | AIOTF |
|  | Notify | Subscribe/Notify | AIOTF |

7.3.2	Namf_AIoT_MessageDelivery service operation

Service operation name: Namf_AIoT_MessageDelivery

Description: The NF consumer requests to send AIoT data (i.e. AIoT inventory/command messages) towards NG-RAN or AIoT devices.

Inputs, Required:

1)	AIoT messages to deliver to NG-RAN.

2)	NG-RAN ID.

3) 	Correlation identifier

4)	AIOTF ID

Outputs, Required: AIoT messages received from NG-RAN, Result indication (Success or Failure), Failure Cause in case of Failure.

* * * Next Change * * * *

7.3.3	Namf_AIoT_Notify service operation

Service operation name: Namf_AIoT_Notify

Description: The NF consumer requests to receive AIoT data from NG-RAN or AIoT devices. If the NF consumer invokes the Namf_AIoT_MessageDelivery, the NF consumer implicitly subscribes to receive the AIoT data from NG-RAN or AIoT devices.

Inputs, Required:

1)	AIoT messages received from NG-RAN.

2)	Correlation identifier

Input, Optional: None.

Outputs, Required: Operation execution result indication.

Output, Optional: None.

Editor's note:	AIoT messages is FFS and coordination with RAN3 is needed.

Editor's note:	It is FFS how to make AIOT be aware of the report is finished in case there are multiple notify for the same operation.

* * * End of Change * * * *