3GPP TSG-SA WG2 Meeting #169	S2-2505081

Fukuoka, Japan, May 19 – 23, 2025

Source:	ZTE, MediaTek Inc

Title:	Solution for Pre-established Dual DRB Support for IMS Voice over NB-IoT via GEO Satellite

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: This contribution proposes a solution to optimize IMS voice over NB-IoT GEO satellite by pre-establishing default and dedicated bearers during PDN connectivity, reducing call setup time and improving user experience.

1. Discussion

In the context of GEO satellite access via NB-IoT connected to the EPC, the traditional bearer establishment mechanism for IMS voice services faces significant performance challenges due to inherent satellite characteristics, particularly the long RTT and limited data rate. NB-IoT supports both CP and UP transmission modes. The CP mode utilizes the S1-MME interface and the SRB-1(-bis) radio bearer to transmit small user data with low signalling overhead/complexity, while the UP mode leverages the S1-U interface, allowing up to two DRBs to be configured for more efficient data transport when the amount of user data is not small.

In conventional EPC-based IMS deployments, bearer establishment follows an on-demand approach. After UE attaches and establishes PDN connectivity to the IMS APN, only the default bearer is initially established for transmitting SIP signaling. A dedicated bearer for voice is set up later during the call setup phase, triggered by the P-CSCF upon receiving the SDP offer and completing policy negotiation with the PCRF. Subsequently, the PGW initiates the dedicated bearer setup based on QoS and policy requirements. While this deferred (i.e., on-demand) bearer establishment mechanism works well in terrestrial LTE networks, the ESM and eRRC procedures for establishing the bearer introduce additional significant delay in GEO satellite scenarios due to the high RTT (up to 540 ms) and low data rate of this radio link. This results in longer end-to-end call setup time and bring negative impact to user experience.

To mitigate this issue, the proposed solution recommends an optimized bearer establishment process specifically tailored to GEO satellite access scenarios. This approach suggests proactively establishing both the default and dedicated bearers during the PDN Connectivity Procedure, which may be triggered either during the initial attach or via a separate PDN connectivity request. By setting up both the default and dedicated bearers during PDN connectivity establishment procedure, some air interface signaling exchanges during IMS call setup are eliminated, significantly reducing the overall call setup time. By minimizing signaling round trips over the GEO satellite link, this method enhances IMS session responsiveness and ensures that the call setup time for voice services meets the required performance standards, making it an ideal solution for satellite-based NB-IoT deployments within the EPC.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-19.

* * * Start of Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X | X |  |  |  |

6.X	Solution #X: Solution for Pre-established Dual DRB Support for IMS Voice over NB-IoT via GEO Satellite

### 6.X.1	High level principles

This solution is to address Key Issue #1: Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC.

The principles of this solution is listed below:

1.	One dedicated EPS bearer specific for GEO voice is established at the very beginning as well as the default bearer during IMS PDN Connection Establishment Procedure.

2.	The SIP signalling is carried in the default EPS bearer (associated with one DRB) and the voice media is carried in the dedicated EPS bearer (associated with another DRB) specific for GEO voice.

3.	UE can indicate its capability information supporting IMS over GEO or subscribe to IMS over GEO.

### 6.X.2	Description

The network architecture is shown in the following figure:

Figure 6.x.2-1: network architecture

As specified in clause 4.7.2.1 of TS23.401, dedicated bearers are not supported over NB-IoT. The PDN GW uses the RAT Type to ensure that no dedicated bearers are active when the UE is accessing over NB-IoT. However, if both IMS signalling and IMS voice data are transmitted over the default bearer, QoS differentiation cannot be guaranteed, and it easily results in potential degradation of voice service quality. To avoid this problem, it is proposed that the at the very beginning of PDN connection establishment, one dedicated bearer specific for GEO voice is created as well as the default bearer. The support for establishing such a dedicated bearer specific for GEO voice in NB-IoT access shall be enabled by RAN.

### 6.X.3	Procedures

6.X.3.1	Attach procedure

The following figure depicts the Attach Procedure, where the PDN connectivity request is included in attach request.

Figure 6.x.3.1-1: Attach procedure

Step 1. UE initiate the attach procedure, indicating "IMS over GEO" capability and requesting PDN connectivity for GEO voice (i.e., IMS PDN).

Step 2. UE perform existing authentication and security procedures.

Step3. Update Location Request to the HSS, as per current procedure.

Step4. HSS send update location response, indicating "IMS over GEO" subscription.

Step5. MME select S-GW and P-GW based on the factors e.g., UE's subscription, capability, RAT type, APN. The PGW selected by the MME supports IMS over GEO,

Step6. MME send Create Session Request to the S-GW, forwarded to the P-GW, including RAT type, APN, and UE capability information. An indication is also present to indicate that the UE is subscribed with IMS voice over GEO, so that to trigger the PGW to create dedicate EPS bearer specific for GEO voice.

Step7 (optional). P-GW decided to establish policy association with PCRF to obtain QoS parameters for default and dedicated bearers.

Step8. P-GW respond with Create Session Response, including bearer contexts; TFT for dedicated EPS bearer specific for GEO voice may be preconfigured based on operator policy or implementationStep9. The default bearer (for IMS signaling) and the dedicated bearer (for IMS voice) are established simultaneously as part of the session creation procedure.

6.X.3.2	UE requested PDN connectivity Procedure

The following figure depicts the UE requested PDN connectivity Procedure, which occurs after the attach process.

Figure 6.x.3.2-1: UE requested PDN connectivity Procedure

Step 1. UE indicates its "IMS over GEO" capability, and MME retrieves the subscription information from HSS, indicating that the UE is subscribed to "IMS over GEO" during the registration procedure.

Step 2. UE send PDN Connectivity Request with "IMS over GEO" capability to initiate the IMS PDN connectivity establishment procedure.

Step3-7. Same as in the Figure 6.x.3.1-1 step 5 to 9.

### 6.X.4	Impacts to Services, Entities and Interfaces

Editor's note:	Further impact is FFS.

UE:

-	Support to indicate IMS over GEO related capability.

-	Support the establishment of dedicated bearer specific for GEO voice in the very beginning as well as the default bearer.

MME:

-	Support retrieval of "IMS over GEO" related subscription information from HSS.

-	Support selection of P-GW that support IMS over GEO services.

P-GW:

-	Support determination and initiation of concurrent establishment of a default EPS bearer (for IMS signaling) and a dedicated EPS bearer (for IMS voice) during session creation.

-	Support retrieval of PCC rules for the dedicated bearer associated with IMS over GEO from the PCRF.

-	Support preconfigured TFT and QoS policies for the dedicated bearer related to IMS over GEO.

HSS:

-	Support storage and delivery of subscription data indicating IMS over GEO service support.

PCRF:

-	Support policy provisioning for the dedicated EPS bearer associated with IMS over GEO.

* * * End of Change * * * *