SA WG2 Meeting ##169	S2-2505088

07 - 11 April, 2025, Fukuoka, Sweden

# Title:	[Draft] Reply LS on Ethernet MA PDU session using MPQUIC-E steering

# Response to:	 (S2-2504573/C4-250554) LS on Ethernet MA PDU session using MPQUIC-E steering

# Release:	Rel-19

# Work Item:	MASSS

Source:	Samsung (TO be SA2)

To:	CT4

Cc:	CT1

Contact Person:

Name:	Naman Gupta

E-mail Address:	nmn.gupta@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR <S2-2505089>

1. Overall Description:

SA2 would like to thank CT4 for the LS on Ethernet MA PDU session using MPQUIC-E steering and providing their input on this issue.

For Question 1 CT4 asked:

CT4 observes that SA2 has not defined any specific requirement about the handling of a MA PDU session using MPQUIC-E with a 3GPP access leg in EPS, e.g. upon the establishment of a 3GPP access leg in EPS (when a non-3GPP access leg has been established in 5GC) or upon a handover of the 3GPP access leg of a MA PDU session from 5GS to EPS. Note that TS 36.413 defines an Ethernet Type IE to enable the eNB to perform header compression appropriately.

Q1: Can SA2 clarify the handling of MA PDU sessions using MPQUIC-E with a 3GPP access leg in EPS with respect to the PDN connection type to be signalled to the eNB?

SA2’s answer:

<TO BE UPDATED based on the final decision at SA2#169>

SA2 is currently discussing on the possible solutions to minimize EPS impacts (more specifically impacts on MME) and need to decide whether interworking with EPS for MPQUIC-E can be supported or not.

For example, PDN connection establishment in EPS enabling MPQUIC-E, may require specific handling from the MME since PDN type is interpreted by the MME from the UE’s request (unlike the case in 5GC where AMF transparently forward the PDU Session type from SMF+PGW-C to AN).

While for cases like, handover from 5GS to EPS with N26 interface, it might be possible to support MPQUIC-E without any MME impacts.

SA2 would update CT4, once agreement in SA2 has been reached.

For Question 2 CT4 asked:

Besides, HR PDU sessions and PDU sessions with an I-SMF can support ATSSS. Per existing specifications, the V/I-SMF transfers ATSSS rules from the anchor SMF to the UE transparently (see ATSSS container in Table 6.1.6.4.4-2 of TS 29.502). Therefore, to support the quoted requirements for a HR PDU session or a PDU session with I-SMF, CT4 assumes that the (H)SMF shall newly signal to the V/I-SMF that the Ethernet PDU session uses an IPv4 or IPv6 transport (when using MPQUIC-E) and that using this new IE, the V/I-SMF shall signal the PDU session type as "IPv4" or "IPv6" in N2 signaling to the NG-RAN.

Q2: Can SA2 confirm CT4 assumption regarding the handling of MA PDU sessions using MPQUIC-E for HR PDU sessions and PDU sessions with an I-SMF.

SA2’s Answer:

SA2 would first like to clarify that as per TS 23.501 ATSSS is not supported for PDU Sessions with I-SMF and neither for ULCL/BP scenarios.

As per clause 5.32.2.1 of TS 23.501:

In this Release of the specification, a MA PDU Session using IPv6 multi-homing (see clause 5.6.4.3) or UL Classifier (see clause 5.6.4.2) is not specified.

In this Release of the specification, support for ATSSS assumes SMF Service Areas covering the whole PLMN or that a MA PDU Session is released over both accesses when the UE moves out of the SMF Service Area.

For the case of Home routed roaming, in order to avoid impact on the VPLMNs, SA2 suggests to re-use Selected PDU Session type IE sent from H-SMF to V-SMF, if possible, that can be set appropriately by the H-SMF, so that V-SMF sets the corresponding PDU Session type in the N2 SM information to (R)AN.

As for the UE, the Selected PDU Session type received in the NAS message would then also be IP (IPv4, or IPv6 or IPv4v6), for which UE can internally store/consider the PDU Session type as Ethernet for this PDU Session ID, based on checking the received ATSSS rules containing MPQUIC-E functionality.

This would avoid the burden for the roaming partners to update their V-SMFs in order to support this new steering functionality, and is in line with how new ATSSS steering functionalities were introduced in previous releases (i.e. the impact was only on the UE and the H-SMF).

Above changes have been captured in the attached <CR S2-2505089>.

SA2 further welcomes any feedback on the SA2’s suggested solution.

2. Actions:

To CT4 group.

ACTION: 	SA2 kindly asks CT4 group to take the above into consideration and provide further clarifications if needed.

3. Date of Next SA2 Meetings:

SA2 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-S2.htm