3GPP TSG-WG SA2 Meeting #169	S2-2505103

# Fukuoka City, Fukuoka, JP, 19th May – 23rd May, 2025	(revision of S2-2504251, 4228)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1251 | rev | 11 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for multi-modal services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The description of the Multi-modal Service Requirements is slightly reworded to emphasize that existing parameters are re-used and further details about the related PCF actions are clarified. A description is added for the scenario that the AF updates the multi-modal service by adding/removing data flows or changing the requirements of an ongoing data flow. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification of functionality related to AF update of a multi-modal service. Furthermore, a note is added to clarify the meaning of “data flow”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete description of functionality for multi-modal services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.27.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 23.502 CR#4702 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR has no stage 3 impacts. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.1.3.27.3	Support for delivery of multi-modal services

Enablers to support interactive media services that require high data rate and low latency communication, e.g. cloud gaming, AR/VR/XR services and tactile/multi-modal communication services are defined in TS 23.501 [2] clause 5.37.2.

For the delivery of a multi-modal service, the AF may request to the PCF directly (or indirectly via an NEF) multiple data flows for a single UE via single PDU session or for multiple UEs (via multiple PDU Sessions and a separate request per PDU Session), to be setup with specific QoS requirements, using the AF session with required QoS procedure described in clause 6.1.3.22.

NOTE 1:	The term "data flow" used in this clause describes a service data flow from the perspective of the AF.

Following additional attributes are supported where the AF provides specific information for the multi-modal service:

-	Multi-modal Service ID: an identifier that refers to the multi-modal communication service. Data flows belonging to the same multi-modal service share the same Multi-modal Service ID.

Multi-modal Service Requirements information in the AF request is consisting of the following existing attributes and is provided by the AF in order to describe specific information and requirements for every data flow of the multi-modal service, i.e. the AF can provide the following existing attributes multiple times, i.e. once per data flow:

-	Optionally, QoS monitoring requirements for the data flow.

-	QoS information and service requirements for the data flow, that consist of: Flow description information of the data flow and individual QoS parameters/QoS Reference as described in clause 6.1.3.22.

NOTE 2:	The attributes for multiple data flows can be provided via a single or multiple AF requests.

NOTE 3:	In order to start the QoS monitoring for the data flows associated to a multi-modal service within a certain period of time, the PCF needs to receive the QoS monitoring requirements for those data flows from the AF within a single request or, in case of multiple requests, within a short period of time.

The PCF determines whether the request from the AF is authorized, derives a PCC rule for each data flow based on the above information provided by the AF (in the same way it is described in clause 6.1.3.22) and provisions the determined PCC rules to the SMF for the affected PDU Session. The PCF may reject the request received from the AF (in the same way it is described in clause 6.1.3.22) if the service requirements of any data flow are not acceptable (i.e. the AF request is only accepted if the PCF authorization for every data flow is successful).

The AF may provide QoS monitoring requirements for each data flow associated with the same Multi-modal Service ID. If the PCF has received the QoS monitoring requirements from the AF, the PCF generates the QoS monitoring policy for the PCC rule corresponding to the data flow.

When the PCF receives a further AF request with the same Multi-modal Service ID and PCF authorization fails, the PCF rejects the AF request (in the same way it is described in clause 6.1.3.22). The Application decides how to deal with the data flows with already authorized AF request with the same Multi-modal Service ID (e.g. stop them, adjust the AF request).

The AF may update the Multi-modal Service Requirements information, either to modify the attributes applicable to existing data flows or to add/remove data flows (see clause 4.2.3.36 of TS 29.514 [36] for the details).

# * * * * End of changes * * * *