3GPP TSG-SA2 Meeting #169	S2-2505131

Fukuoka, Japan, 19th May 2025 - 23rd May 2025

Source:				CEWiT

Title:			Solution for KI#3: Sensing Entity and Sensing Function Discovery

Document for:				Approval

Agenda Item:				20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The contribution proposes a solution for Key Issue #3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

- Introduction

This paper presents a solution that enables the 5G System (5GS) to select and appropriate Sensing entity(ies) and discover Sensing function during a Sensing procedure.

- Text Proposal

It is proposed to agree the following changes in 3GPP TR 23.700-14.

- * * * * Start of Changes * * * *

- 6.X	Solution #X: Sensing Function Discovery and Sensing Entity Selection

- 6.X.1	Description

The proposed solution addresses Key Issue #3.

For the Sensing feature to be supported in the network, a NF namely “SeMF (Sensing Management Function)” is introduced to control and manage sensing service operations.

- 6.X.2	Procedures

### 6.X.2.x		SeMF Discovery and Selection

- SeMF selection and discovery happens during a Sensing procedure. The Sensing Function selection functionality may be supported by the NF service consumer.

- Sensing Function re-selection is a functionality may be supported by the NF service consumer, when necessary, e.g. due to unavailability of SeMF to serve a specific area during a procedure.

- The Sensing function selection/re-selection may be performed by the NF service consumer based on either locally available information i.e. Sensing Function profiles are configured locally at the NF service consumer, or by querying NRF.

The following factors may be considered during the SeMF selection:

-	SeMF Service Area consisting of one or more Cell ID(s), TA(s), Supported GAD shapes.

-	Requested Quality of Service information, e.g.:

-	Probability of False Alarm

-	Probability of Detection

-	Sensing accuracy

-	Sensing range

-	Sensing resolution (range and angle)

-	Response time (latency)

-	Network slicing information, e.g. S-NSSAI and/or NSI ID.

-	AF Identifiers supported by SeMF.

-	SeMF load.

-	Indication of either a single event report or multiple event reports i.e., duration of event reporting.

Figure 6.X.2.x: Procedure to support SeMF Discovery and Selection

1.	Upon a sensing request, the NF service consumer decides to select an appropriate SeMF to initiate sensing request.

2.	The NF service consumer invokes a Nnrf_NFDiscovery_Request service operation to an NRF to discover a SeMF, the service operation includes the discovery parameters as listed above.

3.	The NRF sends a Nnrf_NFDiscovery_Request response which includes the profiles of the selected SeMFs to the NF service consumer.

4.	The NF service consumer selects the appropriate SeMF instance from the list of candidate SeMFs provided by NRF

5.	The NF service consumer invokes the Nsemf_Sensing Request for initiating sensing procedure.

### 6.X.2.y		Sensing Entity Discovery and Selection

Sensing Entity selection and discovery happens during a sensing service to collect sensing data to compute the sensing result. The Sensing Entity selection shall be performed by SeMF.

Sensing Entity re-selection is a functionality shall be supported by the SeMF when necessary, e.g. when the sensing measurement result does not meet the requested accuracy.

SeMF may obtain information about the sensing capabilities of Sensing Entity(ies).

The following factors may be considered during the Sensing Entity selection:

-	Sensing Entity(ies) Capabilities

-	Requested Service Area consisting of one or more Cell ID(s), TA(s), Supported GAD shapes.

-	Network slicing information, e.g. S-NSSAI and/or NSI ID.

-	Sensing Entity(ies) location.

### Sensing Entity selection/re-selection can also be based on locally configured information at SeMF.

- 6.X.3	Impacts on Services, Entities and Interfaces

The following impacts may be present for sensing entity and SeMF discovery,

NF Service Consumer:

-	Support at NF Service Consumer for Discovery and Selection of SeMF

SeMF:

-	Support for discovery and selection of Sensing Entity(ies)

NRF:

-	Support of NF Profile of SeMF

AMF:

### -	Support for Sensing Entity Selection/Re-Selection by SeMF

* * * * End of Changes * * * *