TSG SA WG2 Meeting #169	S2-2505136

19 - 23 May, 2025, Fukuoka, Japan

Source:	Lenovo

Title:	Solution for KI#3: Sensing Function discovery and selection

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing-ARC / Rel-20

Abstract of the contribution: Proposes new solution for sensing function discovery and selection

# 1. Introduction

This pCR proposes the solution for Key issue#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  |  | X |  |  |  |

# * * * * Second change (All Text New) * * * *

## 6.X	Solution #X: < Sensing Function discovery and selection >

### 6.X.0	High-level solution Principles

The high-level principles for sensing function discovery and selection are:

- The NEF or AF may utilize the NRF to discover SF instance(s). E.g., the NEF or AF provides to the NRF the target sensing area and sensing service type when trying to discover SF instances.

- If more than one SF instances are provided by the NRF, the NEF or AF performs Sensing Function selection.

### 6.X.1	Description

This solution aims to address key issue#3 on Sensing Entity and Sensing Function Discovery and (Re-)Selection.

The Sensing Function (SF) discovery and selection functionality is supported by the NEF and AF to determine a SF to handle the sensing services.

When the NEF or AF performs SF discovery and selection for a sensing service request, the following applies:

- The NEF or AF may utilize the NRF to discover SF instance(s), which are registered to NRF unless SF information is available by other means, e.g., locally configured in the NEF or AF. The NEF or AF provides to the NRF the target sensing area and sensing service type when trying to discover SF instances. The NRF provides SF instance(s) to the NEF or AF.

- The NEF or AF selects the SF instance(s) based on the available SF instance(s).

### 6.X.2	Procedures

Figure 6.X.2-1: Sensing Function discovery and selection procedure.

Figure 6.X.2-1 takes NEF performing SF discovery and selection as an example.

- The SF sends Nnrf_NFManagement_NFRegister request message to NRF, which includes SF (i.e., NF type), FQDN or IP address of SF, PLMN ID, supported sensing area and supported sensing service type etc. Sensing service type can be Smart transportation (e.g., High definition map construction, intrusion detection), Smart Low Attitude (e.g., Flight intrusion detection, Flight Path Management), Smart Network (e.g., Channel estimation enhancement, beam management) and Smart Life (e.g., respiratory monitoring, Gesture/posture recognition) etc.

- The NRF responds with Nnrf_NFManagement_NFRegister response message to the SF.

- The AF sends Nnef_SensingService_Request message to the NEF, which includes target sensing area, sensing service type and other sensing requirements etc. The other sensing requirements may include sensing accuracy, sensing resolution, confidence level, Missed detection probability, False alarm probability, Max sensing service latency, refreshing rate etc.

- The NEF authorizes the AF request of the sensing serives based on SLA between 3rd party AF and the 5GS of the mobile network operator, the operator policy and local configuration.

- The NEF sends Nnrf_NFDiscovery_Request message to the NRF, which includes SF (i.e., NF type), target sensing area and sensing service type etc.

- The NRF responses with Nnrf_NFDiscovery_Response message, which includes SF information (e.g., FQDN or IP address of SF etc.).

- If more than one SF instances are provided by the NRF, the NEF performs Sensing Function selection.

### 6.X.3	Impacts on services, entities and interfaces

NEF:

- Interaction with NRF for Sensing Function discovery and selection.

AF:

- Interaction with NRF for Sensing Function discovery and selection.

NRF:

- Interaction with NEF or AF for Sensing Function discovery.

- Interaction with SF for Sensing Function capability registration.

# * * * * End of changes * * * *