3GPP TSG-WG SA2 Meeting #169	S2-2505154

Fukuoka, JP, 19th May – 23rd May, 2025	(revision of S2-2503660)

Source:	Huawei, HiSilicon

Title:	[New Sol] Connection setup for NB-IoT IMS call over GEO

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: A solution for support of IMS voice call over GEO via NB-IoT NTN connecting to EPC.

# 1. Introduction/Discussion

In Rel-20 FS_5GSAT_Ph4-ARC study item, it is proposed to study the IMS voice call over GEO via NB-IoT NTN connecting to EPC as shown below:

- WT-1: IMS voice call over GEO via NB-IoT NTN connecting to EPC

NOTE 1: both CP and UP based NB IoT NTN solutions will be studied in the WT-1, checkpoint will be set before the normative work to align with RAN scope.

NOTE 2: Only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 will not be considered.

- WT-1.2 Whether and how to support QoS for IMS voice service over GEO satellite,

IMS voice call requires 2 DRBs for signalling and data transmission, while NB-IoT devices have lacked capability (up to 2DRBs) and connection over GEO brings long latency. A new mechanism needs to be introduced to support the connection setup (e.g. PDN connection, IMS registration, call setup) and QoS requirement for IMS voice call over GEO via NB-IoT.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-19.

# * * * * First change (All new text) * * * *

## 6.X	Solution X: Connection setup for NB-IoT IMS call over GEO

### 6.X.1	Description

This solution resolves Key Issue #X about the connection setup for NB-IoT IMS call over GEO.

This solution enhances the MME to authorize attach and PDN Connection setup for NB-IoT for IMS, considering the capability of the NB-IoT UE.

### 6.X.2	Procedures

When MME receives an attach/PDN connection setup request for the IMS APN and the RAT type of UE is NB-IoT(GEO), the MME authorizes the UE’s subscription data and setup PDN connection for NB-IoT IMS.

NB-IoT

UE

eNB

MME

SGW/

PGW

PCRF

HSS

IMS

2. AS message (Attach/PDN connection setup request (APN=IMS, UE capability))

3. S1 message (Attach request/PDN connection setup request (APN=IMS, UE capability), RAT type = NB-IoT (GEO))

4. MME determines whether to allow the PDN connectivity establishment, e.g. based on policy, subscription.

5. create session request/response

6. Attach/ PDN connection setup accept

7. Procedures for IMS voice call (e.g. IMS registration, IMS call setup)

1. SIB (NB-IoT (GEO) cell)

Figure 6.X.2-1: high-level procedure for connection setup for NB-IoT IMS call over GEO

- The UE determines that the NB-IoT access is via GEO satellite and whether connection to IMS is supported or not.

- The UE sends attach or a PDN connection setup request, including IMS APN and the UE capability.

Editor’s Note: If UP solution is supported, how to establish the connection when there is no available DRB on the UE is FFS.

Editor’s Note: If CP solution is supported, how to support IMS signalling and data transmission over CP, whether and how to define and enable the QoS of CP are FFS.

- The eNB transfers the attach or PDN connection setup request to MME and indicates the RAT type of UE (i.e. NB-IoT access via GEO).

- If the MME receives a PDN connection setup request with IMS APN, and the RAT type of UE is NB-IoT (GEO), the MME checks the UE subscription and policy data to determine whether the connection setup for IMS via NB-IoT GEO can be authorized.

- If the request is authorized, the MME indicates SGW, PGW to create session for the IMS APN.

- When create session response is received by the MME, it sends attach or PDN connection setup accept to the UE.

- When the attach or PDN connection setup for IMS service is accepted, the UE can execute IMS procedures.

Editor’s Note: Whether and how to enhance the IMS registration, call setup procedures to enable IMS call over NB-IoT GEO access is FFS.

### 6.X.3	Impacts on services, entities and interfaces

UE:

-	Request for connection setup for IMS services when access via NB-IoT (GEO) cell.

-	NB-IoT UE Supports the capability of IMS voice call and indicates its capability in the attach or PDN connection setup request.

MME:

-	Authorize the attach or PDN connection setup request for IMS services from NB-IoT UE.

-	Indicate SGW/PGW to create session with IMS APN for NB-IoT UE.

Editor’s Note: Further updates to the impacts may be needed as other Editor’s Notes are resolved.

# * * * * End of changes * * * *