3GPP TSG-WG SA2 Meeting #169	S2-2505162

# Fukuoka, JP, 19th – 23rd May, 2025		(revision of S2-2503628)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6239 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Report of SMF to EIF for user-plane energy consumption calculation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The issue of SMF reporting collection information for UP EC calculation was raised and discussed in SA2#167 meeting, and it was determined to re-visit this aspect in #168 meeting. Three options are considered: 1) having interval and report on one shot; 2) having interval and report at the end of each period T; 3) report at the end of each period T until EIF tell the SMF to stop. This document prefers option 3), since it is more fit for the current subscribe/unscribe procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description on how does the SMF provdes EIF the information for user-plane energy consumption calculation in 5.51.2.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specificsations |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 5.51.2.2.2	Energy Consumption information collection from SMF

The serving SMFs are retrieved by the EIF from the UDM of the UE based on the provided input parameters including the UE ID and (S-NSSAI, DNN).

The EIF invokes Nsmf_EventExposure_Subscribe as defined in TS 23.502 [3] with the required granularities (UE ID, DNN/S-NSSAI, application information (e.g. Application Identifier, or Packet Filters)) to retrieve the information from SMF, which is shown in Table 5.51.2.2.2-1. The SMF receives the data volume of the required granularities from the PSA UPF. The SMF then sends the collected data volume, along with the serving gNB ID(s) and (I-)UPF ID(s) to EIF for energy consumption calculation. And the information collected from SMF by EIF, is shown in Table 5.51.2.2.2-2.  The SMF periodically notifies EIF the information at the end of each time interval T, until the EIF unsubscribes it based on e.g., the requested time period by 5GC NF or AF as defined in TS 23.502 [3].

Table 5.51.2.2.2-1: Information to SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE ID | SUPI. |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| Packet Filters | Packet Filters as in 5.7.6 for IP or Ethernet traffic. |

NOTE 1:	The user-plane energy consumption information reporting interval from the SMFs is the PLMN-wide configurable starting time and interval T.

Table 5.51.2.2.2-2: Information from SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE IP address | UE IP address. |
| UE ID | SUPI. |
| S-NSSAI | Network Slice applicable to a UE |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| Packet Filters | Packet Filters as in clause 5.7.6 for IP or Ethernet traffic. |
| Application Identifier | Identification for the traffic of the service data flow. |
| List of Data Volume information | The data volume and the associated UPF(s) and gNB(s) serving the UE within the time interval. |
| > UL/DL Data Volume | The UL/DL Data Volume of a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a Service Data flow (UE ID, S-NSSAI, DNN, Packet Filters/Application Identifier). |
| > (I-)UPF ID(s) | Identifier of any (I-)UPF(s) associated to a reported data volume used by a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a Service Data flow (UE ID, S-NSSAI, DNN, Packet Filters/Application Identifier). |
| > gNB ID | Identifier of the gNB serving the UE. |
| Reference to Time Interval | Indicate the time interval of the collected information (e.g. time stamps). |

NOTE 2:	Each entry of List of Data Volume information represents the serving gNB and UPF(s) corresponding to the same Data Volume regarding the required granularities. A new entry is added when the gNB or UPF(s) is changed.

# * * * * End of changes * * * *