3GPP TSG-WG SA2 Meeting #169	S2-2505168

Fukuoka City, Fukuoka, JP, 19th May – 23rd May, 2025	(revision of S2-25xxxxx)

Source:	Huawei, HiSilicon

Title:	WT#3: NF selection profile

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposed the NF selection as the solution of the KI#3.

# 1.	Introduction

In the approved SID SP-250417, the NF selection is considered as one of the candidate key issues to be resolved for energy study in Rel-20:

- WT#3: Whether and how to support NF (re)selection and/or UP path adjustment based on energy related information

This document proposed the solution for UP selection based on energy related parameters

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-67.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| Y |  |  | X |

# * * * * Third change (All text new) * * * *

## 6.y	Solution #y: Selection of the UPF with considering energy related information

This solution is for Key Issue #3: Whether and how to support UP path adjustment based on energy related information.

### 6.Y.1	High level principles

It is proposed to add the Scheduled Switch-Off Time, which can be used for e.g., UPF selection for PDU Session with SSC mode 1, and SMF may not select the UPF about to be in energySaving state in that case.

When SMF selects the UPF, the SMF can also make use of the NF Profiles (i.e., Scheduled switch-off time) by considering the potential usage of the UPF for energy related execution. The UPF with potential usage of energy related execution should be deprioritized to be chosen as the PDU Session Anchor of the PDU session with SSC mode 1.

### 6.Y.2	Procedures

This clause provides an example on how to make use of the proposed energy related information defined in Sol #X & Y

The information to be considered for UPF selection are defined in clause 6.3.3 of TS 23.501 [X1]. When SMF selects the UPF, the SMF can also make use of the NF Profiles by considering the potential usage of the UPF for energy related execution. The UPF with potential usage of energy related execution should be deprioritized to be chosen as the PDU Session Anchor of the PDU session with SSC mode 1.

Figure 6.Y.3.1-1: Selection of the UPF with considering energy related information.

0.	SMF provisioning of UPF instances using NRF.

1.	The UE registers to the PLMN, as described in clause 4.2.2.2 of TS 23.502 [X2].

2.	The PCF makes use of UE Configuration Update procedure to provide UE Policy as defined in clause 4.2.4.3 of TS 23.502 [X2].

3.	UE initiates the UE Requested PDU Session Establishment procedure with including the necessary information, including the requested SSC mode.

4.	The SMF selects the UPF interacting with the NRF based on energy information in the NF profile, for example when the UE has an energy subscription or based on SMF configuration related to Network Energy Saving. If the selected SSC mode of the PDU session is SSC mode 1, the SMF may select the UPF taking into account the Energy Priority in order to select the UPF with Higher priority in term of usage of energy defined in solution #y and/or the Scheduled Switch-Off Time defined in Solution #y in order to avoid to select a UPF which may be soon switch-off for a PDU session with SSC mode 1.

5-6	These steps are the step in PDU session establishment procedure in TS 23.502.

### 6.Y.3	Impact

The impacts are :

-	NRF

-	support the NF profile extension for energy based information defined in solutions for KI#3

-	extension of SBI to send request of NF based on energy indication support

-	SMF

-	support of UPF selection based on energy information

-	support of SBI extension toward NRF

# * * * * End of change * * * *