SA WG2 Meeting #169	S2-2505172

19 - 23 May, 2025, Fukuoka, Japan	(revision of S2-25xxxxx)

Source:	ZTE

Title:	pCR to TS 23.369: Clarification on NG-RAN Node and RAN Reader Selection

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The paper updates NG-RAN node and RAN reader selection based on the progress of RAN3, and addresses the related ENs in clause 5.3.3.

1. Discussion

In the LS from RAN3 (S2-2504523/R3-252481), RAN3 has reached the following working agreement regarding A-IoT Area.

- RAN3 agreed to define a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message. The detailed coding of the new “A-IoT Area” is FFS.

- The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB.

In the endorsed CR R3-252493 (BL CR to 38.300) Introduction of Ambient IoT, the following changes are added:

The AIOTF may request the gNB to perform an A-IoT operation within a specific area (requested service area). The requested service area may be indicated by means of a list of A-IoT Areas and/or a list of readers.

The AIOTF is aware of the area within which the gNB supports provision of A-IoT services by means of OAM configuration. This area may be represented as A-IoT Areas and/or the readers supported by the gNB.

The AIOTF may also be aware of the reader’s location by means of OAM configuration.

It is observed that the supported Area in SA2 document has the same meaning of A-IoT Areas in RAN3 document.

Based on RAN3 agreement and the endorsed CR for TS 38.300, it is proposed to add the description for supported area and remove the related ENs in clause 5.3.3.

2. Proposal

It is proposed to capture the following changes into TS 23.369.

* * * * Start of Changes * * * *

5.3.3	NG-RAN Node and RAN Reader Selection

The AIOTF selects NG-RAN node(s) and optionally RAN readers.

The AIOTF obtains the NG-RAN information (supported Area, and/or RAN reader ID list, and, optionally, the location of each served RAN reader) via OAM. The supported Area is a list of AIoT Areas supported by NG-RAN as specified in TS 38.300 [5]. The AIOTF receives an AIoT service request including the Target Area information from the NEF or trusted AF. Based on the received Target Area information and the NG-RAN information configured by OAM, the AIOTF selects the NG-RAN node(s) and optionally RAN reader(s).

NOTE 1:	The Target Area information can span the supported Area of multiple NG-RAN node(s) or can be a subset of the supported Area of a NG-RAN node.

The AIOTF sends the AIoT service request to the selected NG-RAN node(s), optionally including the RAN Area information derived from mapping the Target Area information to the supported Area and/or RAN reader ID list, if selected, to assist RAN reader selection by NG-RAN node, either directly or through the selected AMF. If multiple NG-RAN nodes are selected, the AIOTF sends the AIoT service request to each selected NG-RAN node along with its corresponding RAN Area information and/or the RAN reader ID list, if selected

If the AIOTF does not provide the RAN Area information or the RAN reader ID list in the AIoT service request to the NG-RAN node, then the NG-RAN node may use all available RAN readers.

NOTE 2:	RAN reader ID is not exposed to the AF.

If an AIoT service request includes AIoT device Identifier(s), the AIOTF may consider the last known serving RAN reader(s) from the AIoT device context to determine targeted RAN reader(s) directly.

NOTE 3:	From the selected NG-RAN node, the AIOTF receives the Inventory Report including the RAN reader ID that represents the AIoT device’s location at reader ID granularity. The AIOTF uses the RAN reader ID to update the last known serving RAN reader information in the local AIoT device context.

* * * * End of Changes * * * *