SA WG2 Meeting #169	S2-2505177

19 - 23 May, 2025, Fukuoka, Japan                  	(revision of S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1647 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding BDC Procedure Update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The paper is proposed to update the description and procedure for adding BDC to an existing IMS session. In AG.2.1.0, only procedures of adding ADC(s) are mentioned; while adding BDC(s) is also described in AG.2.1.4. In AG.2.1.4: The title of AG.2.1.4 is modified to align with other clauses. Step.6 description in figure AG.2.1.4-1 mentions 3rdPartySessionUpdate which should be ExternalSessionUpdateEvent; Step.3 procedure description is regarding BDC adding request, not ADC; Step.7 refers to AC.7.1, while only DC media resource for UE B is needed for this adding BDC procedure. Step.12~15 in the figure are changed to dashed line. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description in AG.2.1.0 is modified; several steps in procedure for adding BDC are clarified and the step description in figure AG.2.1.4 is corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The step description for adding BDC to an existing IMS session is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AG.2.1.0, AG.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

AG.2.1.0	General

This clause describes the procedure for adding, removing or updating bootstrap data channel(s) and application data channel(s) to an existing IMS session via dedicated NEF services.

The DC AS sends a request to the NEF via Nnef_ImsSessionManagement_Update to update an existing IMS session with a specific session ID with a data channel. The procedures of adding A2P, P2P and P2A2P application data channel(s) and adding bootstrap data channel(s) to an existing IMS session are provided below. For removing or updating application data channel(s) in an existing IMS session, as well as for removing, or updating bootstrap data channel(s) in an existing IMS session, the same principle applies.

NEXT CHANGE

AG.2.1.4	Adding Bootstrap Data Channel to an existing IMS session

Figure AG.2.1.4-1 depicts the call flow of adding bootstrap data channel towards UE B to an existing IMS session. In this scenario, the MF-2 is used to anchor the bootstrap data channel between the UE B and the DCSF2.

Figure AG.2.1.4-1: Procedure of adding bootstrap data channel to an existing IMS session

An IMS voice session between UE A and UE B has been established.

0.	DC AS decides to add a bootstrap DC to the existing IMS session, which is notified based on the event notification of the related IMS session to which the DC AS subscribed to.

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to add a bootstrap data channel to the existing IMS session.

The DC AS includes the session ID of the existing IMS session to be updated, the operation type as Adding, media operation set including the parameters for the media type set as BDC, the UE B as the target of the BDC, in Nnef_ImsSessionManagement_Update request. The DC AS may include a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Update request to be notified of the session update progress.

2.	The NEF uses the Nhss_ImsUECM_AsInfoGet service operation to retrieve the IMS AS instance serving UE B.

3.	If the NEF authorizes the request, the NEF sends a Nimsas_ImsSessionManagement_Update request to the IMS AS-2 requiring to add bootstrap data channel(s) to the specific IMS session. The received session ID of the existing IMS session, the operation type as Adding, the media type as BDC, and the UE B as the target of the BDC, shall be included in the request. The NEF may include a Notification Target Address and Correlation ID to the Nimsas_ImsSessionManagement_Update request to be notified of the session update progress.

4.	The IMS AS-2 determines whether the data channel request should be notified to DCSF-2 based on subscription information and DC capability of UE B. If UE B does not have subscription of IMS data channel or does not have the IMS DC capability, or then the request shall be rejected with appropriate cause and subsequent steps are skipped. If the IMS AS-2 allows the request to proceed, it returns a successful Nimsas_ImsSessionManagement_Update response to the NEF.

If the bootstrap DC is already established, the request shall be rejected with an appropriate cause and the following steps are skipped.

5.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

6.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with event set to ExternalSessionUpdateEvent. The notification includes the necessary parameters based on the information extracted from media operation set parameters received in step 3 and other stored parameters in the IMS AS-2 if applicable.

7.	Steps 7 to 10 in clause AC.7.1 applies with the following exception:

-	The data channel media resource in MF-2 is allocated only for UE B.

8.	The IMS AS-2 generates a re-INVITE request in which the SDP offer contains the media information of the bootstrap data channel and existing voice media and sends the request to UE B.

9.	UE B responds 200 OK with SDP answer.

10.	Steps 17 to 18 in clause AC.7.1 applies.

11.	IMS AS sends ACK to UE B to complete the IMS session modification procedure.

12.	If the NEF included a Notification Target Address in the Nimsas_ImsSessionManagement_Update request in step 3, the IMS AS-2 notifies the NEF MediaChangeSuccess event with the Nimsas_ImsSessionManagement_Notify request.

13.	If the DC AS included a Notification Target Address in the Nnef_ImsSessionManagement_Update request in step 3, the NEF notifies the DS AS MediaChangeSuccess event with the Nnef_ImsSessionManagement_Notify request.

14-15.	The DC AS and NEF return responses.

END OF CHANGES