SA WG2 Meeting #169	S2-2505178

19 - 23 May, 2025, Fukuoka, Japan                  	(revision of S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1648 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Standalone DC Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The paper is proposed to update the following standalone DC procedures: In AC.10.2.2 for originating standalone IMS DC Session if application not available in originating UE, it is clarified that the IMS AS shall reject the session initiated by the originating UE not subscribed for a standalone IMS DC session. In AC.10.2.5 for removing all MMTel media from IMS DC session, it is clarified that the IMS AS shall reject the SIP re-INVITE request for removing MMTEL media initiated by UE-A not subscribed for a standalone IMS DC session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the clarification on procedures in AC 10.2.2 and AC 10.2.5 that if the UE doesn’t subscribe for a standalone IMS DC session, the IMS AS shall reject the request for initiating or converting to a standalone IMS DC session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures in AC 10.2.2 and AC 10.2.5 do not describe the situation if the UE doesn’t subscribe for a standalone IMS DC session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.10.2.2, AC.10.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

AC.10.2.2	Originating Standalone IMS DC Session if Application not Available in Originating UE

When an originating UE initiates a standalone IMS data channel session and the DC application is not available in the originating UE, the originating UE initiates standalone bootstrap DC setup. After successful establishment of bootstrap DC with the DCSF, the originating UE downloads the DC application via the bootstrap DC and updates the IMS session to add application DC. If the DC application is not downloaded yet by the terminating UE, the terminating UE does not accept the addition until the DC application is downloaded via the bootstrap DC. If the terminating UE did not successfully download the DC Application, the terminating UE, as an example, does not alert the user at all and tear down the entire IMS session.

Figure AC.10.2.2-1 depicts a signalling flow diagram for originating standalone IMS DC Session if application is not available in the originating UE with the assumption that application list is available in the originating UE.

Figure AC.10.2.2-1: Originating Standalone IMS DC Session if Application not Available in Originating UE

The steps in the call flow are as follows:

1.	The UE#1 sends an initial SIP INVITE request with an initial SDP offer including media component for bootstrap data channel for originating side and optional for terminating side. The SIP precondition is not needed. The originating P-CSCF may include the P-Early-Media header field indicating support of early media authorization in the initial SIP INVITE request if not exists.

2-13.	Steps 2-13 of clause AC.7.1 apply with the following difference:

-	The IMS AS rejects the standalone IMS DC session initiated by the UE#1 if the UE#1 has not subscribed for a standalone IMS data channel session, and the following steps are skipped.

In this scenario, the SDP offer for bootstrap data channel to UE#2 are included.

14.	The UE#2 returns a SIP 18X response with an SDP answer for bootstrap data channel. The IMS AS includes the P-Early-Media header field indicating early media allowed in the SIP 18X response based on the SDP answer not indicating any application data channel. Simultaneously, the UE#2 may alert the terminating user the incoming session is a standalone IMS data channel session that DC application will be indicated later.

15.	Step 21 of clause AC.7.1 applies, except that the UE#1 may download the application list.

16.	The UE#1 sends a SIP PRACK request towards the terminating side before or during performing step 15 and the UE#2 returns a SIP 200 OK response for the PRACK. After the DC application has been downloaded, the UE#1 initiates the SIP UPDATE request to update the IMS session for adding the application DC and to inform the UE#2 the associated DC application binding information.

The IMS AS, the DCSF and the MF repeat steps 3-10 with the media information of bootstrap data channel and application data channel, optionally including allocation of MDC2 resources for originating and terminating side. The IMS AS sends the SIP UPDATE request with the modified SDP offer including media component for bootstrap data channel and application data channel towards the terminating network and the UE#2. The IMS AS includes the P-Early-Media header field indicating early media allowed in the SIP UPDATE request based on the previously received SDP answer not indicating any application data channel.

17.	The bootstrap data channels have been established between the originating MF and the UE#2. If the DC application is not available in the UE#2, the UE#2 may alert the terminating user for DC application downloading. The UE#2 sends application request messages to the MF to request downloading the DC application and application list if needed.

18-23.	When the DC application is available in the UE#2, steps 15-20 of clause AC.7.1 apply with the difference that bootstrap data channel and application data channel established is indicated. The IMS AS may include the P-Early-Media header field indicating early media allowed in the SIP 200 response for the UPDATE forwarded to the originating side.

24.	The UE#2 alerts the terminating user for running the DC application. The UE#2 returns a SIP 180 response to indicate the UE#2 is ringing.

25.	Subsequent procedures continue. When the terminating user accepts to run the DC application for communication, the UE#2 answers the IMS session.

SECOND CHANGE

AC.10.2.5	Removing All MMTel Media from IMS Data Channel Session

When a IMS data channel session between two UEs is ongoing, any of the UE may remove all MMTel media (e.g. audio/video/messaging) from the IMS session, which turns the IMS data channel session to standalone IMS data channel session.

Figure AC.10.2.5-1depicts a signalling flow diagram for removing all MMTel media (e.g. audio/video/messaging) from IMS data channel session between two UEs.

Figure AC.10.2.5-1: Removing all MMTel Media from IMS data channel session

1.	An IMS DC session has been established between the UE-A and the UE-B, which includes MMTel and application DC media.

2.	The UE-A sends SIP re-INVITE request with an SDP offer containing media information for MMTel media and application data channel, which sets the port number of MMTel media stream to zero, as specified in RFC 3264 [72].

3.	The IMS AS, the DCSF and the UE-B handles the SIP re-INVITE request as specified in steps 2-9 of clause AC.7.2.1 with the following difference:

-	The IMS AS rejects the SIP re-INVITE request from the UE-A if the UE-A has not subscribed for a standalone IMS data channel session, and the following steps are skipped.

4.	If the UE-B accepts the session update, the UE-B sends 200 OK response to the UE-A through the IMS AS, as specified in steps 10-18 of clause AC.7.2.1, indicating the MMTel media has been removed.

END OF CHANGES