3GPP TSG-SA WG2 Meeting #169	S2-2505185

Fukuoka, Japan, May 19–23, 2025

Source:	NEC

Title:	KI3 new solution: UPF discovery based on energy related information

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#3 for the Rel-20 FS_EnergySys_Ph2 study.

1. Introduction

This contribution proposes a new solution for KI#3 for the Rel-20 FS_EnergySys_Ph2 study.

-	Whether and how to perform NF (re)selection based on energy related information, including:

-	Whether and how to extend NF profile (e.g., include energy related information) to support NF (re)selection.

-	Which energy related information is to be considered for NF (re)selection.

-	Whether and how to enhance NF (re)selection procedure.

2. Proposal

It is proposed to agree the following key issue for the KI#3 in the TR23.700-67.

* * * Start of change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |
| --- | --- | --- | --- |
|  | 1 | 2 | 3 |
| X |  |  | X |

* * * End of change * * * *

* * * Start of change * * * *

6.X	Solution #X: UPF discovery based on energy related information

6.X.1	Description

This solution applies to Key Issue #3.

EIF is able to request the energy consumption and data volume information per NF level from OAM. This procedure applies when the EIF wants to get informed about UPFs available in the network and supporting a list of parameters. It reuses the procedure for SMF provisioning of UPF instances using NRF in clause 4.17.6.2, TS23.502[3] but the consumer is replaced by EIF. The UPF may support an additional information that indicates energy related information to be collected by OAM.

6.X.2	Procedures

This procedure applies when an EIF wants to get informed about UPFs available in the network and supporting a list of parameters.

Figure 6.x.2-1: UPF discovery using NRF procedure

The following procedure takes place when an EIF expects to be informed of UPFs available in the network:

- The EIF subscribes Nnrf_NFManagement_NFStatus service to be notified with the target UPF Provisioning Information it is interested in.

- At any time, a new UPF instance is deployed and configured with the NRF identity to contact for registration and with its UPF Provisioning Information. An UPF is not required to understand the UPF Provisioning Information beyond usage of this information to register.

- The UPF instance issues an Nnrf_NFManagement_NFRegister Request operation providing its NF type, the FQDN or IP address of its N4 interface and the UPF Provisioning Information configured. UPFs may be associated with UPF Provisioning Information in the NRF. The UPF Provisioning Information additionally consists of new attribute:

- Support of energy related information to be collected by OAM

- Alternatively OAM registers the UPF on the NRF indicating the same UPF Provisioning Information as provided in step 4. This configuration mechanism is out of scope of this specification.

- Based on the subscription in step 1, NRF issues Nnrf_NFManagement_NFStatusNotify to all EIFs with a subscription matching the UPF Provisioning Information of the new UPF.

- The NRF issues Nnrf_NFManagement_NFStatusNotify with the list of all UPFs that currently meet the EIF subscription. This notification indicates the subset of the target UPF Provisioning Information that is supported by each UPF.

- EIF invokes Nnrf_NFDiscovery request to discover UPF(s) that support energy related information to be collected by OAM

- NRF discovers UPF(s) that support energy related information to be collected by OAM

- NRF respond with a list of UPF(s) that support energy related information to be collected by OAM

6.X.3	Impacts on services, entities and interfaces

NRF:

- Support of energy related information to be collected by OAM

* * * End of change * * * *