3GPP TSG-SA WG2 Meeting #169	S2- 2505189

19 - 23 May, 2025, Fukuoka, Japan

Title:	23.700-19: New Solution for WT#2.2: UE-SAT-UE communication via UPF onboard satellite for non-IMS services

Source:	ETRI

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract:	This contribution proposes a solution to support Work Task WT-2.2: UE-to-UE communication via UPF deployed only onboard NGSO (LEO/MEO) satellites for non-IMS services.

Proposal

The following Solution is proposed for inclusion in TR 23.700-19 to address the work task WT-2.2, defined in the SID titled “Study on Integration of Satellite Components in the 5G Architecture Phase 4” (SP-250400):

-	WT-2.2 UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, e.g. a 5G-LAN service with IP and Ethernet PDU Session type.

* * * First Change * * * *

6.X	Solution #X: Local Switch for UE-SAT-UE Communication via UPF onboard Satellite

6.X.1	High level principles

Editor's note:	This clause will describe the solution principles.

This solution introduces support for non-IMS UE-to-UE communication using local switching via User Plane Functions (UPFs) deployed onboard NGSO satellites (LEO/MEO). The Packet Session Anchor (PSA) UPF remains on the ground, while UL Classifier/Branching Point (UL CL/BP) and Local PSA (L-PSA) UPFs are onboard the satellite. The goal is to enable direct satellite-based switching of user plane traffic between two UEs under the coverage of the same or connectable satellites, without breaking UE IP continuity.

This solution reuses the local switching model for GEO satellites defined in clause 5.43.3.3 of TS 23.501 [2] and adapts it for NGSO constellation. The architecture supports both IP and Ethernet PDU Session types and ensures consistent SMF control over both UEs and satellite resources.

6.X.2	Description

Editor's note:	This clause will describe high-level solution. (Sub) clause(s) may be added to capture details.

In this solution:

- The PSA UPF is located on the ground, anchoring the UE's IP address.

- UL CL/BP and L-PSA UPFs are deployed onboard the NGSO satellite to enable localized switching of UE traffic.

- The SMF makes routing decisions based on satellite ID, AF request input, and uplink traffic analysis.

- If both UEs are under the same satellite or connectable satellites, the SMF may activate local switching by inserting UL CL/BP and L-PSA onboard the satellite.

Uplink traffic from UE is first routed to onboard UL CL The UL CL forwards packets to L-PSA if the destination IP matches another UE in the group; otherwise, traffic is forwarded to the ground-based PSA. The L-PSA UPF performs local switching based on IP address matching. For UEs under different satellites, N6 interfaces or N19 tunnels may be used to interconnect the onboard L-PSAs to complete the UE-SAT-UE path.

6.X.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

(1) Local switching activation:

- The SMF receives an AF request (clause 5.29.2 of TS 23.501[2]) or destination IP information (5.8.5.7 of TS 23.501 [2]).

- The SMF verifies that both UEs are under the same or connectable satellites based on satellite ID reported by the AMF.

- Following verification, the SMF selects the satellite UPF via DNAI and inserts UL CL/BP and L-PSA on satellite while retaining the PSA UPF on the ground.

(2) N4 Session management:

- SMF configures UL CL with rules to forward uplink traffic destined for peer UE to the L-PSA.

- The L-PSA is configured with local switching rules to route traffic directly to the peer UE.

- If UEs are on different satellites controlled by the same SMF, inter-UPF tunnels (N6 or N19) may be established between L-PSAs.

(3) Satellite movement:

- When the serving satellite changes due to satellite or UE mobility, the AMF reports target satellite ID to the SMF.

- The SMF decides to maintain or reassign UL CL/BP and L-PSA on the new satellite, following the procedure in clause 4.3.5.7 of TS 23.502 [3].

- If no onboard UPF resources are available, the SMF falls back to PSA-only routing. The PDU session remains anchored to the ground PSA, and the UE IP address remains unchanged.

6.X.4	Impacts to Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

Services:

Enables UE-SAT-UE communication for non-IMS traffic (e.g., 5G LAN-type services).

Entities:

SMF: Selects and inserts satellite-based UL CL/BP and L-PSA; configures N4 rules; manages N6/N19 tunnels as required.

AMF: Reports satellite ID/DNAI to SMF.

UPF (UL CL/BP and L-PSA onboard satellite): Performs local switching.

PSA UPF (ground): Anchors the IP session and ensures IP continuity.

Interfaces:

N4: Used to configure UL CL/BP and L-PSA rules.

N19/N6: Used to route traffic between L-PSAs onboard different satellites.

* * * End of Changes * * * *