3GPP TSG-SA2 Meeting #169	S2-2505190

Fukuoka, Japan, May 19th – 23rd 2025	was S2-2503800

Source:	Huawei, HiSilicon, CAICT

Title:	High-level Procedure and Architecture of the Sensing service

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Providing on the high-level Procedure and architecture for FS_Sensing_ARC.

1. Introduction

This pCR proposes the overall call flow for sensing, which touches all KIs in the study. Specifically, a Sensing Function, SF, is introduced to manage, as far as possible all the sensing related functionality within 5GC, so as to provide an extensible solution.

The goal of this solution is to create an agreeable framework for the high-level sensing procedure steps. The details for each of the logical operations in the framework can be further elaborated in dedicated solutions. For example, in this solution, the Sensing Function discovery/selection is denoted by one single "box" (i.e., in a very general way), and the details/procedures/criteria regarding this will be further addressed by the solutions of KI#3.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X | X | X | X | X | X | X |

# * * * * Second change (All Text New) * * * *

6.X	Solution #X: High-level procedure and architecture for sensing

6.X.0	High level principles

The solution follows the following principles:

-	Provide a general framework for sensing, that captures all the operations related to the sensing service and can be used as a common baseline;

-	intentionally keep the general procedure at high level so that details can be described in solution to specific Key Issues.

6.X.1	Description

6.X.1.1	Terms used within this solution

For the purposes of the present solution the following definition applies:

Sensing service subscription: Subscription data regarding sensing service, which may include the subscription for the Sensing service consumer (which will be managed by the NEF), and for the requested parameters of the Sensing service consumer (which will be managed by the Sensing Function).

6.X.1.2	Solution description

This solution aims to provide the general procedure to support the sensing functionality.

The main principles of the procedure are:

-	the AF acting as the Sensing Service Consumer initiates the procedure for sensing by sending the sensing service subscription request to the Core Network.

-	The Network Exposure Function determines an appropriate Sensing Function according to the request from the AF.

-	Upon receiving a request for subscription to sensing service from the AF, the Sensing Function proceeds as follows:

-	Discovers the Sensing Entity/Entities and provides the configuration parameters to them;

-	May establish the path for the reporting of the 3GPP sensing data from the Sensing Entity/Entities if not done yet. The reporting of 3GPP sensing data (as defined in TS 22.137 [2]) is performed via user plane.

-	The 3GPP sensing data is transmitted to the Sensing Function, which further processes the data, obtains the Sensing Result (as defined in TS 22.137 [2]) and exposes the Sensing Result to the AF.

The following Figure 6.X.1.2-1 provides the reference architecture baseline for sensing.

Figure 6.X.1.2-1: Reference Architecture.

6.X.2	Procedure

6.X.2.1	General procedure

Figure 6.X.2.1-1: Procedure of the sensing general framework.

1.	The Sensing Service Consumer (i.e., AF) requests the NEF to obtain the Sensing result from the network via Ngw_SensingService_Subscribe Request.

NOTE 1:	Detailed information of sensing service requirement, sensing service type and corresponding information is addressed by solutions to Key Issue #2 and/or Key Issue #5.

2.	Upon receiving request from AF, the NEF authorizes the AF for usage of the Sensing Service.

3. 	The NEF may discover and select the candidate Sensing Function(s).

NOTE 2:	Details regarding NEF authorising the AF is addressed by solutions to Key Issue 2.

4.	The NEF sends the request to the Sensing Function, where the request message contains information received in step 1.

5.	The Sensing Function may perform further authorisation for the sensing request based on the parameters provided by the AF.

NOTE 3:	Details regarding Sensing Function authorising the AF's request is addressed by solutions to Key Issue #2.

NOTE 4:	the Sensing Function or NEF may respond to the upstream nodes indicating the result of sensing service subscription.

NOTE 5:	Details regarding Sensing Function discovery/selection is addressed by solutions to Key Issue #3.

6.	If the Sensing Function performs authorisation, it may indicate whether the request has been authorised or rejected via Nsf_SensingServiceExposure_Subscribe Response.

7. 	The NEF notifies the AF whether the subscription has been authorised or rejected via Ngw_SensingService_Subscribe Response.

8	The Sensing Function performs the discovery and selection for the Sensing Entity, based on e.g., the sensing service requirement, and sensing capability supported by the Sensing Entity.

NOTE 6:	The details regarding the discovery and selection of the Sensing Entity are addressed by the solution to Key Issue #3.

9.	The Sensing Function determines the information for the subsequent Sensing measurement and sends the sensing service request message including the configuration parameters to the Sensing Entity.

NOTE 7:	Details regarding Sensing Function sending the configuration parameters is addressed by solution to Key Issue 1 and Key Issue 6.

10.	Upon receiving the sensing service request message from the Sensing Function, the Sensing Entity may establish the path for 3GPP sensing data reporting if the path has not been established yet.

11.	The Sensing Entity performs the Sensing measurement and obtains 3GPP sensing data.

12.	The 3GPP sensing data is transmitted to the Sensing Function.

NOTE 8:	Details regarding the 3GPP sensing data and the reporting is addressed by the solution to Key Issue 4.

13.	After receiving the 3GPP sensing data, the Sensing Function determines the Sensing result.

14.	The Sensing Function sends the Sensing result to the NEF via Nsf_SensingServiceExposure_Notify.

15.	The NEF provides the Sensing result to the AF via Ngw_SensingService_Notify.

NOTE 9:	The steps 11-15 may be performed one-time or a periodically or based on event trigger depending on the sensing service request from AF.

NOTE 10:	Details regarding the Sensing result is addressed by the solution to Key Issue #5.

NOTE 11:	Details regarding the procedure between the Sensing Function and the Sensing Entity from step 9 to 12 will be addressed by the associated solutions.

6.X.2.2	CP-UP Split of the Sensing Function

To enable flexible scaling of Sensing Functions, the Sensing Function in clause 6.X.2.1 is further split into Sensing Control Function (SCF) and the Sensing Processing Function (SPF). The Sensing Control Function can be located in a central area which can control multiple distributed Sensing Processing Functions.

With the Sensing Function Splitt, steps 4-6 and steps 8-9 in Figure 6.X.2.1-1 are performed by the Sensing Control Function, while step 10, step 12 and step 13 are performed by the Sensing Processing Function.

Figure 6.X.2.2-1: Procedures regarding Sensing Control Function and Sensing Processing Function.

Figure 6.X.2.2-1 presents the interactions between SCF and SPF. After SCF received a sensing request in step 8 in Figure 6.X.2.1-1, the SCF sends a Sensing Processing Configuration Request to SPF to request the SPF for the following execution in step 3. After SPF determined sensing result, it sends a Sensing Processing Result Report to SCF. The Report may be performed one-time or a periodically or based on event trigger depending by step 2.

6.X.3	Impacts on Services, Entities and Interfaces

NEF:

-	Selection of Sensing Function.

-	Interaction with AF and Sensing Function for sensing authorization, invocation and revocation.

-	Provision the Sensing result retrieved from Sensing Function to AF.

Sensing Function:

For the Sensing Control Function:

-	Interaction with NEF for exposure of Sensing Results and reception of sensing requests.

-	Interaction with sensing entities for requesting to perform sensing measurement.

-	Interaction with Sensing Process Function.

For the Sensing Process Function:

-	Reception of 3GPP Sensing Data;

-	Determine the Sensing Result based on the 3GPP Sensing data.

-	Interaction with Sensing Control Function.

AF:

-	Interaction with 5GC for requesting sensing service to obtain the Sensing result(s).

-	Acquisition Sensing result(s) from 5GC.

# * * * * End of changes * * * *