3GPP TSG-SA2 Meeting #169	S2-2505192

Fukuoka, Japan, May 19th – 23rd 2025

Source:	Huawei, HiSilicon, CAICT

Title:	KI#3, New Sol: Sensing Entity and Sensing Function Discovery and (Re-)Selection

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The Discovery and selection of sensing entities for FS_Sensing_ARC is proposed.

1. Introduction

This pCR proposes the solution for Key issue#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[x1]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  |  | X |  |  |  |

# * * * * Second change (All Text New) * * * *

6.X	Solution #X: Discovery and selection of Sensing Function

6.X.0	High level Principle

This solution is based on the High-level Procedure and Architecture documented in Sol#X and address KI#3. The principles of this solution are:

- 	The Sensing functionality is composed by the Sensing Control Function and Sensing Processing Function.

-	The NEF requests NRF for Sensing Control Function discovery and then select Sensing Control Function based on the candidate Sensing Control Functions provided by the NRF. NEF determines the Sensing Control Function based on the supported Sensing Service Area of the Sensing Control Function and Target Sensing Service Area requested by the AF.

-	The Sensing Control Function selects the Sensing Processing Function based on Target Sensing Service Area requested by the AF and SPF Service Area.

-	The Target Sensing Service Area can span more than one Sensing Service Area and the Target Sensing Service Area can be smaller than the Sensing Service Area.

NOTE:	Whether the Sensing Service Area can be a dedicated TA/cell is to be determined during conclusion.

6.X.1	Description

For the purposes of the present solution the following definition applies:

Sensing Service area: The Sensing service area supported by the Sensing Control Function in which the Sensing Control Function provides the Sensing Result.

Sensing Control Function: Indicating the logical function which is involved to support Sensing Service and responsible of selecting the Sensing Entities and of providing the related configuration information.

Sensing Processing Function responsible of receiving the 3GPP Sensing Data and of performing the elaboration for determining the Sensing Result,

Sensing Service type: indicating the Sensing scenarios and it includes Object Detection within a Target Sensing Service Area, Border Crossing Detection, Path Monitoring and Path Tracking.

SCF Service Area: The collection of SPF Service Areas of all SPFs which can be controlled by one SCF.

SPF Service Area: An area consisting of one or more Sensing Service Area, and within which the generated 3GPP Sensing Data by gNB(s) can be sent to the SPF directly.

Sensing Service Area: a logical concept of an area where the gNB can obtain the 3GPP Sensing data without interacting with other sensing entities.

NOTE:	The Sensing Control Function and Sensing Processing Function cannot be a Sensing Entity.

6.X.2	Procedure

NOTE:	This sub-clause provides the details of the discovery and selection of Sensing Function procedure, and corresponds to step 1 and 3 of the sensing general framework.

The NEF requests NRF for Sensing Control Function discovery and then select Sensing Control Function(s) from the candidate Sensing Control Function list provided by the NRF. The NEF selection of the Sensing Control Function is based on the SCF Service Area of the Sensing Control Function and Target Sensing Service Area requested by the AF.

The Sensing Control Function may register itself to the NRF, update its information, or deregister itself, as defined in clause 4.17.1.1, 4.17.2, and 4.17.3 of TS 23.502 [x1], respectively. In case of registration, the Sensing Control Function sends the Nnrf_NFManagement_NFRegister including the Sensing Control Function capability information, e.g., SCF Service Area.

The NEF performs the discovery of Sensing Control Function as defined in TS 23.502 [x1] clause 4.17.4, with the following modifications:

-	the Nnrf_NFDiscovery_Request message sent to NRF contains the Sensing Service information, i.e., the Target Sensing Service Area, and the required sensing service type (i.e., Object Detection, Border Crossing Detection, Path Monitoring and Path Tracking) derived from the request sent by AF to NEF.

-	The NRF authorizes the request and determines whether the NEF is allowed to discover the expected Sensing Control Function instance(s).

-	if the target NF is the Sensing Control Function, the NRF determines a set of Sensing Control Function instance(s) matching the information within the Nnrf_NFDiscovery_Request message based on internal policies, the SCF Service Area of the Sensing Control Function overlapping the Target Sensing Service Area and the, Sensing Service Type.

The following information are included in the Sensing Control Function profile in the NRF:

-	SCF Service Area.

-	Supported Sensing Service Type i.e., Object Detection, Border Crossing Detection, Path Monitoring and Path Tracking.

Once the NEF received the candidate Sensing Control Function, the following factors may be considered for Sensing Control Function discovery and selection by NEF:

-	SCF Service Area.

- 	Sensing Capabilities.

-	Sensing Service requirements sent by AF.

-	If the Target Sensing Service Area matches more than one SCF Service Area, the NEF can select all required Sensing Control Function(s) and provide the sensing service request received from the AF to the selected Sensing Control Function(s).

Sensing Processing Function reports its serving area, i.e., SPF Service Area, to Sensing Control Function when associated with the Sensing Control Function, the Sensing Control Function selects one or more than one Sensing Processing Function(s) based on Target Sensing Service Area and SPF Service Area(s), which ensures that the selected Sensing Processing Function(s) can serve the Target Sensing Service Area.

6.X.3	Impacts on services, entities and interfaces

NEF:

-	Support Sensing Control Function discovery and selection

NRF:

-	Support Sensing Control Function capability registration.

Sensing Control Function:

-	Support Sensing Processing Function discovery and selection.

# * * * * End of changes * * * *