3GPP TSG-SA2 Meeting #169	S2-2505193

Fukuoka, JP, May 19th – 23rd 2025

Source:	Huawei, HiSilicon

Title:	KI#5, New Sol: Sensing Result Exposure

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The solution on providing Sensing Result for FS_Sensing_ARC is proposed.

1. Introduction

This pCR proposes the solution for Key issue#5 Sensing Result Exposure.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  |  |  |  | X |  |

# * * * * Second change (All Text New) * * * *

6.X	Solution #X: Sensing Result Exposure

6.X.0	High-level solution principles

This solution is based on the High-level Procedure and Architecture documented in Sol#X and address KI#2. This solution is based on the following principles:

-	The Sensing functionality is composed of a Sensing Control Function and a Sensing Processing Function as defined in clause 6.X.1.1.

-	The use cases defined in TS 23.137 [2] are classified to the sensing service type, namely Object Detection, Border Crossing Detection, Path Monitoring and Path Tracking.

-	The exposure of Sensing Result to Sensing Service Consumer is performed by NEF.

-	The Sensing Service Consumer is the AF.

-	The type of request for exposure of Sensing Result can be one time, periodical or event subscription.

-	The Target Sensing Service Area can be provided based on geographical information (e.g. a polygon as defined in TS 23.032 [x]) or using 3GPP specific parameters.

-	The AF may submit a follow-up request referring to previous request(s) based on previous Sensing results.

6.X.1	Description

6.X.1.0	Solution scope

This solution aims to address key issue#5 Providing Sensing Result and other related information.

6.X.1.1	Terms used within this solution

For the purposes of the present solution the following definition applies:

Object Identifier: an identifier that the Sensing Function allocates to a detected object and that can be used by the Application Function as input to subsequent sensing service requests

Result Identifier: an identifier of the results of the sensing provided to the requesting AF. It can be used by the AF and Sensing Function to perform additional requests.

6.X.1.2	Sensing Service types

The Sensing Function can support the following sensing service types:

-	Object Detection within a Target Sensing Service Area: the detection of the objects within the Target Sensing Service Area

-	Border Crossing Detection: the detection of the object that crosses the border of the Target Sensing Service Area, for example to detect objects that enter or exit a predetermined area

- 	Path Monitoring: the detection and monitoring of object(s) present in a path, for example to monitor the status of a road, the object(s) present on a road, etc...

-	Path Tracking: the tracking of detected object(s), for example to report position, heading and speed of object(s) in a Target Sensing Service Area.

The input, output and type of request information for the above sensing service types are:

-	Object Detection within a Target Sensing Service Area:

-	Input: The AF provides the Target Sensing Service Area requesting a list of the detected object(s) within the Target Sensing Service Area. Some filtering criteria for the objects to be detected may also be provided.

-	Output: Sensing Control Function provides the Sensing Result including the list of detected objects with associated sensing information (e.g., object identifiers and result ID) within the Target Sensing Service Area that match the filter criteria (if provided). Other Sensing Contextual Information may be also provided

-	Type of request: one time or periodical.

NOTE 1: 	the AF may issue a follow up request based on the Sensing Results e.g. to obtain additional information on a detected object.

-	Border Crossing Detection:

-	Input: The AF provides the Target Sensing Service Area (the Border is the boundary of the target service area) requesting the Sensing Function to provide an alert when it detects an object(s) crossing the border. The AF can specify whether the alert is provided only when an object crosses the border in a specific direction (e.g. in/out or out/in) or in both directions.

-	Output: when the Sensing Control Function detects a border crossing matching the direction requested from the AF, it provides the Sensing Result including the indication whether the object is moving in or out of the Target Sensing Service Area and its location. Other Sensing Contextual Information may be also provided.

-	Type of request: subscription to event.

-	Path Monitoring

-	Input: The AF provides the Target Sensing Service Area (which in this case corresponds to the path to be monitored) requesting that the Sensing Result provide a list of the object(s) in the monitored path.

-	Output: Sensing Control Function provides the Sensing Result including list of object(s) with associated sensing information matching the criteria, if any. Other Sensing Contextual Information may be also provided.

-	Type of request: subscription to event or periodical.

NOTE 2:	The path monitoring service type is a superset of the Border crossing detection service type where the border corresponds to the boundary of the path.

-	Path Tracking:

-	Input: The AF provides the Target Sensing Service Area requesting the Sensing Result to track the object(s) within the Target Sensing Service Area (i.e., position, heading, speed).

-	Output: Sensing Control Function provides the Sensing Result including for each sensed object a vector with position, heading and speed with associated sensing information matching the criteria, if any. Other Sensing Contextual Information may be also provided. The Sensing Result may include a result ID and an object identifier for each object discovered. The result ID and object identifier may be used by the AF for follow up request(s) to indicate to the Sensing Control Function which objects are to be tracked.

-	Type of request: subscription to event or periodical

6.X.1.3	First Request

The solution considers the case where the AF may need to perform a first Sensing Service request over a Sensing Target Area to identify objects of interest. The sensing results will return a result ID and an enumeration of the objects detected that the AF can use as inputs in follow-up requests.

6.X.2	Procedure

NOTE 1:	This sub-clause provides the details of sensing request and sensing result exposure. Specifically, step 1 and step 2 of this procedure correspond to step 1 and 4, while step 5 and step 6 correspond to step 14 and step 15, of the sensing general framework in Sol#X.

Figure 6.X.2-1: Sensing Result exposure related procedure.

1.	The AF sends Nnef_SensingService_Subscribe request message to the NEF, the request message contains the Target Sensing Service Area, the Sensing Service type, optionally, and if previously provided in Nnef_SensingService_Notify message, the Result ID and Object ID(s) This information is defined in clause 6.X.1.1.

2.	The AF request is authorized. For details of the authorisation procedure at NEF level, refer to KI#2 solutions. It is assumed in this procedure that the AF is successfully authorised.

3.	NEF may discover and select the candidate Sensing Control Function(s). For details of the discovery and selection of Sensing Control Function(s) refer to KI#3 solutions.

4.	The NEF sends the AF request to the Sensing Control Function(s) via Nscf_SensingServiceExposure_Subscribe Request message appending the AF ID.

5-7.The Sensing Control Function may perform further authorisation for the requested Sensing Service type taking into account input parameters and AF ID. For details of the authorisation procedure at the Sensing Control Function, refer to KI#2 solutions. It is assumed in this procedure that the sensing service is successfully authorised.

8.	The Sensing Function performs the discovery and selection for the Sensing Entity/Entities. If the Sensing Service request contained a result ID and Object ID(s), the Sensing Control Function takes them into consideration for the discovery and selection of the Sensing Entity and may pass the relevant object ID to the Sensing Processing functions involved in the sensing operation. For Details of the discovery and selection of Sensing Entity/Entities involved in sensing refer to KI#3 solutions.

9.	Once the Sensing Processing Function(s) has obtained 3GPP Sensing data from Sensing Receiver(s), the Sensing Control Function initiates the sensing data processing task on Sensing Processing Function. For details of how the 3GPP Sensing data are obtained, refer to KI#4 and KI#6 solutions.

10.	After receiving the 3GPP Sensing Data, the Sensing Processing Function Determines the Sensing Result.

11. The Sensing Processing Function reports the sensing result to Sensing Control Function.

12.	Sensing Control Function assigns a result ID and Object ID(s) to each the detected object and includes this information in the Sensing Result which is provided to NEF using Nscf_SensingServiceExposure_Notify. The AF may use the result ID and Object ID(s) in follow up Sensing Service requests.

NOTE 2: 	Possible use of the object IDs includes narrowing down the Target Sensing Area, indicate to the Sensing Control function which specific object(s) to track, subscribe to event of a specific object crossing a boundary and so on.

13.	The NEF then provides the Sensing Result to the AF using Nnef_SensingService_Notify which the NEF forwards transparently. The corresponding output of each Sensing Service type are defined in clause 6.X.1.1:

The steps from 9 to 12 may be repeated in case of periodical reporting.

6.X.3	Impacts on Services, Entities and Interfaces

NEF:

-	Support Sensing Result and other related information provisioning.

Sensing Function with Sensing Control Function and Sensing Processing Function:

-	Support the determination of the Sensing Result by processing the 3GPP Sensing Data.

-	Support Sensing Result and other related information provisioning.

AF:

-	Support Sensing Service requirement, Sensing Service type and corresponding information provisioning, and Sensing Result acquisition.

# * * * * End of changes * * * *