3GPP TSG-SA2 Meeting #169	S2-2505197

# Fukuoka, Japan, May 19 – May 23, 2025   									   (revision of S2-2504417)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6255 | rev | 6 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Available Bitrate Reporting when the Serving RAN Node Changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [InterDigital Inc.], Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When the serving NG-RAN node changes, it can not be assumed that the target NG-RAN node supports Available bitrate Reporting and it can not be assumed that the target NG-RAN node can provide the same Available Bitrate that was last reported by the source NG-RAN. The clause on Available bitrate monitoring should reference the reporting procedure in clause 4.15.12 of TS 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added text to say that the SMF can notify the UPF when the serving NG-RAN node changes so that the UPF can report that the last reported Available bitrate is no longer valid. Adding also text that the Target NG-RAN if it supports Available bitrate Reporting it does so immediately after the handover. Added a reference to clause 4.15.12 of TS 23.502. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AS might assume an Available bitrate that can not be supported. There would be no reference to clause 4.15.12 of TS 23.502 so the reporting procedure would be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.45.1, 5.45.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.45.6	Available bitrate monitoring

The QoS Monitoring for available bitrate allows the measurement and reporting of the UL and/or DL available bitrate for a GBR QoS Flow without Alternative QoS Profiles by the NG-RAN. The available bitrate refers to the bitrate that NG-RAN is able to provide in the respective direction for a QoS Flow within a duration. The duration is based on RAN implementation.

NOTE 1:	How the NG-RAN measures the available bitrate is NG-RAN implementation specific.

NOTE 2:	The available bitrate can be larger than the GFBR but can never exceed the MFBR. The available bitrate can also be smaller than the GFBR for a QoS Flow with QoS Notification Control enabled without Alternative QoS Profiles.

For the reporting of the available bitrate, periodic reporting is not applied and only event triggered reporting applies. Based on the PCC rule containing available bitrate as QoS monitoring parameter and Reporting threshold(s) per direction in the QoS Monitoring policy (as described in clause 6.1.3.27.1 of TS 23.503 [45]), the SMF configures the NG-RAN to measure and report the available bitrate by sending QoS monitoring request including the threshold(s). Based on the QoS monitoring request, if the bitrate that NG-RAN is able to provide changes and matches or exceeds indicated threshold(s), the NG-RAN reports the UL and/or DL available bitrate information of the highest fulfilled threshold(s) for the UL and/or DL direction to the PSA UPF via the GTP-U header of UL packets (or dummy UL packets). The NG-RAN shall only send a report when the available bitrate information is different from the previously reported value(s) for at least one direction. If thresholds exist for both directions, the NG-RAN shall always indicate the available bitrate information for both directions when sending a report.

.

NOTE 3:	The NG-RAN is expected to immediately report the UL and/or DL available bitrate when it matches or exceeds a lower threshold value and report the UL and/or DL available bitrate when it matches or exceeds a higher threshold value with some hysteresis.

The SMF may configure the PSA UPF to perform QoS monitoring for available bitrate as described in clause 5.8.2.18. For the reporting of the available bitrate via the PSA UPF, only event-based reporting applies in the PSA UPF. Reporting by the UPF is performed as described in clause 4.15.12 of TS 23.502 [3]. The Reporting threshold(s) in the PCC rule are not sent to UPF. In the UPF, the event for the reporting is the reception of available bitrate information from NG-RAN. According to the QoS Monitoring request for the available bitrate from the SMF, the PSA UPF monitors and exposes the available bitrate information reported from NG-RAN to the target NF as instructed.

During handover, the QoS Monitoring configuration is provided to the Target NG-RAN per QoS Flow in the QoS Monitoring configuration. The SMF can notify the UPF when the serving NG-RAN node changes so that the UPF can report that the last reported Available bitrate is no longer valid. If the Target NG-RAN supports QoS Monitoring for available bitrate it shall report it for the UL and/or DL direction immediately after the handover.

NOTE 4:	The AF can subscribe to reporting for the change in  support for QoS monitoring as described in clause 5.45.1 to receive notifications when support of Available Bitrate monitoring changes in the Target NG-RAN.

# * * * * Next change * * * *

### 5.45.1	General

QoS monitoring comprises of measurements of QoS monitoring parameters and reports of the measurement result for a QoS Flow and can be enabled based on 3rd party application requests and/or operator policies configured in the PCF. Event Reporting from PCF is specified in clause 6.1.3.18 of TS 23.503 [45] and User Plane QoS Flow related QoS monitoring and reporting in clause 5.8.2.18.

The AF may request measurements for one or more of the following QoS monitoring parameters, which may trigger QoS monitoring for service data flow(s):

-	UL packet delay, DL packet delay, round trip packet delay for a service data flow, see clause 5.45.2.

-	Congestion, see clause 5.45.3.

-	Data Rate, see clause 5.45.4.

-	Packet Delay Variation, see clause 5.37.7.

-	Round trip packet delay considering UL on a service data flow and DL of another service data flow, see clause 5.37.4.

-	Available Bitrate, see clause 5.45.6.

The following AF requested QoS requirements may trigger QoS monitoring for service data flow(s):

-	Round-trip latency requirement, see clause 5.37.6.

The PCF may generate the authorized QoS Monitoring policy for a service data flow based on the QoS Monitoring request received from the AF (as described in clause 6.1.3.21 of TS 23.503 [45]) or based on PCF local policy or configuration reasons, such as PCF awareness of dynamic satellite backhaul connection. The PCF includes the authorized QoS Monitoring policy in the PCC rule and provides it to the SMF.

The QoS monitoring parameter(s) that can be measured by means of QoS monitoring are listed below. The QoS monitoring policy in PCC rule (described in clause 6.3.1 of TS 23.503 [45]) may include the following:

-	UL packet delay, DL packet delay, round trip packet delay, see clause 5.45.2.

-	Congestion, see clause 5.45.3.

-	Data Rate, see clause 5.45.4.

-	Available Bitrate, see clause 5.45.6.

The SMF configures the UPF to perform QoS monitoring for the QoS Flow and to report the monitoring results as described in clause 5.8.2.18 with parameters determined by the SMF based on the authorized QoS Monitoring policy received from the PCF or local configuration or both.

The SMF may also configure NG RAN to measure the QoS monitoring parameters by sending QoS monitoring request based on the authorized QoS Monitoring policy received from the PCF and/or local configuration. The QoS monitoring request to the NG RAN for different parameters is as defined in clause 5.45.2, 5.45.3 and 5.45.6.

Based on configuration, the AMF may know whether  the QoS monitoring feature (i.e. each of Packet delay monitoring and/or Congestion monitoring and/or Available Bitrate monitoring) is supported by all NG RAN nodes in an area. The AMF shall, when configured, inform the SMF about the support of the QoS monitoring feature by sending the parameter NG RAN QoS monitoring capability during PDU Session establishment, PDU Session modification, Service request and UE mobility procedures. The SMF determines whether QoS monitoring is possible or not for the PDU Session based on whether the NG RAN QoS monitoring capability is received (or not) from the AMF and the QoS monitoring capability of the UPF. QoS monitoring is possible if the SMF has received the NG RAN QoS monitoring capability in the last message received from AMF (i.e. Nsmf_PDUSession_CreateSMContext or Nsmf_PDUSession_UpdateSMContext service operation) and the UPF also supports the corresponding QoS monitoring capability. Otherwise, QoS monitoring is not possible for the PDU Session for the respective QoS monitoring parameter(s). The SMF may need to notify the PCF when the support for QoS monitoring has changed as described in clause 6.1.3.5 of TS 23.503 [45] also indicating the affected QoS monitoring parameter(s).

NOTE 1:	The AMF is not required to immediately update the SMF about a change of support of the QoS monitoring feature (by all NG RAN nodes in an area) when the change is happening outside of the UE mobility procedures. As AF logic can depend on the QoS monitoring reports, the change from support of QoS monitoring to no support of QoS monitoring is more important to notify than the change in the other direction and is therefore expected to take place in a timely manner.

The following clauses describe the QoS monitoring parameters which can be measured and any specific actions or constraints for their measurement.

NOTE 2:	The QoS monitoring parameter which can be measured are parameters which describe the QoS experienced in the 5GS by the application, i.e. they are not restricted to the 5G QoS Parameters defined in clause 5.7.2.

# * * * * End of changes * * * *