3GPP TSG-SA WG2 Meeting #169	S2-2505200

Fukuoka, Japan, May 19 – 23, 2025

Source:	Xiaomi

Title:	KI#4: New Sol: Location Provision for IMS emergency call over NB-IoT via GEO

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution for KI#4.

1. Introduction/Discussion

This solution addresses KI#4 to provide UE location for IMS emergency call over NB-IoT via GEO.

2. Proposal

It is proposed to capture the following text in FS_5GSAT_Ph4_ARC TR 23.700-19.

* * * Start of Change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| x |  |  |  | x |

6.X	Solution #x: Location provision for the IMS emergency call over NB-IoT via GEO

6.x.1	High level principles

High level solution principles are:

-	UE coarse location information defined in TS 23.401 is used for the IMS emergency call.

-	The EPC Network Induced Location Request procedure defined in TS 23.271 can be reused to acquire UE location for the IMS emergency call.

6.x.2	Description

As specified in TS 23.167, the purpose of UE location information is to enable the IMS network to determine a suitable PSAP and to get more accurate or updated location information during the emergency session required by local regulation. The UE or EPC NF (e.g. MME) provided location information or the location information through positioning can be used for the emergency session purpose.

This contribution proposes the UE provided coarse location information specified in TS 23.401 or the UE location information via EPC-NI-LR defined in TS 23.271 can be used for the IMS emergency session.

6.x.3	Procedures

Figure 6.x.3.1-1: Handling of location information in IMS emergency call over NB-IoT via GEO

The procdure defined in clause 7.6.1 of TS 23.167 can be reused to acquire the UE location information by IMS core with the following additions:

2.	The UE sends an INVITE with an emergency indication to the IMS core. The INVITE contains the UE coarse location information if UE provided.

4.	when the MME receives a location request from the LRF, if it has not the UE location information, it may request the UE to report its location or trigger the EPC-NI-LR procedure defined in TS 23.271 to acquire the UE location information.

Editor's note:	whether other location information (e.g. serving cell, TAI) is required for supporting IMS emergency session is FFS.

6.x.4	Impacts on services, entities and interfaces

UE:

-	Sends UE coarse location information to the IMS core if available

MME:

-	Provides UE coarse location information to the IMS core if required

* * * End of Change * * * *