SA WG2 Meeting #169	S2-2505230

Fukuoka, Japan, 19 – 23 May 2025                   	(revision of xxx)

Source: 	CATT

Title: 	New Solution for UE-SAT-UE communication via 5G-LAN

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes the solution for UE-SAT-UE communication via 5G-LAN.

1 Discussion

This solution addresses the support for UE-SAT-UE communication via 5G-LAN onboard LEO/MEO satellites. Based on the 5G VN Group defined in TS 23.501, considering the specific constraints such as satellite access/mobility and N6 connectivity limitations, this solution utilizes UL-CL/BP and L-PSA functions on satellites to support local traffic switching between 5G VN group members.

1.1 Architecture considerations

The architecture shall consider the following aspects:

-	The 5G LAN-type service defined in TS 23.501 clause 5.29, a dedicated SMF, a dedicated SMF Set, or multiple SMF (Sets) can be responsible for all the PDU Sessions for communication of a certain 5G VN group, otherwise, only a single SMF is supported for local switching in 5G Satellite Backhaul scenario defined in TS 23.501 clause 5.43.

It is proposed to align with basic 5G LAN-type service to support more than one SMF set or SMF instances in an SMF Set to enable SMF redundancy for reliability of the 5G VN group communication for this solution.

1.2 SSC mode considerations

The SSC mode shall consider the following aspects:

-	SSC mode 1 should be the baseline mode for this solution as UE supporting PDU connectivity shall support SSC mode 1, with SSC modes 2 and 3 being optional as SSC mode 2/3 is optional to be supported in the UE as described in TS 23.501 clause 5.6.9.

-	SSC mode 1, SSC mode 2 and SSC mode 3 are applicable to N6-based traffic forwarding of 5G VN communication within the associated 5G VN group as described in TS 23.501 clause 5.29.3.

1.3 Traffic forwarding considerations

For UEs belonging to the same 5G VN Group, traffic shall be forwarded according to the following aspects:

-	When UEs access in the same satellite, the L-PSA UPF shall directly switch traffic locally based on the destination IP/MAC address.

-	When UEs access in different satellites and inter-satellite links (ISL) are available, traffic shall be forwarded via N19 or N6 between the different L-PSAs on different satellites.

-	When neither condition 1 nor 2 is met, fall back to forward via the ground PSA UPF, which may introduce additional latency but ensures connectivity.

-	For broadcast and multicast communication scenario, the satellite changes could bring many problems, e.g., how to distinguish whether the broadcast has finished when satellite changing and how to resolve it, so the SMF should restrict UE-Sat-UE to support broadcast and multicast communication.

With considerations above, the “6.x.1 High level principles” clause shows the proposal.

2 Proposal

It is proposed to add a new solution for UE-SAT-UE communication via 5G-LAN to TR 23.700-19 V0.1.0.

******************** 1st Change ********************

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 |
| #x |  |  |  |  | X |

******************** 2nd Change ********************

6.x	Solution for UE-SAT-UE communication via 5G-LAN

6.x.1	High level principles

The high level principles of the solution include:

-	Reuse the general logic of local switch for UE-to-UE communications via UPF deployed on GEO satellite for 5G Satellite Backhaul scenario as defined in TS 23.501[2] clause 5.43.3, as follows:

-	Baseline for this solution: Local switching with UL CL/BP and local PSA UPF deployed on satellite

-	Option for this solution: Local switch with PSA UPF deployed on satellite.

-	Based on the above reused general logic, the enhancements for this solution mainly include:

-	SMF supports to activate local switching when the type of serving satellite is LEO or MEO.

-	SMF determines whether to continue supporting UE-Satellite-UE communication when the serving LEO/MEO satellite changes, which may refer to the support of UE-Satellite-UE communication in TS 23.501[2] clause 5.4.14.

-	Alignment with the 5G LAN-type service as defined in TS 23.501[2] clause 5.29, e.g., supporting more than one SMF set or SMF instances in an SMF Set to enable SMF redundancy for 5G VN group reliability.

-	Constraint:

-	Only unicast UE-Satellite-UE communication is supported.

6.x.2	Description

When UEs belonging to the same 5G VN group are accessing the network via LEO/MEO satellite access supporting regenerative payload, the data traffic between these UEs can be routed more efficiently by establishing communication paths with L-PSA UPF(s) onboard satellites.

When UEs in the same 5G VN group are served by the same satellite, SMF can select and insert UL-CL/BP and an L-PSA UPF onboard satellite to enable local data switch. Figure 6.x.2-1 illustrates local data switching via an L-PSA UPF onboard satellite.

For scenarios where UEs in the same 5G VN group are served by different satellites and ISLs (Inter-Satellite Links) are available between the satellites, local data switching can be extended across satellites by supporting N19 or N6 forwarding between the L-PSA UPF(s) onboard satellites. Figure 6.x.2-2 illustrates local data switching via multiple L-PSAs onboard satellites.

When these conditions are not met, the communication falls back to being forwarded via the ground PSA UPF, which may introduce additional latency but ensures connectivity.

Figure 6.x.2-1: Local Data Switching via an L-PSA onboard satellite

Figure 6.x.2-2: Local Data Switching via multiple L-PSAs onboard satellites

When the serving LEO/MEO satellite changes, the SMF may determine whether to continue supportting UE-Satellite-UE communication.

6.x.3	Procedures

6.x.3.1 UE-SAT-UE Communication via 5G LAN by the L-PSA UPF onboard

The procedure is shown in Figure 6.x.3-1.

Figure 6.x.3-1: UE-satellite-UE communication via 5G LAN by UPF onboard

The procedure in Figure 6.x.3-1, follows the adding a PDU Session Anchor and a Branching Point or UL CL procedure defined in clause 4.3.5.4 in TS 23.502 [3] as a baseline, to clarify the enhancements for UE-satellite-UE communication via 5G LAN by UPF onboard as follows:

-	In step 1, reuse the PDU Session establishment and update procedures as described in clauses 4.3.2 and 4.3.3 of TS 23.502 [3], and the difference is:

-	If a dedicated SMF (Set) is used to serve a 5G VN, the network is configured so that the same SMF (Set) is always selected for a certain 5G VN group, and for option in case of multiple SMF Sets are serving the 5G VN, the selection and interworking of different SMF Sets is up to network implementation as specified in TS 23.501 [2] clause 5.29.

-	Based on local configuration for the 5G VN, the SMF may decide PSA UPF to be on ground as default, or on satellite as optional.

-	In step 2, 3, if SMF has decided PSA UPF to be on ground in step 1, then based on the information of satellite identifier reported by AMF, SMF decides whether to trigger UE-SAT-UE communication via UPF(s) onboard LEO/MEO satellite:

-	If the SMF is aware that there is more than one PDU session established with the same satellite identifier for the 5G VN group, the SMF may decide to insert ULCL/BP and L-PSA UPF onboard to enable local data switching.

-	If PDU sessions with different satellite identifier and inter-satellite links (ISL) are available, the SMF may establish N19 tunnel to enable local data switching via multiple L-PSA UPF(s) onboard.

6.x.3.2 Support for UE-satellite-UE communication via 5G LAN when serving satellite changes

If the UE's serving satellite changes (e.g. due to UE movement or the satellite movement), the Xn/N2-based handover procedure can be performed to change the serving gNB from that on the source satellite to that on the target satellite. The AMF includes the target satellite ID in the Nsmf_PDUSession_UpdateSMContext Request to the SMF.

If the PSA UPF is on ground, the SMF decides whether to fall back to forward via the ground PSA UPF or retain the UL CL/BP and L-PSA UPF on the satellite.

-	If the UE's serving target satellite is same with all the other members of the 5G VN group, the local data switching onboard can continue.

-	If the ISLs are available between the UE's serving target satellite and source satellite or the serving satellites of the other UEs of the 5G VN group, the local data switching onboard can continue.

-	Otherwise, the communications between the UE and the other members of the 5G VN group may fall back to the ground.

If the PSA UPF is onboard LEO/MEO satellite, the SMF may decide to change PSA based on the SSC mode of UE as specified in TS 23.502 [3] clause 4.3.5.

6.x.4	Impacts on Services, Entities and Interfaces

SMF:

-	Supports to activate local switching when the type of serving satellite is LEO or MEO.

-	Determines whether to continue supporting UE-Satellite-UE communication when the serving LEO/MEO satellite changes.

-	Alignment with the 5G LAN-type service to support more than one SMF set or SMF instances in an SMF Set for local data switching onboard LEO/MEO satellite.

******************** End of Change ********************