3GPP TSG-WG SA2 Meeting #169	S2-2505238

# Fukuoka, Japan, May 19 – 23, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6250 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configuration of TAI list for regenerative satellite payload |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, CSCN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Liaisons from RAN3 and SA5 (R3-245845, R3-250895 and S5-250842) provides information and clarifications on the OAM based solution for support of regenerative satellite payload for NR NTN: - The information of supported TAIs for NTN gNB is provided to AMF by OAM, to avoid the need for periodic and frequent RAN Configuration Update procedures. Both AMF and gNB share the same OAM configuration on supported TAI list of the on-board gNB. - NG Setup procedure including the supported TAs is always expected to take place according to TS 38.413. - Any potential discrepancy between OAM configuration and parameters received via NG signalling, if present, is expected to be resolved via implementation. - AMF is pre-configured with multiple subsequent time windows instances within which the NTN gNB ID and supported TAI list are specified. In other words, the pre-configured ‘timetable’ indicates which TAIs the gNB will support at what time and avoids the OAM configuration for each time as the satellite moves. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description that the AMF is configured with the supported TAI list of the on-board gNB, to support regenerative satellite payload for NR NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures, e.g. paging, may fail due to unawareness in the AMF of the actual TAIs served by the gNB on board the satellite. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.11.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

5.4.11.9	N2 interface and connection management for regenerative satellite payload

The NG Removal procedure defined in TS 38.413 [34] can be used to remove the interface between a gNB and an AMF in a controlled manner, e.g. when the gNB is leaving the service area of an AMF when using regenerative satellite payload.

The serving AMF for a UE can remain the same during feeder link switchover.

The AMF may be configured by OAM with information of the supported TAI list of the gNB onboard the satellite.

NOTE 1:	If a gNB IP address changes due to feeder link switchover, then the procedures described in e.g. clause 5.21.1 can be used.

NOTE 2:	RAN transitioning between different NTN GWs, TAIs and potentially between AMFs can be supported by using existing procedures, e.g. the gNB releasing UEs to CM-IDLE before changing NTN gateway, or by using existing load re-balancing, triggering handover when it realizes it is going to leave a tracking area due to its own movement, etc.

NOTE 3:	The existing NG Setup procedure including the supported TAIs can be used by the gNB onboard the satellite as described in TS 38.413 [34]. Any potential discrepancy between OAM configuration and parameters received via NG signalling, if present, is expected to be resolved via implementation.

* * * End of Changes * * * *