3GPP TSG-WG SA2 Meeting #168	S2-2505239

# Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6251 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UE-SAT-UE communication when serving satellite changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NOTE 1 requires clarification for mobility scenarios where serving satellite changes to avoid potential misinterpretation of the assumption. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified NOTE 1 to explicitly state that the assumption of co-located gNB, UL CL/L-PSA, IMS-AGW applies to both source and target satellites. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this clarification, the assumption in NOTE 1 could be misinterpreted in mobility scenarios, potentially leading to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.14.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 0: Initial version |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.4.14.3	Support for UE-satellite-UE communication when serving satellite changes

NOTE 1:	The following mechanism assumes that the source gNB, UL CL/L-PSA, and IMS-AGW are onboard the same source satellite, and the target gNB, UL CL/L-PSA, and IMS-AGW are onboard the same target satellite.

In the UE-SAT-UE communication, if the UE's serving satellite changes (e.g. due to UE movement or the satellite movement), the Xn/N2-based handover procedure can be performed to change the serving gNB from that on the source satellite to that on the target satellite. The AMF includes the target satellite ID in the Nsmf_PDUSession_UpdateSMContext Request to the SMF. The SMF decides whether to route IMS user plane traffic to the PSA on the ground or retain the UL CL/L-PSA on the satellite based on local configuration and other factors as specified in clause 6.3.3.

NOTE 2:	The SMF can decide to route the IMS user plane traffic to the PSA on ground, e.g., due to no connection between the source satellite and the target satellite.

If the SMF decides to route the IMS user plane traffic to the PSA on the ground and the procedure for removing the UL CL/L-PSA as described in clause 4.3.5.5 of TS 23.502 [3] is to be performed, the SMF provides an early notification without satellite ID via Nsmf_EventExposure_Notify to PCF. The PCF forwards this notification to the P-CSCF via Npcf_PolicyAuthorisation_Notify, indicating that the 5GC user plane needs to fall back to the ground.

If the SMF decides to retain the UL CL/L-PSA on the satellite, it provides an early notification with the target satellite ID via Nsmf_EventExposure_Notify to PCF and the PCF forwards this target satellite ID to the P-CSCF via Npcf_PolicyAuthorisation_Notify. In this case, the IMS informs 5GC whether the UE-SAT-UE communication can or cannot continue, as outlined in clause AE.5.2.1 of TS 23.228 [15], i.e. the PCF receives a response from the P-CSCF and forwards it to the SMF:

-	If the response indicates that the UE-SAT-UE communication can continue, the SMF allocates the UL CL/BP and L-PSA on the target satellite by performing a simultaneous change of UL CL/BP and additional PSA for the PDU Session, as described in clause 4.3.5.7 of TS 23.502 [3], where the UL CL/BP and L-PSA rule shall be updated with the IMS-derived transport address and optional N6 traffic routing information associated with target DNAI as received in the response. A late notification is sent by SMF to the P-CSCF via PCF afterwards, as outlined in clause 4.3.6.3 of TS 23.502 [3].

-	If the response indicates that the UE-SAT-UE communication cannot continue, the SMF removes the UL CL/BP and L-PSA on the source satellite by following the procedure described in clause 4.3.5.5 of TS 23.502 [3].

# * * * * End of change * * * *