3GPP TSG-SA2 Meeting #169	S2-2505244

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

Source:	Nokia

Title:	Discussion on prioritization of RTP retransmissions in 5GS

Document for:	Discussion

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2, FS_5G_RTP_Ph2 /Rel-19

Abstract of the contribution: This contribution discusses the available options for preferential treatment of RTP retransmissions in 5GS, triggered by the SA4 LS on RTP retransmissions S2-2504566 (S4-250738).

# Background

During the FS_5G_RTP_Ph2 study (Rel-19), SA4 discussed enabling network awareness of RTP retransmissions (as specified in RFC 4588) with the goal of improved PDU Set handling in the 5G network, as documented in clause 6.9 of TR 26.822. SA4 has identified the following 2 options:

“Option 1: Source stream and retransmission stream are mapped by the 5GC into the same QoS flow”.

In this case service differentiation needs to be ensured by other means, e.g., by mapping retransmission PDUs to a different PDU set and/or PDU Set Importance (PSI)

“Option 2: Source stream and retransmission stream are mapped by the 5GC into different QoS flows.”

IETF defined an RTP retransmission payload format in RFC 4588. Retransmission packets carry copies of lost packets along with sequence numbers and timestamps to facilitate accurate reconstruction at the receiver. The timing and frequency of retransmission packets are controlled by the sender based on network conditions and feedback from the receiver. This allows a trade-off between reliability and delay; the endpoint may give up on retransmitting after a given buffering time. RFC 4588 requires the original and retransmission packets to be sent in two separate streams. As described in SA4 discussion paper S4-250686, two options are available:

- “Session-multiplexing: the streams are multiplexed by sending them in two different sessions. In this case, the original and retransmission streams are sent to different network addresses or port numbers.”

- “SSRC-multiplexing: the streams are sent in the same session using different Synchronization Source (SSRC) values. This allows minimizing the port usage since the same port can be used for both streams. Note that SSRC multiplexing is the more typical case for retransmissions, and its support is mandated by MTSI and WebRTC.”

Based on the above, SA4 requests via S2-2504566 feedback from SA2 on the potential usage and value of RTP retransmission related information for 5GC and/or RAN and raised the following question:

“Question to SA2: Considering the options identified above, are there any benefits to providing application-layer retransmission information to the 5GC when PDU Set based handling is enabled? If so, SA4 would like to receive feedback on what type of application-layer retransmission information would be beneficial for PDU Set based handling in the 5GC.”

# Discussion

The possible occurrence of RTP retransmission is negotiated and configured end-to-end between a sender and the receiver. Currently, there is no specified mechanism to inform the 5G network on whether an application uses RTP retransmissions (e.g., a flag in the RTP HE for PDU Set marking). Thus, the network can neither identify whether retransmissions are used, nor the exact PDUs that correspond to application retransmissions. However, as retransmitted PDUs generally have tighter requirements, e.g. stricter delay requirements, 5GS provides the necessary means for preferential treatment of retransmissions based on application provided information/configuration.

Observation 1: As per Rel-19, 5GC is not aware of the PDUs corresponding to a retransmissions stream. Nevertheless, it can support service differentiation of retransmission and source streams which could be useful for the application.

Next, we analyse the mechanisms available in 5GS and the optioned indicated by SA4.

Analysis of option 1

Option 1 can be supported by assigning retransmissions to a different PDU set of higher importance (lower PSI value). Regarding 5GC, this could result to service differentiation at the transport as per the Rel-19 solution (KI#3 of XRM_ph2) that enables PSI-based transport level marking (based on SMF-provided PSIó DSCP mapping) - see TS23.501 excerpt below:

“For QoS Flows that are configured for PDU Set QoS handling, the SMF may additionally take into account the PDU Set Importance when determining the transport level packet marking values. In this case, the SMF provides a list of transport level packet marking values for the downlink direction to the UPF in a FAR, each of the transport level packet marking values corresponding to one or more PDU Set Importance values”

Notice that PDU Set marking only the source PDUs of a media flow, would leave retransmissions unmarked. If retransmissions are not allocated to a specific PDU set, PDU set importance is set according to UPF configuration, as per TS23.501.

“NOTE 4:	If the PSA UPF receives a PDU that does not belong to a PDU Set, then it is assumed that the UPF determines the PDU Set Importance value based on pre-configuration.”

Regarding the NG-RAN, setting a higher importance to retransmissions (lower PSI value) would be also beneficial as the PSI is used for PDU set handling in congestion situations:

“The 5G-AN may use the Priority Level (see clause 5.7.3.3) across QoS Flows and PDU Set Importance within a QoS Flow for PDU Set level packet discarding in presence of congestion.

NOTE 1:	In addition to considering the PDU Set Importance within a QoS Flow, 5G-AN could also consider the relative PDU Set Importance across QoS Flows of the same Priority Level when determining which PDU Set needs to be discarded, which is up to implementation and configuration of operator.”

Observation 2: Option 1 could support a different transport level marking and congestion handling at RAN based on PSI. The mechanism is not specific to retransmissions.

Analysis of option 2

Option 2 could be implemented in 2 different ways, either with the use of expedited data transfer mechanism for retransmissions or by mapping the source and retransmission streams to different QoS flows.

In Rel-19, XRM-Ph2 work introduced Expedited Data Transfer with reflective QoS to speed up the data transfer of specific IP flow(s) of XR applications. Although the feature was originally intended to the delivery of large payloads, it can be also used to associate the retransmissions with different QoS requirements. The application would mark retransmissions with Expedited Transfer Indication (ETI) set to true and the original stream with ETI set to FALSE. The main limitations of this solution are that it is only applicable to downlink and non-GBR flows, and reflective QoS needs to be supported.

Observation 3: Expedited data transfer could be used for prioritizing retransmissions for non-GBR flows in the downlink non-GBR flow.

Alternatively, a separate QoS flow could be established for retransmissions throughout the lifetime of the service. As per Rel-19, IP Packet Filter Set has been extended with SSRC and hence retransmissions can be identified.

Observation 4: Retransmissions can be mapped to different QoS flow (uplink and downlink) based on: i) IP 5-Tuple for the case of session-multiplexing or ii) the Synchronization Source (SSRC), if SSRC-multiplexing is used.

# 3	Conclusion

SA2 evaluated the QoS flow mapping options for retransmissions and their interaction with the marking options provided by SA4 and concluded that the application has sufficient means to differentiate the handling of retransmissions, namely by

- providing a different PSI value for retransmissions, resulting to differentiated handling at transport and RAN, even if the same QoS flow is used; or

- using the mechanism of expedited data transfer to map retransmissions to a better QoS (temporarily); or

- creating different QoS flows for the retransmission and the source stream based on SSRC (IP Packet Filter Set extended in Rel-19)

SA2 also confirms the feasibility of indicating that a stream corresponds to retransmissions based on application provided information, if RAN groups confirm the usefulness of such information.