3GPP TSG-SA WG2 Meeting #169	S2-2505273

Fukuoka, Japan, 19 – 23 May 2025

Source:	ETRI

Title:	New solution for KI#4: Sensing data collection, transport, and processing

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This contribution proposes a solution for KI#4 by defining the mechanisms for sensing data and associated information collection, transport, and processing for sensing result generation, leveraging the SeMF/SePF architecture.

# 1. Discussion

This contribution addresses the work tasks identified in KI#4 of TR 23.700-14 by proposing a new functionality for sensing data and associated information collection and transport, and identifying the entity responsible for sensing result generation. The solution defines how sensing data is collected from sensing entities (gNBs/UEs) by the SeMF, how this data is transported over CP or UP, the role of associated information (assistance and contextual), and where the collected data is processed into sensing results.

- New NFs (Architectural assumption as proposed in S2-2505271)

  - SeMF: Sensing Management Function

  - SePF: Sensing Provisioning Function

- Key functionalities

  - Sensing data management (by SeMF)

    - collection

      - SeMF requests/collects sensing data to/from sensing entities

      - data transport:

        - CP: N1 or N2

        - UP: N3 (as like LPP over UP)

    - processing/reply

      - SeMF generates sensing result from sensing data collected, and reply the result to the requestor (SePF or other NFs)

        - content: object type, location, velocity, direction, etc.

      - with contextual information

    - storage

      - SeMF manages sensing data collected in its own stoage

      - for re-use with different TTLs

      - for analytics by NWDAF (FFS)

        - for accuracy, prediction, resource efficiency, etc.

# 2. Proposal

It is proposed to add the following text to TR 23.700-14.

* * * Start of Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #4> | <Key Issue #y> |
| #1 |  |  |
| #2 |  |  |
| #x | O |  |

* * * Next Change * * * *

6.X	Solution #X: Sensing data collection, transport, and processing

6.X.1	Description

This solution addresses Key Issue #4 by defining the procedures and mechanisms for collecting sensing data and associated information from sensing entities and transporting it to a designated network function for sensing result generation.

Assuming the SeMF/SePF architecture (as per KI#1) and the discovery/selection procedures (as per KI#3), SeMF is responsible for requesting and collecting sensing data from the sensing entities it has selected for a sensing request. Sensing data, which is measured by sensing entities, may be transported to 5GC, utilizing CP or UP or both depending on the data volume and latency requirements. Based on the collected sensing data, SeMF generates sensing result to be reported back to sensing service requestor (e.g, AF via SePF/NEF; or other CN NFs).

SeMF may also manage the storage of collected sensing data and facilitates the provision of sensing results for further use and analytics but is FFS. The contextual information may be handled alongside the sensing data but the content is also FFS.

6.X.2	Procedures

Figure 6.X.2-1: Procedure for sensing data collection and transport

The procedures for sensing data collection and transport are performed as the following steps:

- (Sensing request) SePF or other CN NFs request SeMF to perform sensing data collection and sensing result generation. The request may include sensing type (e.g., object detection, tracking, environment monitoring, etc.), target (e.g., object, area), QoS (e.g., accuracy, resolution, latency), assistance information (object information, area information), etc.

- (Sensing data measurement request) The SeMF requests the selected sensing entities to perform the sensing data measurement. This is done by sending a request or enabling reporting via CP (e.g., N1/NAS or N2/NGAP via AMF).

Editor’s Note: The content of sensing request to sensing entities needs coordination with RAN.

- (Sensing data collection/transport) The sensing entities measure sensing data using their radio capabilities based on their configuration; and send the measured sensing data and associated information towards SeMF. Sensing data transport utilizes either CP or UP paths, depending on the data characteristics (e.g., volume, latency, etc.)

Editor’s Note: The content of measured sensing data needs alignment with RAN work.

- (Sensing data processing) SeMF further processes the collected sensing data and associated information to generate sensing result (e.g., object type, location, velocity, direction, etc.) and contextual information (e.g., timestamp, location, confidence level). SeMF may also manage the storage of collected sensing data or sensing results for potential re-use or analytics purposes (FFS).

- (Sensing result provision) Once the sensing result is generated, it is provided back to the sensing service requestor (e.g., SePF or other CN NFs) with the associated contextual information.

6.X.3	Impacts on Services, Entities and Interfaces

The following NFs and interfaces are impacted:

SeMF:

-	a new network function for sensing data management including sensing data collection, processing, and storage

-	new services and interfaces for sensing sensing data collection, and sensing request

gNB/UE:

-	enhancement for handling request for sensing data measurement from SeMF (via N1/N2)

Editor’s Note: The scope of RAN WGs.

* * * End of Change * * * *