3GPP TSG-SA WG2 Meeting #169	S2-2505283

Fukuoka, Japan, May 19 – 23, 2025

Source:	CATT

Title:	KI#2 solution update for policy control based on notifications from the EIF

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes updates to Solution #6 "Policy control based on notifications from the EIF".

1. Introduction

Updates to Solution #6 "Policy control based on notifications from the EIF" are proposed, to resolve the ENs and update the procedures.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

* * * Start of Change * * * *

6.6	Solution #6: Policy control based on notifications from the EIF

6.6.0	High-level solution principles

This solution proposes that the PCF takes the energy related information in the notifications from the EIF into account for policy control.

NOTE:	It is required that the energy related information in the notifications from the EIF is accurate enough for the PCF to take into account for policy decision.

6.6.1	Description

This solution addresses Key Issue #2.

The PCF can subscribe to notifications from the EIF, e.g. Energy Consumption information at UE, S-NSSAI, PDU Session and Service Data Flow (e.g. per UE per application) granularity as described in clause 5.51 of TS 23.501 [2]. The PCF may decide to subscribe to notifications from the EIF based on operator policy (e.g. to perform policy control considering energy related information for specific area or time period), AF request (e.g. Energy indicator as specified in clause 6.1.2.4 of TS 23.503 [4]), or notifications of UE subscription data change (e.g. the value of Energy Saving indicator changes) from the UDR.

The PCF may determine and update the following policies based on energy related subscription data (i.e. Energy Saving Indicator), notifications from the EIF and operator policy:

-	Access and mobility related policy, e.g. UE-AMBR, RFSP Index.

-	Session management related policy, e.g. Session-AMBR, MA PDU Session Control information.

Examples for AM policy control taking into account the notifications from the EIF are as follows:

-	If notification(s) from the EIF indicates energy consumption for the UE is high based on operator policy (e.g. higher than a predefined UE average energy consumption threshold per PLMN or per network slice), the PCF may modify the UE-AMBR value to be lower than the subscribed UE-AMBR or the UE-AMBR in use for the UE based on operator policy, to reduce the energy consumption for transmitting the Non-GBR QoS Flows of the UE.

-	If notification(s) from the EIF indicates energy consumption for the UE over 5G access is high based on operator policy (e.g. higher than a predefined UE average energy consumption threshold per PLMN or per network slice), the PCF may modify the RFSP Index value to indicate that EPC/E-UTRAN access is prioritized over the 5G access for the UE with validity time based on operator policy as specified in clause 6.1.2.1.1 of TS 23.503 [4].

Editor's note:	It is FFS whether the PCF takes notifications from the EIF into account to decide the RFSP Index value in AM policy.

Examples for SM policy control taking into account the notifications from the EIF are as follows:

-	If notification(s) from the EIF indicates energy consumption for the UE or for a PDU Session of the UE is high based on operator policy (e.g. higher than a predefined UE average or PDU Session average energy consumption threshold per PLMN or per network slice), the PCF may modify the Session-AMBR value to be lower than the subscribed Session-AMBR or the Session-AMBR in use for the PDU Session based on operator policy, to reduce the energy consumption for transmitting the Non-GBR QoS Flows of the PDU Session.

-	If notification(s) from the EIF indicates energy consumption for the UE over 3GPP access is high based on operator policy (e.g. higher than a predefined UE average energy consumption threshold per PLMN or per network slice), the PCF may modify the MA PDU Session Control information (e.g. Steering Mode and associated threshold values) in the PCC rules to switch some or all the Non-GBR QoS Flows of the MA PDU Session from 3GPP access to non-3GPP access as described in clause 6.1.3.20 of TS 23.503 [4].

Editor's note:	It is FFS whether the PCF takes notifications from the EIF into account to decide the MA PDU Session Control information in the PCC rules.

In addition, based on notifications from the EIF, the PCF may provision and update validity conditions (e.g. validity time period, validity area) in the policies, for network energy saving within a specific time period and/or specific area.

6.6.2	Procedures

The existing policy control procedures as specified in TS 23.502 [3] and TS 23.503 [4] apply, with the enhancements that based on possible triggers (e.g. operator policy, AF request or UE subscription data change) the PCF subscribes to and receives notifications from the EIF before making policy decisions.

Figure 6.6.2-1: Policy control based on notifications from the EIF

1.	The PCF decides to subscribe to notifications from the EIF based on operator policy, AF request (e.g. Energy indicator as specified in clause 6.1.2.4 of TS 23.503 [4]), or notifications from the UDR on UE subscription data change (e.g. the value of Energy Saving indicator changes).

2.	The PCF subscribes to and receives notifications from the EIF as specified in clause 4.29.2 of TS 23.502 [3].

3.	The PCF makes policy decisions taking into account the notifications from the EIF.

4.	The PCF performs policy control as specified in TS 23.502 [3] and TS 23.503 [4].

6.6.3	Impacts on services, entities and interfaces

PCF:

-	Subscribes to notifications from the EIF based on possible triggers (e.g. operator policy, AF request or UE subscription data change).

-	Performs policy control taking into account of notifications from the EIF.

EIF:

-	Sends energy related notifications to the PCF.

* * * End of Change * * * *