3GPP TSG-SA WG2 Meeting #169	S2-2505287

Japan, Fukuoka, May 19 – 23, 2025

Source:	MediaTek Inc.

Title:	Solution for KI#2: Support of policy adjustments

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes solution to the key issue for KI#2 on Support of policy adjustments.

* * * Start of Change * * * *

6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |
| 1 | X |  |  |
| 2 | X | X |  |
| 3 |  | X |  |
| 4 |  | X |  |
| 5 |  | X |  |
| 6 |  | X |  |
| 7 |  | X |  |
| X |  | X |  |

6.X	Key Issue #X: Support of policy adjustments for URSP

6.X.0	High-level solution principles

The solution proposes that PCF can obtain the energy related information ((e.g., energy consumption information for a UE and the energy attributes/status (e.g., renewable energy information for a network slice) from EIF and OAM and then based on the received information to make the proper PCC rules to SMF

6.X.1	Description

As study conclusion in TR 23.700-66 [yy], the PCF may receive UE subscription data and notification related to the energy related information to trigger making policy decisions (reusing the existing parameters) based on operator policy.

Based on resuing the existing parameters, in this solution, it assumes there is no new field that will be added to the URSP rules. The PCF generates the URSP rule based on the energy related information (e.g., energy consumption information for a UE and the energy attributes/status (e.g., renewable energy information for a network slice)) from the EIF and network slicing information from OAM.

6.X.2	Architectural assumption

The architecture can be referred to clause 4.2.18 of TS 23.501 [2] to enhance the service operations from EIF to other 5G NFs, e.g., PCF.

The existing URSP rules can be reused as specified in TS 23.503 [4].

6.X.3	Policy adjustment for URSP rule generation

The PCF can interact with EIF to obtain the energy related information and the energy attributes/status information (e.g., renewable energy information) and to obtain network slicing information to generate URSP rules and network slice information, e.g.,

-	A Route Selection Descriptor (RSD) in a URSP rule, as specified in Table 6.6.2.1-3 in TS 23.503 [4], may have optional fields such as time window and location criteria. If these two fields are presented in the RSD of a matched URSP rule, they need to be met before the UE associates the application to a PDU Session. If utilizing these two fields by an operator, the operator (e.g. PCF) based on the energy related information, can generate URSP rules where the time window and location criteria are configured based on e.g., renewable energy usage information/energy consumption for a UE/energy attributes/status to influence the UE to request the greener network slices/DNN.

6.X.4	Procedure

1a. PCF subscribes the service from EIF and gets the energy related information (e.g., energy consumption information for a UE and the energy attributes/status (e.g., renewable energy information for a network slice) from EIF

1b. PCF interacts with OAM to obtains the network slicing information.

2. After making decision based on the information from 1a and 1b, the PCF updates the policy (e.g., URSP rules) to SMF based on clause 4.16.5 of TS 23.502 [3].

6.X.5	Impacts on existing services, entities and interfaces

OAM:

-	Needs to provide the energy attributes of RANs to EIF

PCF:

-	Needs to interact with EIF to obtain the energy related information

-	 Needs to interact with OAM to obain the network slicing information

EIF:

-	needs to interact with PCF and provides the energy related information (e.g., energy consumption for the UE in different granularities)

-	if subscribed by PCF, the EIF needs to periodically or event triggered based to notify the energy related information to the PCF

* * * End of Change * * * *