3GPP TSG-SA WG2 Meeting #169	S2-2505294

Japan, Fukuoka, May 19 – 23, 2025

Source:	MediaTek Inc.

Title:	Solution for KI#3: Support of UP path adjustments for the UE

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes solution to the key issue for KI#3 on Support of UP path adjustments for the UE.

* * * Start of Change * * * *

6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |
| 1 | X |  |  |
| 2 | X | X |  |
| 3 |  | X |  |
| 4 |  | X |  |
| 5 |  | X |  |
| 6 |  | X |  |
| 7 |  | X |  |
| X |  |  | X |

6.X	Key Issue #X: Support of UP path adjustments

6.X.0	High-level solution principles

The solution is to address how UPF is selected or reselected by SMF

-	the energy value in NR profile in NRF

-	after PCF updates the PCC rules based on the energy related information from EIF

The energy value in NR profile can be calculated according to operator policy by considering one or more of the energy related information.

6.X.1	Description

This solution is to address the issue for the UP path adjustments for the UE based on the energy attributes and energy related information from EIF.

The UP path adjustment is to (re)select UPF by SMF based on the energy related information from EIF.

The trigger events for SMF to (re)select a UPF for a PDU Session can be:

-	from PCF due to PCC rules change/update after PCF interacts with EIF

-	from  NF profile with a new value ("energy related value in NR profile") calculated according to operator policy by considering one or more of energy related information

6.X.2	Architectural assumption

The architecture can be referred to clause 4.2.18 of TS 23.501 [2] to enhance the service operations from EIF to other 5G NFs.

6.X.3	UP path adjustment

The UPF can be (re)selected by SMF as specified in clause 6.3.3 of TS 23.501 [2] due to e.g., UE mobility, UE traffic offloading. The UPF (re)selection can be triggered also due to the update of the energy related information. The SMF can interact with EIF to obtain the energy related information, and the SMF can (re)select a UPF based on the obtained energy related information.

On the other hand, the SMF can also utilize the NRF to (re)select a UPF based on the value calculated considering the energy related information .

The UPF is (re)select based on the calculated value in NF profile for the UPF candidates provided by NRF when the SMF queries with NRF. When a SMF is triggered to (re)select a UPF, the SMF queries NRF to get the list of candidates of UPFs, each of those candidates UPFs has one energy related value in NF profile. The SMF basically can select a value which has higher or lower value than the rest candidates of those UPFs in the list.

The meaning of higher or lower value for a UPF to be selected can be decided by operator policies, e.g., the UPF from the candidates which has highest (lowest) value can be selected by SMF from the list of candidates of UPFs.

The values can be calculated with the weight ratio defined by operators considering one or more of the energy related information listed below, e.g., :

-	energy consumption information;

-	ratio of renewable information per slice

-	energy status of NG-RAN

- energy efficiency

When the SMF decided to (re)select a UPF for UE traffic routing, the SMF can trigger PDU Session establishment modification which includes the N4 Session modification procedure in clause 4.4.1.3 of TS 23.502 [3] to reselect a UPF and update the corresponding N4 rules to the re-selected UPF.

Besides the above, it is possible that after performing the SM policy association modification procedure with PCF, the SMF may also trigger UP path adjustment to reselect a UPF

6.X.4	Procedures for UP path adjustment

The existing procedures as specified in TS 23.502 [3] such as PDU Session modification procedure can be re-used by SMF to reselect a UPF for UE traffic routing.

6.X.5	Impacts on existing services, entities and interfaces

SMF:

-	may reselect a UPF for a PDU Session for UE traffic routing based on

-	after interacting with PCF

-	after interacting with NRF

PCF:

-	generate PCC rules to SMF that may trigger to reselect a UPF after interacting with EIF

EIF:

-	interact with PMF such that SMF may trigger to reselect UPF for UE traffic routing due to policy change

* * * End of Change * * * *