3GPP TSG-SA WG2#169	S2-2505298

Fukuoka, Japan, 19-23 May 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	Ensuring an inter-operable Ambient IoT system

Document for:	Discussion/Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: This paper addresses the open question for AIoT discussed at SA2#168 (whether further parameters need to be standardized for Read and Write commands in AIoT NAS), analyses alternatives and proposes a way forward.

# 1.	Background

Over the course of the last two SA2 meetings consensus could not be reached how to support commands for AIoT devices. A command can be one of the following operations:

-	reading data from an AIoT device;

-	writing data to an AIoT device;

-	permanently disabling an AIoT device.

SA2 already agreed that an application function (AF) indicates the command to be executed (read, write or disable) to the AIOTF and that the AIOTF then indicates the command to be executed to the AIoT Device using the AIoT NAS protocol.

The open question is particularly for reading and writing data from/to the AIoT Device whether further parameters need to be specified for AIoT NAS to indicate to the AIoT Device which data to read (e.g., identified by memory location and the amount of data to read) and which data to store and where (identified by memory location and the actual data).

This question is important as it determines the level of interoperability of 3GPP’s Ambient IoT system.

Observation 1: The key open question for AIoT discussed at SA2#168 (whether further parameters need to be standardized for Read and Write commands in AIoT NAS) is essential as it determines whether 3GPP’s Ambient IoT system will be fully interoperable or not.

The following alternatives were discussed at SA2#168:

-	Alternative 1: Specify necessary parameters for read and write operations in AIoT NAS to ensure an inter-operable AIoT system:

-	Read command:

-	Address to identify the location at which to read data

-	Length of data to be read

-	Write command:

-	Address where to store data

-	Data to be stored

-	Alternative 2: Leave the details of read and write commands to implementation

-	Exchange transparent command containers between AF and AIoT Device

The remainder of this paper discusses these two proposals and proposes a way forward.

# 2.	Discussion

Alternative 2 practically means that the protocol for read and write commands is proprietary, i.e. that the AIoT Device to AF interface is closed. In other words, reading and writing data from/to an AIoT Device can only be supported by an AF that supports the proprietary container format for a particular AIoT Device model.

For AIoT network operators this means the following:

-	If an additional AIoT Device model is to be supported in the network, the AF implementation needs to be updated to support the proprietary container format of the additional AIoT Device model.

-	AIoT network deployments that support multiple AIoT Device models need to ensure that the correct proprietary container format is used depending on the AIoT Device from which data is supposed to be read or to which data is to be written, e.g. by ensuring the correct AF implementation is used depending on the targeted AIoT devices.

-	This requires additional development, integration and testing efforts.

Observation 2: Leaving the details of read and write commands to implementation (Alternative 2) means that for each additional AIoT Device model to be supported in a network, the AIoT network operator needs to develop/integrate and test an additional proprietary AIoT Device to AF protocol (e.g. by deploying AIoT Device model specific AFs for each supported AIoT Device model).

Another drawback of Alternative 2 is that it suggests that AIoT networks have visibility and control over the commands that an AF sends to specific AIoT Devices.

However, just naming an AIoT NAS operation Read and Write without specifying further details does not mean anything if both operations are just used to transfer transparent containers between AIoT Device and AF. It is important to highlight that an implementation could use the Read operation to send a transparent container to the AIoT device that can contain any type of command.

For instance, an implementation could use the Read operation to write data to the AIoT Device (by including a proprietary write command in the transparent container of the Read operation). Another implementation could send a proprietary Disable command using the Read operation (by including a proprietary write command in the transparent container of  the Read operation).

Observation 3: Leaving the details of read and write commands to implementation (Alternative 2) means that AIoT networks have no visibility and control over the commands that an AF sends to specific AIoT Devices. An implementation could trigger for instance a Write or Disable command using the Read operation of the AIoT NAS protocol (by including a proprietary write or disable command in the transparent container of the AIoT NAS Read operation).

As illustrated above, leaving the details of the Read and Write AIoT NAS operations to implementation means that transparent containers would be exchanged between AF and AIoT Device by the Read and Write operation. However, what is unclear is why the AIoT NAS should support two NAS operations to send transparent containers between AF and AIoT Device.

Some companies have argued that it may help NG-RAN for radio resource planning to know whether an AIoT NAS operation is a Read or a Write operation. However, as per Observation 3, an implementation could send any type of proprietary command using the Read or Write operations, i.e. there is no guarantee that the AIoT NAS Read operation will trigger reading of data or something else. Also, it has been agreed that the AIOTF provides NG-RAN with assistance information including the expected size of the D2R message, which is sufficient for radio resource planning.

In other words, specifying Read and Write AIoT NAS operations but leaving the details to implementation essentially means to specify two procedures with the exact same purpose, to exchange transparent containers between AIoT Device and AF, without any additional benefits.

This is not only inefficient from protocol perspective but also unnecessarily increases development, testing and integration efforts.

Observation 4: Specifying Read and Write AIoT NAS operations but leaving the details to implementation (Alternative 2) essentially means specifying two procedures for the exact same purpose, to exchange transparent containers between AIoT Device and AF, without any additional benefits. This is not only inefficient from protocol perspective but also unnecessarily increases development, testing and integration efforts.

Alternative 1 proposes to specify the necessary parameters for read and write operations in AIoT NAS to ensure an inter-operable AIoT system. This allows AIoT networks to support read and write operations for different AIoT Devices without having to support proprietary read and write protocols for different AIoT device models.

Alternative 1 also ensures that AIoT networks have visibility and control over the commands that are sent to AIoT Devices.

Observation 5: The benefit of Alternative 1 is that it allows AIoT networks to support read and write operations for different AIoT Devices without having to support proprietary read and write protocols for different AIoT device models. Alternative 1 also ensures that AIoT networks have visibility and control over the commands that are sent to AIoT Devices.

# 3.	Proposal

This paper made the following observations:

-	Observation 1: The key open question for AIoT discussed at SA2#168 (whether further parameters need to be standardized for Read and Write commands in AIoT NAS) is essential as it determines whether 3GPP’s Ambient IoT system will be fully interoperable or not.

-	Observation 2: Leaving the details of read and write commands to implementation (Alternative 2) means that for each additional AIoT Device model to be supported in a network, the AIoT network operator needs to develop/integrate and test an additional proprietary AIoT Device to AF protocol (e.g. by deploying AIoT Device model specific AFs for each supported AIoT Device model).

-	Observation 3: Leaving the details of read and write commands to implementation (Alternative 2) means that AIoT networks have no visibility and control over the commands that an AF sends to specific AIoT Devices. An implementation could trigger for instance a Write or Disable command using the Read operation of the AIoT NAS protocol (by including a proprietary write or disable command in the transparent container of the AIoT NAS Read operation).

-	Observation 4: Specifying Read and Write AIoT NAS operations but leaving the details to implementation (Alternative 2) essentially means specifying two procedures for the exact same purpose, to exchange transparent containers between AIoT Device and AF, without any additional benefits. This is not only inefficient from protocol perspective but also unnecessarily increases development, testing and integration efforts.

-	Observation 5: The benefit of Alternative 1 is that it allows AIoT networks to support read and write operations for different AIoT Devices without having to support proprietary read and write protocols for different AIoT device models. Alternative 1 also ensures that AIoT networks have visibility and control over the commands that are sent to AIoT Devices.

Based on these observations, this paper proposes to select Alternative 1 as the way forward, i.e., to specify the necessary parameters for read and write operations in AIoT NAS to ensure an inter-operable AIoT system. Related changes are proposed in S2-2505344 [1]

Proposal 1: Select Alternative 1 as the way forward, i.e., specify necessary parameters for read and write operations in AIoT NAS to ensure an inter-operable AIoT system.

Should Alternative 1 not be agreeable, then Alternative 2 becomes an option. However, for Alternative 2 to become acceptable, the following is essential:

As pointed out in Observation 4, specifying Read and Write AIoT NAS operations but leaving the details to implementation means that 3GPP would specify two procedures for the exact same purpose: to exchange transparent containers between AIoT Device and AF. As illustrated above, this is inefficient and increases development, testing and integration efforts.

Therefore, this paper proposes to specify only one generic procedure to send AIoT Device specific commands (and the related command responses) between AF and AIoT Device. Related changes are proposed in S2-2505300 [2].

Proposal 2: Should Alternative 1 not be agreeable, then Alternative 2 becomes an option. However, for Alternative 2 to become acceptable, this paper proposes to specify only one generic procedure to exchange AIoT Device specific commands (and the related command responses) between AF and AIoT Device.

# 4	References

[1]	Huawei, Qualcomm: Support of AIoT Device Memory and Associated Commands. S2-2505344.

[2]	Qualcomm Incorporated: Support implementation-specific Read and Write operation through generic AIoT Device specific command. S2-2505300.