3GPP SA WG2 Meeting #169	S2-2505315

Fukuoka, Japan, May 19 - 23, 2025

Source:	Ericsson

Title:	Discussion Paper on Paging ID Length

Document for:	Agreement

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The contribution discusses the paging ID length

1. Introduction

SA2 received an LS from RAN2 (R2-2503197) to inform SA2 about RAN2’s agreements and ask SA2 to provide feedback on the Paging ID length:

From SA2 point of view, the Paging ID to be used in A-IoT paging is the AIoT Identification Information specified in step 7 of clause 6.2.2, and also described in step 4:

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. The AIOTF may perform authorization as specified in clause 5.6. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a Correlation ID corresponding to this AF service operation request.

The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or a single AIoT Device Identifier.

……

7.	The AIOTF sends the Inventory Request message including the Correlation ID, the AIoT Identification Information to be included in the paging message, and assistance information to the selected NG-RAN.

[Observation-1] The AIoT Identification Information is included in the Inventory Request sent to the NG-RAN node.

[Observation-2] The AIoT Identification Information can be a Filtering Information or single AIoT Device Identifier.

According to [Observation-1], in RAN3 running CR (R3-252480), the AIoT Identification Information is the A-IoT Device Identification Requested IE in Inventory Request Transfer IE:

#### 9.3.x.1	Inventory Request Transfer

This IE provides the inventory request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | Reject |
| A-IoT Device Identification Requested | M |  | 9.3.3.xx5 |  | YES | Reject |
| Requested Service Area Information | M |  | 9.3.1.xx3 |  | YES | Reject |
| Inventory Assistance Information | O |  | 9.3.1.xx4 |  | YES | Reject |
| Follow on Command Indication | O |  | ENUMERATED (true, ...) | This IE indicates that there is command(s) to be transmitted. | YES | Reject |

……

#### 9.3.3.xx5	A-IoT Device Identification Requested

This IE represents the Requested Identity of the A-IoT device(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| FFS |  |  |  |  |

Editor’s Note: this IE enables identification of a single A-IoT device, a group of A-IoT devices, all A-IoT devices, detailed coding FFS.

Even though it is still FFS, based on the received A-IoT Device Identification Requested IE, the NG-RAN node understand the Paging ID, as well as the length of the Paging ID, and then inform the readers. The reader can then fill in the Paging ID length field.

[Proposal-1] The NG-RAN node can inform the readers the length of Paging ID together with the Paging ID itself.

In the LS, RAN2 also wants to understand the possible length of the Paging ID, so that the field of the Paging ID length can be properly designed. Based on [Observation-2], the Filter information and the signal AIoT Device Identifier options can be discussed separately.

For the Filtering Information

The Filtering Information is specified in clause 5.8 of TS 23.369:

The filtering information is used to identify or filter multiple AIoT Devices and is constructed by one or multiple components of the AIoT Device Permanent Identifier as defined in clause 5.7.

The filtering information includes a list of filtering element information. The filtering element information includes:

-	Information indicating a which component of the AIoT Device Permanent Identifier that is used to match the bitstring.

-	A bitstring which is used to compare with the component.

Each bitstring is corresponding to one component of the AIoT Device Permanent Identifier.

The AIoT Device Permanent Identifier is specified in clause 5.7.2 of TS 23.369:

The ecoding of AIoT Device Permanent Identifier is going to be specified by CT4. And in our view, we can make a rough estimation and ask CT4 to comment.

- ID Type: can be 4 bits to indicate the presence of 3 components in Domain Information, and the type of Identification Information.

- PLMN ID: 5-6 digits according to TS 23.003

- NID: 11 digits according to TS 23.003

- Third Party Identifier: assume it is GS1 Company Prefix, the length is from 6 to 12 digits.

- Identification Information: According to TS 23.369, 96 bits and 128 bits are supported.

Then, for an AIoT Device Permanent Identifier, the longest one should contain all components, even thought it is not the typical case. Then maximum length can be estimated to be 4 + 6*4 + 11*4 + 12*4 + (128 + 8) = 256 bits, taking the assumption that Binary-Coded Decimal is used for the encoding of the digits, and a 8 bit field is used to identify the length Identification Information (or total length).

About the Filtering Information, the indicator of the components can be estimated to be 2 bits, as it needs to refer to 4 components in total. The bitstrings are also assumed to be the full length of the component maximum.

For the components in the Domain Information, it is assumed to be a full ID match, so that the length of the bitstring is not provided:

- One filtering information element for PLMN ID: 2 + 6*4 = 26 bits

- One filtering information element for NID: 2 + 11*4 = 46 bits

- One filtering information element for Third Party Indentifier: 2 + 12*4 = 50 bits

For the Identification Information, the bitstring usualy is a sub-string of the Identification Information. And thus, a length field may be needed, which can be 8 bits to cover the maximum length of 128 bits.

- One filtering information element for Identification Information 2 + 128 + 8 = 138 bits

In addition, there may be an additional field to indicate the number of filtering information elements, which requires 2 bits.

In total, the maximum length could be roughly estimated to be 2 + 26 + 46 + 50 + 138 = 262 bits

In our view, the 262 bits should be the very extreme case, which is unlikely to happen. Typically, there are the following factors which may reduce the total length:

- Not all components in Domain Information will be used in the deployment in most cases, due to AIoT Device Permanent Identifier usually does not inlucde all components in Domain Information.

- To address a group of devices, the substring of the Identification Information should not be full length. Otherwise, it will not only target to one individual device, but also cause provicay concern.

[Proposal-2] For a group of AIoT Devices, Filtering Information is used a paging ID. The maximum length of Filtering Information should be less than 262 bits, based on our calculation with some assumptions.

[Proposal-3] The stage 3 design of Filtering Information is under the remit of CT4. CT4 may propose a more efficient design than the one above. The feedbacks or a better estimation can be provided by CT4.

For the single AIoT Device Identifier

There are privacy issues if the AIoT Device Permanent Identifier is delivered in A-IoT paging message as plain texts. The single AIoT Device Identifier should be the temporary ID as described in clause 7.3 TR 33.713:

## 7.3	Conclusion on KI #3: Privacy by protecting AIoT device identifiers

The following aspects and principles are agreed for the conclusion on KI#3

-	A mechanism to protect AIoT device ID based on the use of temporary ID shall be supported.

-	Mechanism shall allow unambiguous identification of the AIoT device.

-	A mechanism to re-synchronize de-synchronized temporary IDs shall be supported.

-	The network and the device shall support a mechanism for the use of temporary ID.

Editor’s Note: Solution are FFS.

[Observation-3] Temporary ID is used for single AIoT Device Identifier

In TS 33.369, the relevant clause is not ready:

## 5.4	Protection of AIoT device identifier privacy

Editor’s Note: This clause contains the security procedures for AIoT device identifier privacy.

[Observation-4] The details of Temporary ID and the relevant solution are under discussion in SA3.

At this stage, without the understanding of what is the Temporary ID and how it works, SA2 cannot determine the possible length of the temporary ID.

[Proposal-4] For a single AIoT Device, temporary ID is used in paging ID. SA2 cannot provide the feedback about the possible length of temporary ID without the input from SA3. SA2 can ask SA3 to provide the feedback.

[Proposal-5] It is proposed to agree on the reply LS (S2-2505314) based on the discussion above.

2. Proposal

[Proposal-1] The NG-RAN node can inform the readers the length of Paging ID together with the Paging ID itself.

[Proposal-2] For a group of AIoT Devices, Filtering Information is used a paging ID. The maximum length of Filtering Information should be less than 264 bits, based on our calculation with some assumptions.

[Proposal-3] As Filtering Information design is under the remit of CT4, the comments from CT4 are needed.

[Proposal-4] For a single AIoT Device, temporary ID is used in paging ID. SA2 cannot provide the feedback about the possible length of temporary ID without the input from SA3. SA2 can ask SA3 to provide the feedback.

[Proposal-5] It is proposed to agree on the reply LS (S2-2505314) based on the discussion above.