SA WG2 Meeting #169	S2-2505322

Fukuoka, Japan, 19-23 May, 2025

# Title:	Paging re-attempt configuration

# Source:	InterDigital Inc.

# Document for:	Approval

Agenda Item:	19.14.2

Abstract:	The pCR proposes to include “Paging re-attempt configuration” in the assistance information to enable the NG-RAN to re-attempt inventory/paging.

1. Discussion

AIoT paging has higher ratio of failure due to the reasons such as the target AIoT device (s) may not be sufficiently powered, or the device fails the contention-based Random Access, etc. One obvious solution to address this issue is that the NG-RAN re-attempt AIoT paging a few times until the response is successfully received. However, the re-attempt of AIoT paging may not be suitable in some scenarios. For example, when the AIoTF initiates Inventory Request that’s followed by a Command, the extra delay caused by paging re-attempt may not be desired.

To guide the NG-RAN to perform or not perform paging re-attempts, it is proposed that the AIoTF may provide “Paging re-attempt configuraiton” as part of the Assistance Information to the NG-RAN. The configuration may include the following information:

- Whether the paging re-attempt is allowed or not;

- Maximum number of paging attempts;

- Time interval between paging attempts.

2. Proposal

It is proposed to adopt the proposed change in TS 23.369.

* * * * First Change (All new text) * * * *

## 5.4	Assistance information provided to NG-RAN node

The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests:

a)	AIoT service type (e.g. Inventory, Command).

b)	Optionally, approximate number of AIoT devices based on AF request.

c)	Optionally, approximate D2R message size based on AF request.

d)	Optionally, time interval as AIoT aggregation assistance information.

e)	Optionally, paging re-attempt configuration that includes an indication whether paging re-attempt is allowed or not, and optionally maximum number of paging re-attempts and time interval between paging re-attempts.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	It is FFS where detailed description on time interval as AIoT aggregation assistance information will be captured, i.e. in this clause or in another clause.

If not provided by the AF, bullets b) and c) in the above assistance information provided by the AIOTF may be based on local configuration based on SLA between the AIoT service provider represented by an AF and the operator.

If the AF has provided a time interval, then the AIOTF should set bullet d) to NG-RAN that is equal or shorter than the time interval received from the AF.

NOTE:	Based on local configuration, the AIOTF can reject the AF request, e.g. if the AF provided time interval is shorter than a locally configured minimum interval.

The assistance information is used by the NG-RAN for performing service operations, e.g. radio resource allocation by using bullets b) and c), AIoT Device responses aggregation by using bullet d), AIoT paging re-attempt by using bullet e).

* * * * End of Change * * * *