3GPP TSG-SA2 Meeting #169	S2-2505328

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0741 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIML_CN KI#1 Add DCCF as Client for Nlmf_DataExposure service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. CT3 asked in their incoming LS S2-2504505 why the DCCF is not listed as client for Nlmf_DataExposure service, although the service serves to collect input data for the NWDAF. 2. According to Clause 5.18, for model training and performance monitoring related to LMF based models for AI/ML positioning, the LMF checks with UDM the user consent status before collecting UE related data. This is not reflected in the overview clause 4.3.6 for the UDM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. As suggested by CT3 in incoming LS S2-2504505, add DCCF as client for Nlmf_DataExposure service. 2. Add information how user consent in UDM is used to overview clause 4.3.6 to allign with clause 5.18. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. NWDAF data collection to train models for LMF based positioning deviates from thze principles for NWDAF data collection outlined in TS 23.288. 2. Inconsistent specification related to user consent checking at UDM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6, 8.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

### 4.3.6	UDM

The UDM contains LCS subscriber LCS privacy profile and routing information. The UDM is accessible from an AMF, GMLC or NEF via the Nudm interface.

The UDM may also contain an indication whether a UE is allowed to serve as a PRU and indication whether PRU is stationary PRU as part of the UE subscription data.

The UDM may also contain LMF identifier(s) and indication of user plane positioning between UE and LMF in UE LCS subscription data.

The UDM may also contain user consent information for a UE. For model training and performance monitoring related to LMF based models for AI/ML positioning, the LMF checks the user consent status at the UDM  before collecting UE related data.

2nd change

### 8.3.1	General

The following table shows the LMF Services and LMF Service Operations.

Table 8.3.1-1: List of LMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nlmf_Location | DetermineLocation | Request/Response | AMF |
|  | EventNotify | Notify | GMLC |
|  | CancelLocation | Request/Response | AMF |
|  | LocationContextTransfer | Request/Response | LMF |
|  | MeasurementData | Request/Response | LMF |
|  | UPConfig | Request/Response | AMF |
|  | UPSubcribe | Subscribe/Notify | AMF |
|  | UPNotify | Subscribe/Notify | AMF |
| Nlmf_Broadcast | CipheringKeyData | Notify | AMF |
| Nlmf_DataExposure | Subscribe | Subscribe/Notify | NWDAF, DCCF |
|  | Notify | Subscribe/Notify | NWDAF, DCCF |
|  | UnSubscribe | Subscribe/Notify | NWDAF, DCCF |

End of changes