3GPP TSG-WG SA2 Meeting #169	S2-2505342

Fukuoka City, Fukuoka, JP, 19th May – 23rd May, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	Update of Procedures between AIOTF and NG-RAN for Indirect Connectivity

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: the pCR is to update Procedures between AIOTF and NG-RAN for Indirect Connectivity.

1. Introduction/Discussion

Clause 6.2.4 describes the Procedures between AIOTF and NG-RAN for Indirect Connectivity.

Step 3 covers several Ambient IoT operation cases in NG-RAN:

3.	The NG-RAN sends an NGAP message (AIOTF ID, AIoT Transfer Container) to an AMF. AIoT Transfer Container may be Inventory Response Transfer, Inventory Report Transfer, Inventory Failure Transfer, Command Response Transfer or Command Failure Transfer.

Within all the cases, they can be categorized into two aspects:

- Immediate response from NG-RAN: Inventory Response Transfer, Inventory Failure Transfer, Command Failure Transfer;

- Response with delay in NG-RAN: Inventory Report Transfer, Command Response Transfer;

However, current description in step 4 is a notify operation, which is not the correct service operation for the first category.

For the latter category NG-RAN needs to initiate procedures on air interface with AIoT device before replying to the core network. Therefore the “notify” in step 4 is not an immediate response after step 2/3, which needs to be clarified in the figure and step description.

It is proposed to clarify the two aspects in this pCR.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change * * * *

6.2.4	Procedures between AIOTF and NG-RAN for Indirect Connectivity

An AIOTF and NG-RAN can use an indirect interface via an AMF as described in clause 4.2.2.4. The additional steps used for indirect interface between AIOTF and NG-RAN are shown in Figure 6.2.4-1.

Figure 6.2.4-1: Procedure for AIOTF and NG-RAN for indirect connectivity via an AMF

1.	The AIOTF sends Namf_AIoT_MessageDelivery Request message (AIoT Transfer Container, NG-RAN ID, AIOTF ID, correlation ID, Notification URI, Message Type for AIoT Transfer Container) to the AMF. The AIoT Transfer Container may be Inventory Request Transfer or Command Request Transfer.

2.	The AMF sends the Namf_AIoT_MessageDelivery Response message (ACK) to the AIOTF immediately.

3.	The AMF sends an NGAP message (AIOTF ID, correlation ID, AIoT Transfer Container) to the target NG-RAN. The NG-AP message is either an Inventory Request or a Command Request message.

4a.	In case that step 3 is an Inventory Request message, the NG-RAN sends an NGAP inventory response message (AIOTF ID, correlation ID, AIoT Transfer Container) to the AMF indicating that the inventory request indicated by the AIoT Transfer Container is received successfully. AIoT Transfer Container may be Inventory Response Transfer, or Inventory Failure Transfer.

4b. In case that step 3 is a Command Request message, the NG-RAN may send an NGAP command response message (AIOTF ID, correlation ID, AIoT Transfer Container) to the AMF indicating that the command request indicated by the AIoT Transfer Container is received successfully. AIoT Transfer Container is Command Failure Transfer.

5.	Once receiving either step 4a or 4b, the AMF sends the Namf_AIoT_Notify message (AIoT Transfer Container, Notification URI) to the AIOTF.

(Interaction between NG-RAN and AIoT Device)

6a.	The NG-RAN sends an NGAP inventory report message (AIOTF ID, correlation ID, AIoT Transfer Container) to an AMF. AIoT Transfer Container is Inventory Report Transfer.

6b.	The NG-RAN sends an NGAP command message (AIOTF ID, correlation ID, AIoT Transfer Container) to an AMF. AIoT Transfer Container is Command Response Transfer.

7.	Once receiving either step 6a or 6b, AMF sends the Namf_AIoT_Notify message (AIoT Transfer Container, Notification URI) to the AIOTF. AIoT Transfer Container may be an Inventory Report Transfer or Command Response Transfer.

# * * * * End of changes * * * *