3GPP TSG-SA WG2#169	S2-2505348

Fukuoka, Japan, 19th – 23rd May, 2025	(was S2-2503829)

Source:	Huawei, HiSilicon

Title:	Discussion on Rel-20 FS_XRM Ph3

Document for:	Discussion

Agenda Item:	30.6

Work Item / Release:	Rel-20

# 1	Discussion

## 1.1	General

This paper intends to further justify the necessities for study on XRM Ph3 and also give the potential objectives in the end.

# 2	Justification

With the higher bandwidth, lower latency, and larger capacity provided by 5GS, various emerging multimedia services are expected to be more and more popular in the mobile network, such as short video, live streaming and on-demand high resolution video, Augment Reality (AR) / Virtual Reality (VR) and cloud gaming services. In order to better support these services, the following aspects need to be considered:

1. Coordination between Application and Network

Since the beginning of 5G era, plenty of features have been introduced to enhance the coordination between the application and networks via N33/N5 network APIs, e.g., application function requested QoS handling, the PDU Set based handling, the network information exposure and ECN marking for L4S.

However, the commercialization progress of such AF/AS based coordination features via network APIs is not very successful due to a number of factors including difficulty for the 3rd party applications of understanding how to deploy on Application Function; the need to perform API tests with hundreds of MNOs all over the world and the need for MNOs to negotiate with thousands of 3rd party applications. Another major drawback of the network APIs mechanism is that it does not allow real time coordination between AF/AS and 5G system which is a key aspect for emerging real-time application services, e.g. AR/VR, cloud gaming.

Hence, it is necessary to introduce a mechanism allowing real time coordination between the application and the 5G system as alternative to the existing AF/AS based coordination. This mechanism should be easier to deploy (for MNOs) and more friendly to use (for application developers). The information that can be made available in the pre-Rel-20 specifications such as network information related to congestion, available data rate and traffic characteristics (periodicity) can form a solid basic set of what can be exchanged in real time.

In addition to the AF/AS based coordination, the coordination can also take place between application client on board UE and network.

The coordination between application client and 5G system is better realised by allowing UE side involvement. At UE side, thanks to the developer-friendly development ecosystem, one or more application clients can coordinate with the network via the well-defined APIs exposed by UE/ mobile OS.

With this approach, the UE/OS provider is acting as a bridge between application and the network and can acquire more knowledge on upper layer applications and network conditions improving the service rendering. At the same time, the operator can also realize potential network capability monetization by using the existing OS/UE chipset ecosystem.

Given the position of this coordination layer within the device, there could be several possible approaches to the realisation of the coordination between application and network, i.e. with UE modem involvement and without UE modem involvement.

- 	Coordination layer as a middleware, i.e. SDK, deployed above the mobile OS layer. In this case, the coordination layer exposes APIs to the upper application layer and use existing functionalities provided by the mobile OS to enable bidirectional information exchange for better user experience.

-	Coordination layer deployed within the mobile OS. In this case, the mobile OS needs to expose the relevant APIs to the upper application layer for bidirectional information exchange with Core Network via the existing network connection capability provided by the modem layer.

-	Coordination layer deployed within the Modem layer. In this case, the mobile OS exposes the relevant APIs to the upper application layer and further exchanges with the modem layer to enable the bidirectional information exchanges between application layer and the Core Network.

The above may be left to the device internal implementation without standardization. However, besides of which information to be bidirectionally exchanged between the application layer and the core network, the details on the protocols/header extensions used for this purpose also needs to be further studied depending on the detailed use cases.

It should be noted that SA6 is already studying the evolution of the network exposure framework in their SEAL phase 4 SID and one of the objectives is to study how application client can obtain network services via SEAL client deployed in UE side. As a consequence, it would also be a feasible option for SA6 to design the APIs for exchanging network information between application client and SEAL client with SA2 defining how and what information is exchanged between UE and core network.

As an example of coordination between application client and 5G system we can consider the following scenario: the application client could receive assistance information for application-level parameters adjustment, e.g. frame rate, resolution based on the real network information and UE resources constraints (e.g. background services).

To realise this example, the application client subscribes to the real time network information and also UE constraints via specific APIs. Then the coordination layer in the UE side interacts with the core network to retrieve these information and when certain criteria are met, the subscribed application client would be notified.

Through the same channel, the application client can also send its traffic characteristics to core network via the coordination layer for appropriate resource scheduling and/or power saving.

Based on the above analysis, the following Work Task is to be proposed:

WT-X: Enhancements for information exposure to the application and improved traffic management

-	Study whether and how to enable coordination between CN and Application via a direct communication between the Application Client (AC) at the UE and the UPF,  enabling bi-directional information exchange of e.g. rate limitation information, available bit rate information, congestion information  (from the network to the AC), traffic characteristics (from the AC to the network).

NOTE: The information exchange shall be based on operator policies/SLAs.

2. Efficient In-Network Media Delivery

User experience of live streaming services is highly affected by end-to-end latency, jitter and video resolution, which means low latency and low jitter can reduce the first screen delay and video frame freezing and high video resolution provides better picture quality. However, the QoS model of the 5G network does not consider the QoS treatment on the N6 side, the existence of unstable N6 delay and jitter has a negative impact on the end-to-end delay and jitter.

In addition, the video resolution selection may be restricted by the server side due to outbound bandwidth cost, for example, the high resolution may be limited during busy hours (with a large number of access users), which restricts the use of the high-resolution video.  In particular for the case of high-volume traffic the traditional architecture using content delivery network or application server, high egress costs on N6 (i.e. between the CDN or application server and the UPF) can occur.  Even in the case the CDN or application server deployed by the operator of mobile network, in practice it still typically operates by a separate entity from core network and egress costs will still be a factor on N6.

Distribution of high-resolution video at UPF has the potential to solve the above issues which can be significant for the high volume, high data-rate applications.

To support high-resolution video distribution at UPF, the definition of UPF capability of media content distribution is needed, and also the study of discovery of such UPF for UE (e.g. based on DNS procedure) is needed.

For the high-resolution video distribution at UPF, SLA is needed between mobile operator and application service provider to determine which video are to be distributed at UPF, and mobile operator is not to be aware of the content of the video, e.g. video content can be encrypted between UE and APP server at application layer. Also, when mobile operator is to provide the service, the user’s subscription needs to be considered.

Some additional benefits of high-resolution video distribution at UPF:

-	Low deployment cost. Only needs updates on the existing UPF, no additional device/server is needed.

-	No need for customization of N6 protocol for traffic characteristics/information transmission from App server to UPF. Some traffic characteristics/information (e.g. data burst size, TTNB etc) have been defined, which can be used by NG-RAN to assist radio resource management, currently the traffic characteristics/information needs to be provided from App server to 5GC via a predefined N6 protocol, but in some cases, it may not easy for such N6 protocol(s) to be adopted by the OTT server.

Based on the above analysis, the following Work Task is to be proposed:

WT-Y: Study improvements for media delivery by the user plane in 5GC/UPF when multiple users access the same media service e.g. live streaming service, in order to reduce transmission latency and bandwidth, for the use of MoQ and other protocols e.g. http(s).

3. N3 delay awareness for latency guarantee at RAN

The CN-PDB for the transport network indicates the upper bound of packet latency over N3/N9, which is based on the operator’s configuration. However, it can be quite dynamic given the number of routers that the packet travelled, the load status of these routers, etc. Besides, RAN is discussing how to enhance latency guarantee in Rel-20 based on coordination between RAN and CN to enable/disable N3 interface delay measurement from CN to gNB in order to assist gNB for DL radio resource scheduling considering the N3 delay. Current N3 delay measurement mechanism designed for QoS Monitoring could be reused with considering the new usage in RAN side, which depends on RAN discussion.

Based on the above analysis, the following Work Task is to be proposed:

WT-Z: Study how to enable/disable N3 interface delay measurement to support better latency guarantee at the RAN node.

NOTE: The scope of this WT should be aligned with RAN.

# 3	Proposals

The following WTs are proposed for FS_XRM_Ph3:

WT-X: Enhancements for information exposure to the application and improved traffic management

-	Study whether and how to enable coordination between CN and Application via a direct communication between the Application Client (AC) at the UE and the UPF,  enabling bi-directional information exchange of e.g. rate limitation information, available bit rate information, congestion information  (from the network to the AC), traffic characteristics (from the AC to the network).

NOTE: The information exchange shall be based on operator policies/SLAs.

WT-Y: Study improvements for media delivery by the user plane in 5GC/UPF when multiple users access the same media service e.g. live streaming service, in order to reduce transmission latency and bandwidth, for the use of MoQ and other protocols e.g. http(s).

WT-Z: Study how to enable/disable N3 interface delay measurement to support better latency guarantee at the RAN node.

NOTE: The scope of this WT should be aligned with RAN.