SA WG2 Meeting #169	S2-2505367

# May 19 – 23, 2025, Fukuoka, Japan

Source: 	NTT DOCOMO

Title: 	KI#1: New Sol: Support of CP/UP combination and QoS for IMS voice service over GEO satellite

Document for: 	Approval

Agenda Item: 	20.1.1 (FS_5GSAT_Ph4_ARC)

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#1.

1	Introduction

This contribution proposes a solution for KI#1 on IMS voice service over GEO satellite, in particular addressing QoS aspects. The solution utilizes paired two PDN connections to address the QoS aspects. In addition, this solution allows EPC to use any combinations of CP and UP for transfer of SIP signalling and voice media.

There is an issue of coexistence of S11-U and S1-U which occurs for the cases "SIP message in CP, voice media in UP" and "SIP message in UP, voice media in CP", while this issue does not occur for the cases "SIP message in CP, voice media in CP" and "SIP message in UP, voice media in UP". This issue needs to be resolved if the former cases are considered. This contribution proposes a solution for this issue as well.

NOTE:	This paper is a revision of S2-2503575. The description related to the above issue is added (i.e., clause 6.x.4 and part of clause 6.x.5).

2	Proposal

It is proposed to add the following text to TR 23.700-19 v010.

********** First Change **********

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 |  |  |  |
| X | X |  |  |  |

********** Next Change (All new text) **********

6.x	Solution #X: Support of CP/UP combination and QoS for IMS voice service over GEO satellite

6.x.1	High level principles

This solution addresses KI#1, in particular QoS aspects.

This solution utilizes a pair of PDN connections.

NOTE:	This solution does not address any optimization aspect e.g., on how to achieve a shorter call setup time or how to transfer SIP message/voice media efficiently in CP or UP. This solution is intended to be used together with other solutions to achieve such optimization. This solution itself can be further optimized for that aspect.

6.x.2	Description

Principles of this solution are as follows:

- No impacts on IMS nodes in the network.

- No impacts on the QoS mechanism in UE using NB IoT.

- Two PDN connections are used for IMS voice, where one is used for SIP message exchange and the other is used for voice media. These two PDN connections are linked together by using (newly introduced) PDN connectivity pair ID. Each of these two PDN connections is with or without DRB.

- Each of the two PDN connections, when established with DRB, uses a default bearer only as per the existing specification. Different QCI is used for the default bearer depending on APN.

- UE may explicitly request to establish PDN connection without DRB by using (newly introduced) "Control Plane Only Indicator requested" indicator.

- UE internally associates contexts of the two PDN connections.

- PCRF internally associates contexts of the two PDN connections.

- With this solution, EPC can cope with four different combinations of CP and UP, depending on UE preference, UE capability, UE's remaining UP resource, and operator policy.

NOTE:	UE preference is shown with "Control Plane Only Indicator requested" indicator. Dependency on UE capability, UE's remaining UP resource, and operator policy is according to the existing specification.

  - SIP message in CP, voice media in CP,

  - SIP message in CP, voice media in UP,

  - SIP message in UP, voice media in CP, while this seems not very useful, and

  - SIP message in UP, voice media in UP.

Aspects for further consideration are as follows:

- Whether QCI=1 is sufficient or a new QCI value needs to be introduced in order to express the difference between the usual voice media that assumes ptime=20ms and the GEO voice media that assumes ptime= 80ms or 100ms.

6.x.3	Procedures

6.x.3.1	General

The procedures in the below are an example and depict the case where SIP messages are transferred in CP and voice media is transferred in UP.

6.x.3.2	Establishment of PDN connection for SIP message exchange

Figure 6.x.3.2-1: Establishment of PDN connection for SIP message exchange

- UE performs Attach without PDN Connectivity. The Subscription-Data that MME has received from HSS contains APN-Configuration-Profile for APN=ims and QCI=5 and for APN=ims_media (a new value) and QCI=1 (or a new QCI value for the GEO voice media).

- UE that uses NB IoT recognizes that two PDN connections are required for the GEO voice call; one for SIP message exchange and the other for voice media. UE may decide to use Control Plane for SIP message exchange, while still having available DRB(s). UE sends the PDN connectivity request containing APN=ims, "Control Plane Only Indicator requested" (a new indicator), and PDN connectivity pair ID (a new parameter).

- Based on "Control Plane Only Indicator requested", MME decides to set Control Plane Only PDN Connection Indication. MME sends to S-GW the Create Session Request containing APN=ims, Control Plane Only PDN Connection Indication, and PDN connectivity pair ID.

- S-GW sends to P-GW the Create Session Request containing APN=ims, Control Plane Only PDN Connection Indication, and PDN connectivity pair ID.

- P-GW sends to PCRF the CC-Request containing APN=ims, "Control Plane Only Indicator" (a new indicator), and PDN connectivity pair ID. PCRF recognizes that this PDN connection is for SIP message exchange only and not for voice media.

- The rest is the same as the existing procedure.

- PDN connection is established. This PDN connection uses NAS for transferring data (i.e., SIP messages) between UE and MME and uses U-plane between MME and P-GW via S-GW.

6.x.3.3	IMS voice call setup

Figure 6.x.3.3-1: IMS voice call setup

- A path for SIP message exchange is established as a result of the procedure depicted in the previous sub-clause.

- UE initiates IMS voice call setup by sending SIP INVITE.

- P-CSCF receives SIP 183 Session Progress.

- P-CSCF sends to PCRF the AA-Request to establish a dedicated bearer for voice media.

- PCRF recognizes that the targeted PDN connection is for SIP message exchange only and that UE is expected to establish another PDN connection for voice media when the UE receives SIP 183 Session Progress. PCRF decides not to send any message to P-GW. PCRF sends to P-CSCF the AA-Answer with success. (new behavior of PCRF)

- P-CSCF sends SIP 183 Session Progress. SIP 183 Session Progress is transferred to MME on U-plane via P-GW and S-GW.

- MME sends SIP 183 Session Progress to UE by using NAS.

- UE recognizes the need to establish PDN connection for voice media. UE sends the PDN connectivity request containing APN=ims_media and PDN connectivity pair ID. UE sets the value of the PDN connectivity pair ID to be the same as that for the PDN connection for SIP message exchange.

- MME confirms that UE has available DRB(s). MME sends to S-GW the Create Session Request containing APN=ims_media, QCI=1(or a new QCI value for the GEO voice media), and PDN connectivity pair ID.

- S-GW sends to P-GW the Create Session Request containing APN=ims_media, QCI=1(or a new QCI value for the GEO voice media), and PDN connectivity pair ID.

- P-GW sends to PCRF the CC-Request containing APN=ims_media, QCI=1(or a new QCI value for the GEO voice media), and PDN connectivity pair ID. PCRF internally associates contexts of the two PDN connections having the same PDN connectivity pair ID. PCRF recognizes that this newly established PDN connection is for voice media.

- The procedure for PDN connectivity establishment continues.

- PDN connection is established. This PDN connection uses U-plane for transferring data (i.e., voice media) between UE and P-GW via S-GW.

- The procedure for IMS voice call setup continues.

- S-CSCF sends SIP 200 OK to P-CSCF.

- When the procedure of Enabling of IP Flows is used, P-CSCF sends the AA-Request to PCRF.

- PCRF receives the AA-Request to the PDN connection for SIP message exchange, checks the context for the PDN connection for voice media as well, and decides to send the RA request to P-GW for PDN connection for voice media. PCRF sends to P-CSCF the AA-Answer with success. (new behavior of PCRF)

- PCRF sends the RA-Request to P-GW for the PDN connection for voice media.

- P-GW sends the RA-Answer to PCRF.

- The procedure for IMS voice call setup continues. Then, IMS voice call is setup.

6.x.4	Consideration for a particular issue

6.x.4.1	General

The issue of coexistence of S11-U and S1-U occurs for the cases "SIP message in CP, voice media in UP" and "SIP message in UP, voice media in CP", while this issue does not occur for the cases "SIP message in CP, voice media in CP" and "SIP message in UP, voice media in UP". This issue needs to be resolved if the former cases are considered.

6.x.4.2	Issue

TS 23.401, clause 4.10, contains the following: "the Control Plane CIoT Optimisation can be used to support PDN connections to an SCEF, while regular S1-U data transfer is used independently to support PDN connections to P-GW. The MME shall consistently include Control Plane Only Indicator either in all SGi PDN connections of a UE or in none of them. All the SGi PDN connections of a UE shall either use S11-U or S1-U at any point in time. "

6.x.4.3	History of discussion of this issue

The initial version of the above text was introduced by CR S2-163058 in Rel-13. The cover page contains the following: "CR disallows supporting multiple SGi PDN connections for a UE , some using CP and others using UP concurrently, i.e. concurrent S11-U and S1-U bearers, as there is no use case/benefit and this would cause larger SGW impacts as changing the existing EPS principle design (for SGW to see all its S1-U bearers or none established at the same time for LTE)."

This CR is based on a discussion paper S2-161443. This discussion paper contains the following: "One other issue that was identified in the SA2 CIoT conference call of 22 March is which addresses the Modify Bearer Request in step 9 is going to contain. If some PDN connections are provided using UP optimisation/S1-U and some PDN connections using CP it means that the Modify Bearer Request needs to contain both the MME address (for the PDN connections using the CP optimisation) and the eNB address (for the PDN connections using the UP optimisation/S1-U). In order to avoid this complexity it is proposed that: Proposal 2: Only data using “PDN connections to SCEF” can use CP optimisation after the AS context is established/resumed."

6.x.4.4	Proposed solution to this issue

The solution below is proposed to solve this issue:

UE having both an SGi PDN connection using S1-U and an SGi PDN connection using S11-U activates the former PDN connection even when the latter PDN connection has a pending data. MME sends to S-GW both S1-U eNB TEID, which MME receives in Initial Context Setup response, and S11-U MME TEID, which MME reserves in itself, by using Modify Bearer Request. Then both these PDN connections are activated. UE sends the pending data via the latter PDN connection. In order to allow this, MME is configured to allow establishing these PDN connections under a certain condition (e.g., UE has sent Preferred CIoT network behaviour and UE's usage setting=voice in the Attach procedure).

6.x.5	Impacts to Services, Entities and Interfaces

UE:

- Handling of a new parameter "PDN connectivity pair ID" and of a new indicator "Control Plane Only Indicator requested".

- In order to solve the issue of coexistence of S11-U and S1-U:

  - Handling of the order of activation of an SGi PDN connection using S1-U and activation of an SGi PDN connection using S11-U.

MME:

- Transparent forwarding of a new parameter "PDN connectivity pair ID". Handling of a new indicator "Control Plane Only Indicator requested".

- In order to solve the issue of coexistence of S11-U and S1-U:

  - Making sure that S-GW receives all the necessary S1-U eNB TEID and S11-U MME TEID when SGi PDN connections are activated.

  - Allowing coexistence of an SGi PDN connection using S1-U and an SGi PDN connection using S11-U under certain conditions.

S-GW, P-GW:

- Transparent forwarding of a new parameter "PDN connectivity pair ID"

PCRF:

- Handling of a new parameter "PDN connectivity pair ID" and of a new indicator "Control Plane Only Indicator"

********** End of Changes **********