SA WG2 Meeting #169	S2-2505369

# May 19 – 23, 2025, Fukuoka, Japan

Source: 	NTT DOCOMO

Title: 	KI#1: New Sol: Support of PDN connection where packets are transported as Non-IP over the GEO satellite and seen as IP by IMS nodes

Document for: 	Approval

Agenda Item: 	20.1.1 (FS_5GSAT_Ph4_ARC)

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#1.

1	Introduction

This contribution proposes a solution for KI#1 on IMS voice service over GEO satellite, in particular addressing message size optimization. In this solution, for UL, UE sends Non-IP packets and P-GW adds IP header. For DL, P-GW receives IP packets, removes IP header, and sends Non-IP packets.

2	Proposal

It is proposed to add the following text to TR 23.700-19 v010.

********** First Change **********

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 |  |  |  |
| Z | X |  |  |  |

********** Next Change (All new text) **********

6.x	Solution #Z: Support of PDN connection where packets are transported as Non-IP over the GEO satellite and seen as IP by IMS nodes

6.x.1	High level principles

This solution addresses KI#1, in particular message size optimization.

In this solution, for UL, UE sends Non-IP packets and P-GW adds IP header. For DL, P-GW receives IP packets, removes IP header, and sends Non-IP packets.

NOTE:	This solution is intended to be used together with other solutions.

6.x.2	Description

Principles of this solution are as follows:

- No impacts on IMS nodes in the network.

- For UL, UE sends Non-IP packets and P-GW adds IP header. For DL, P-GW receives IP packets, removes IP header, and sends Non-IP packets.

- UE receives an IP address allocated to it from P-GW at the PDN connection establishment and uses the IP address when creating contents of SDP messages.

- UE sends to P-GW an IP address allocated in IMS AGW, when the UE receives the IP address in SDP. P-GW uses this IP address when adding IP header to voice media packets.

6.x.3	Procedures

6.x.3.1	General

The procedures in the below are an example and depict the case where the proposed solution is used together with another solution shown in "Support of CP/UP combination and QoS for IMS voice service over GEO satellite".

6.x.3.2	Establishment of PDN connection for SIP message exchange

Figure 6.x.3.2-1: Establishment of PDN connection for SIP message exchange

- UE performs Attach without PDN Connectivity. The Subscription-Data that MME has received from HSS contains APN-Configuration-Profile for APN=ims and QCI=5 and for APN=ims_media (a new value) and QCI=1 (or a new QCI value for the GEO voice media).

- UE that uses NB IoT recognizes that two PDN connections are required for the GEO voice call; one for SIP message exchange and the other for voice media. UE may decide to use Control Plane for SIP message exchange, while still having available DRB(s). UE sends the PDN connectivity request containing APN=ims, "Control Plane Only Indicator requested" (a new indicator), and PDN connectivity pair ID (a new parameter). This PDN connectivity request additionally contains PDN type=IPv6 and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- Based on "Control Plane Only Indicator requested", MME decides to set Control Plane Only PDN Connection Indication. MME sends to S-GW the Create Session Request containing APN=ims, Control Plane Only PDN Connection Indication, and PDN connectivity pair ID. This Create Session Request additionally contains PDN type=IPv6 and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- S-GW sends to P-GW the Create Session Request containing APN=ims, Control Plane Only PDN Connection Indication, and PDN connectivity pair ID. This Create Session Request additionally contains PDN type=IPv6 and Extended protocol configuration options that include "CN header handling request" (a new indicator).

- P-GW sends to PCRF the CC-Request containing APN=ims, "Control Plane Only Indicator" (a new indicator), and PDN connectivity pair ID. PCRF recognizes that this PDN connection is for SIP message exchange only and not for voice media.

- PCRF sends to P-GW the CC-Answer. P-GW recognizes the need of the CN header handling. P-GW receives or assigns an IP address for UE and recognizes that this IP address is to be used for the source address for UL packets for the CN header handling for this PDN connection. P-GW recognizes that an P-CSCF address that the P-GW has known by configuration is to be used for the destination address for UL packets for the CN header handling for this PDN connection based on Access Point Name = ims and PDN connectivity pair ID. (new behavior of P-GW)

- P-GW sends to S-GW the Create Session Response containing PDN Address Allocation that includes the allocated IPv6 address and Extended protocol configuration options that include "CN header handling accepted indication" (a new indicator).

- S-GW sends to MME the Create Session Response containing Control plane only indication, PDN Address Allocation that includes the allocated IPv6 address, and Extended protocol configuration options that include "CN header handling accepted indication" (a new indicator).

- MME sends to UE the Activate Default EPS Bearer Context Request containing Control plane only indication, PDN Address Allocation that includes the allocated IPv6 address, and Extended protocol configuration options that include "CN header handling accepted indication" (a new indicator). UE recognizes the IPv6 address allocated to it.

- UE sends to MME the Activate Default EPS Bearer Context Accept.

- PDN connection is established. This PDN connection uses NAS for transferring data (i.e., SIP messages) between UE and MME and uses U-plane between MME and P-GW via S-GW. In addition, this PDN connection is set to transfer Non-IP packets between UE and P-GW and the P-GW is expected to add IP header for UL packets and remove IP header for DL packets.

6.x.3.3	IMS voice call setup

Figure 6.x.3.3-1: IMS voice call setup

- A path for SIP message exchange is established as a result of the procedure depicted in the previous sub-clause.

- UE sends SIP INVITE without IP header to MME by using NAS.

- MME sends SIP INVITE without IP header to P-GW on U-plane via S-GW.

- P-GW adds IP header to SIP INVITE without IP header; P-GW adds the UE IP address as the source address and the P-CSCF IP address as the destination address. (new behavior of P-GW)

- P-GW sends SIP INVITE with IP header to P-CSCF.

- P-CSCF sends SIP INVITE to S-CSCF.

- P-CSCF receives SIP 183 Session Progress.

- P-CSCF sends to PCRF the AA-Request to establish a dedicated bearer for voice media.

- PCRF recognizes that the targeted PDN connection is for SIP message exchange only and that UE is expected to establish another PDN connection for voice media when the UE receives SIP 183 Session Progress. PCRF decides not to send any message to P-GW. PCRF sends to P-CSCF the AA-Answer with success. (new behavior of PCRF)

- P-CSCF sends SIP 183 Session Progress to P-GW.

- P-GW removes IP header of SIP 183 Session Progress. (new behavior of P-GW)

- P-GW sends SIP 183 Session Progress without IP header to MME on U-plane via S-GW.

- MME sends SIP 183 Session Progress without IP header to UE by using NAS.

- UE recognizes the need to establish PDN connection for voice media. UE recognizes an IP address allocated in IMS AGW in SDP answer in SIP 183 Session Progress without IP header. UE sends the PDN connectivity request containing APN=ims_media and PDN connectivity pair ID. UE sets the value of the PDN connectivity pair ID to be the same as that for the PDN connection for SIP message exchange. This PDN connectivity request additionally contains "Destination Address for CN header handling" (a new parameter) that includes the IP address allocated in IMS AGW, PDN type=IPv6, and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- MME confirms that UE has available DRB(s). MME sends to S-GW the Create Session Request containing APN=ims_media, QCI=1(or a new QCI value for the GEO voice media), and PDN connectivity pair ID. This Create Session Request additionally contains "Destination Address for CN header handling" (a new parameter) that includes the IP address allocated in IMS AGW, PDN type=IPv6, and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- S-GW sends to P-GW the Create Session Request containing APN=ims_media, QCI=1(or a new QCI value for the GEO voice media), and PDN connectivity pair ID. This Create Session Request additionally contains "Destination Address for CN header handling" (a new parameter) that includes the IP address allocated in IMS AGW, PDN type=IPv6, and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- P-GW sends to PCRF the CC-Request containing APN=ims_media, QCI=1(or a new QCI value for the GEO voice media), and PDN connectivity pair ID. PCRF internally associates contexts of the two PDN connections having the same PDN connectivity pair ID. PCRF recognizes that this newly established PDN connection is for voice media.

- PCRF sends to P-GW the CC-Answer. P-GW recognizes the need of the CN header handling. P-GW receives or assigns an IP address for UE and recognizes that this IP address is to be used for the source address for UL packets for the CN header handling for this PDN connection. P-GW recognizes that an IMS AGW address that the P-GW has received in "Destination Address for CN header handling" (a new parameter) is to be used for the destination address for UL packets for the CN header handling for this PDN connection based on Access Point Name = ims_media and PDN connectivity pair ID. (new behavior of P-GW)

- The procedure for PDN connectivity establishment continues.

- PDN connection is established. This PDN connection uses U-plane for transferring data (i.e., voice media) between UE and P-GW via S-GW. In addition, this PDN connection is set to transfer Non-IP packets between UE and P-GW and the P-GW is expected to add IP header for UL packets and remove IP header for DL packets.

- The procedure for IMS voice call setup continues.

- S-CSCF sends SIP 200 OK to P-CSCF.

- When the procedure of Enabling of IP Flows is used, P-CSCF sends the AA-Request to PCRF.

- PCRF receives the AA-Request to the PDN connection for SIP message exchange, checks the context for the PDN connection for voice media as well, and decides to send the RA request to P-GW for PDN connection for voice media. PCRF sends to P-CSCF the AA-Answer with success. (new behavior of PCRF)

- PCRF sends the RA-Request to P-GW for the PDN connection for voice media.

- P-GW sends the RA-Answer to PCRF.

- The procedure for IMS voice call setup continues. Then, IMS voice call is setup.

- UE sends UL voice media packets without IP header to P-GW on U-plane via S-GW.

- P-GW adds IP header to UL voice media packets without IP header; P-GW adds the UE IP address as the source address and the IMS AGW IP address as the destination address. (new behavior of P-GW)

- P-GW sends voice media packets with IP header to IMS AGW.

- P-GW receives voice media packets with IP header from IMS AGW.

- P-GW removes IP header of voice media packets. (new behavior of P-GW)

- P-GW sends voice media packets without IP header to UE on U-plane via S-GW.

6.x.4	Impacts to Services, Entities and Interfaces

UE:

- Handling of new indicators "CN header handling request" and "CN header handling accepted indication".

- Extracting an IMS AGW IP address from SDP and informing it to MME by using a new parameter "Destination Address for CN header handling".

MME:

- Transparent forwarding of a new parameter "Destination Address for CN header handling"

S-GW:

- Transparent forwarding of a new parameter "Destination Address for CN header handling"

P-GW:

- Handling of new indicators "CN header handling request" and "CN header handling accepted indication”

- Adding IP header for UL packets and removing IP header from DL packets.

- Receiving an IP address in a new parameter "Destination Address for CN header handling" from S-GW and using it when adding IP header for UL packets as the destination address.

********** End of Changes **********