2276SA WG2 Meeting ##169	S2-2505376

19 - 23 May, 2025, Fukuoka, Japan

# Title:	DRAFT response LS on Handling of UE Radio Capability for Paging

# Response to:	R3-252276=S2-2504518

# Release:	Rel-16

# Work Item:	NR_newRAT-Core

Source:	Vodafone [ to be SA2]

To:	RAN3

Cc:	RAN2

Contact Person:

#### Name:	Chris Pudney

#### Tel. Number:

#### E-mail Address:	chris dot pudney at vodafone dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR 6247 to TS 23.501

1. Overall Description:

SA2 thanks RAN3 for their LS and would like to provide the following answers to RAN3’s questions.

Q1) 	RAN3 would like to ask SA2 that when the NG-RAN node receives the UE Radio Capability for Paging IE in the INITIAL UE CONTEXT SETUP REQUEST message, and if the UE Radio Capability for Paging Information does not contain paging information on all NG-RAN RATs that the UE supports, then should the NG-RAN node send to the AMF the paging information on the missing NG-RAN RATs or all NG-RAN RATs that the UE supports in the UE RADIO CAPABILITY INFO INDICATION message?

The AMF simply stores the UE Radio Capability for Paging IE that is sent to it from the RAN and inserts it into subsequent NG interface paging messages. The AMF never interprets the information in it, nor processes it. Hence SA2 believe that the RAN will need to send the information on ALL of the NG-RAN RATs to the AMF in a single information element.

SA2 has agreed the attached CR which may clarify this. The CR leaves it up to RAN 3 to decide on how to do this update, but it would be appreciated if the solution requires no changes to the AMF, e.g. by using existing signalling.

Q2) 	RAN3 would also like to ask if the “that PLMN” in TS 23.501 clause 5.4.4.3 is interpreted as the serving PLMN?

After checking TS 21.905 and TS 24.301, SA2 observe that “serving PLMN” is not a defined term. SA2 expect that RAN3 can regard “that PLMN” as the serving PLMN, but SA2 have not changed the wording in TS 23.501 as there may be corner cases (e.g. registering on a 2G network in a different PLMN would not delete the UE context in the AMF; or, the use of EPLMN functionality to have different PLMN IDs on different RATs of that PLMN) when the term “serving PLMN” is incorrect (and SA2 has no wish to ask CT4 and CT1 to work with SA2 on an exhaustive analysis).

2. Actions:

To SA2.

ACTION: 	SA2 politely asks RAN3 to take the above into account.

3. Date of Next TSG SA2 Meetings:

SA2#170  	25-29 August 2025 	Goteborg, SE

SA2#171	13-17 October 2025	China (TBC), CN