3GPP SA WG2# 169	S2-2505387

Fukuoka, Japan, 19 – 23 May 2025

# Source:	NEC

# Title:	Update to new WID on Indirect Network Sharing phase2

# Document for:	Agreement

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Indirect Network Sharing phase2

Acronym:	NetShare_ph2-ARC

Unique identifier:

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| TEI19_NetShare | SA2 | 1030011 | Indirect Network Sharing |
| NetShare_Ph2 | SA1 | 1050106 | Indirect Network Sharing Phase 2 |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification:

# 3	Justification

WT#1: R19 leftover

When the operator deploys the Indirect Network Sharing currently, the serving PLMN ID in the message which is sent to the NFs of the participating operator network is set to the HPLMN ID or EHPLMN ID if the shared RAN broadcasts the HPLMN ID or EHPLMN ID of UE of participating operator. In this case, the NFs of the participating operator network may not be able to identify the INS case and the hosting operator network. It will cause that some PLMN related subscription data (e.g. mobility restriction parameter) may not be retrieved by the hosting operator network accurately and the PLMN specific policy (e.g. session management policy, charging policy) may not be performed correctly. The above issue will also happen if more than one hosting operators correspond to one participating operator and the same PLMN ID broadcast by the shared RAN of these hosting operators for the participating operator, the participating operator cannot differentiate the hosting operator.

WT#2: INS for disaster condition

With the development and deployment of 5G network, the operator has paid more and more attention to the robustness of 5G network, e.g. how to minimize the service interruption when the network meets some unexpected disaster conditions.

The existing mechanism to solve the network failure for disaster condition is disaster roaming as specified in MINT feature in R17. This mechanism needs the tight coordination between the network and UE, the UE is required to support the disaster roaming (e.g. UE needs to support lots of indicators sent from core network and supports to bypass the list of forbidden PLMNs). However, most of the existing UEs do not support these functionalities based on the factual situation from the operator network, it causes the MINT mechanism cannot be deployed quickly.

For these reasons, the Indirect Network Sharing mechanism for disaster condition considering the factual situation of legacy UEs needs to be considered and this use case has been specified in TS 22.261. As shown in Figure-1 below, if the OP1’s RAN broke down, the UE of OP1 can access the network via OP2’s network using the mechanism of Indirect Network Sharing for disaster condition.

Figure-1

Specifically, the participating operator OP1 may have the Indirect Network Sharing deployment and agreement with hosting operator OP2 in the specific area. However, in the other area, OP1 network will not accept any messages from the OP2 network if there is the non-shared area. When the disaster arises in the non-shared area, how to perform the Indirect Network Sharing for disaster condition, e.g. how to ensure the OP1 network to accept the message from OP2 and in particular let the OP1 UDM response the specific subscription data related to the disaster condition to OP2 network needs to be considered.

Additionally, when a huge amounts of UEs of participating PLMN with the disaster condition attempt to access the hosting PLMN(s), in order to avoid the service impact on the UE of the hosting PLMN, how to prevent the high load condition of the core network of hosting PLMN needs to be considered. In the factual case that there may be three or even more operators' 5G networks in the same geographic area as shown in Figure-2 below. In this case, if the NG-RAN nodes of OP1 broke down in the disaster area, the Indirect Network Sharing for disaster condition can be performed via OP2 network or OP3 network for the UEs of OP1, in this case if the signaling strength of the shared RAN of OP2 is better than OP3 in the specific area, the UEs of OP1 may try to register to the OP2 network first, then how to distribute part of UEs of OP1 to the OP3 network if the OP2 network meets the high load condition needs to be considered.

Figure-2

In rel-19, there is a limitation that the UE cannot get a service over non-3GPP access when the UE is registering or registered over 3GPP access to the participating network. In case the user is accessing the services over non-3GPP access, and the UE starts registering to participating network over 3GPP access then the service over non-3GPP

access will not be continued. This will seriously affect the user experience.

# 4	Objective

The objectives of this work item are the stage 2 aspects for further enhancement on Indirect Network Sharing and supporting the SA1 requirements:

WT#1: Enable the NFs of the participating operator’s network (i.e. UDM, SMF, UE-PCF, SM-PCF) to detect the Indirect Network Sharing case and identify the hosting operator’s network by e.g., adding some hosting operator information in the corresponding service operation messages.

WT#2: Support UEs without MINT capability using Indirect Network Sharing mechanism during disaster condition

-	Assumption is that the hosting operator is made aware of the participating operator’s disaster condition by means not in scope of 3GPP. Once Indirect Network Sharing for disaster condition has been triggered to hosting operator network for the participating operator which is in disaster condition, it needs to ensure the NFs of participating operator network (i.e. UDM, AUSF, SMF) to be aware of which UE(s) are in disaster condition via the message sent from the hosting operator network instead of in the normal Indirect Network Sharing case by e.g., adding the new indicator related to disaster in the corresponding service operation or reusing the existing indicator as specified in the disaster roaming if possible.

NOTE 1:  In the case that more than one hosting operators can provide the Indirect Network Sharing for disaster condition for the participating operator in the disaster area, the existing congestion control mechanism may be used to distribute the UE access to the different hosting operator networks in order to avoid the overload condition of each hosting operator network. This can consider to be recorded in the SA2 specification.

NOTE 2:  The Indirect Network Sharing for disaster condition can be applicable for the UE which cannot support the MINT feature.

NOTE 3:  The existing mechanism of access class barring may be reused to ensure the access priority of the specific UE (e.g. the UE of rescue worker) during the disaster condition. This can consider to be recorded in the SA2 specification.

WT#3: Support simultaneous services over 3GPP access and non-3GPP access in indirect network sharing case.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or is depended on completion of other WTs |
| --- | --- | --- | --- | --- |
| WT#1 | 0 | 1 | No |  |
| WT#2 | 0 | 1 | No |  |
| WT#3 | 0 | 0.5 | No |  |

Total TU estimates for the normative phase: 2 TUs

# 5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | The description for the enhancement of the impacted function. | TBD |  |
| TS 23.502 | Enhancement on the procedure and service operation according to the impacts identified in TS 23.501 | TBD |  |

# 6	Work item Rapporteur(s)

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>}

{Optional: <FamilyName>, <GivenName>, <Company>, <email address>: Secondary task(s)}

{The first listed Rapporteur is the work item primary Rapporteur. The role of a Rapporteur is further described in www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec. By default, the primary Rapporteur shall ensure the production of the post-completion summary. 
Secondary Rapporteur(s) are possible for specific secondary task(s), such as: "Write the post-completion summary"; "In charge of a specific aspect of the work item (specify which)"; "Rapporteur for a secondary responsible WG (specify which)"}

# 7	Work item leadership

SA WG2

# 8	Aspects that involve other WGs

SA WG5 – Management, Orchestration and Charging, if any identified.

SA WG3 - security aspect, if any identified.

# 9	Supporting Individual Members

Supporting IM name

NEC

China Unicom?

Huawei?

ZTE?

CATT?

China Telecom?