3GPP TSG-WG SA2 Meeting #169	S2-2505391

Fukuoka, Japan, 19-23 May, 2025

Source:	Xiaomi

Title:	Discussion on Rate Control

Document for:	Discussion

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2/ Rel-19

Abstract: This paper discusses open issues of XR rate control.

1. Introduction

Based on the LS from RAN2 (S2-2502820/R2-2501347) on uplink rate control, RAN2 discussed how the gNB gets the information of QoS flows which are subject to uplink rate control, i.e., whether the information is from CN or UE, and agreed that:

RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2.

During SA2#168 meeting, LS reply (S2-2504252) from SA2 was agreed and sent to RAN as follow:

- SA2 confirms that the CN can indicate to the gNB that a QoS flow is subject to uplink rate control.

- SA2 would like to inform RAN WGs that SA2 has not decided yet on whether the existing or a new mechanism can be reused.

- SA2 would like RAN2 to confirm that the RAN2 LS on UL rate control applies to both GBR and non-GBR.

After the above RAN LS received by SA2, there are two progresses achieved in RAN, as follows:

- In RAN2#129bis meeting, following agreements was agreed by RAN on XR rate control:

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

RAN2 to continue working on the uplink solution and to liaise with SA2/SA4 if there are issues if the same approach would be extended for the DL as well, assuming the solution extension for DL would not cause issues in RAN2 side (not more work than what would be needed to prevent use of the solution for downlink direction). RAN3 to be kept informed of the progress.

In RAN#106 meeting, downlink congestion indication (in addition of the uplink) was discussed, and RP-243305 is endorsed with related content as below:

The RAN discussion of open issues of XR rate control is ongoing, including the DL rate control, QoS identification, rate control MAC CE format etc.

In this contribution, we discuss open issues of XR rate control.

2. Discussion

The purpose of the RAN2 requirement (S2-2502820/R2-2501347) is to support the adjustment of UL data rate, considering the range / granularity of uplink bit rate recommended by SA4, during the network congestion.

If the gNB can get the indication that which QoS flows are subject to uplink rate control, the gNB can send the related MAC CE to UE to downgrade the UL data rate, when the UE can have the rate control capability (e.g., to adjust the Codec to achieve the rate control). Then it will have less impact with the QoE while the QoS downgrade to continue the XR service.

For now, no related indication or QoS information form CN can be reused to indicate the NG-RAN, whether the UL Rate Control can be triggered or not for the XRM service on one or more QoS flows.

Proposal 1: Indication for Rate Control is proposed to provide to the gNB to indicate that related QoS flow is subject to uplink rate control.

The range/granularity of uplink bit rate for Rate control is recommended by SA4. But whether the minimum and maximum value of range for uplink bit rate, is included in the GFBR and MFBR of the GBR QoS flow, i.e., Minimum value > GFBR and the Maximum value < MFBR, is not clarified.

With regard to the GBR QoS flow, when the Indication rate control provided to NG-RAN,

- For the Minimum value > GFBR and the Maximum value < MFBR, the NG-RAN can send the selected data rate to UE to downgrade the UL data rate when the Network congestion, and to send the reselected data rate to UE to upgrade the UL data rate when the Network congestion has been resolved.

No QoS notification for the UL QoS update is required from NG-RAN to the CN.

- For the Minimum value < GFBR, the NG-RAN can send the selected data rate to UE to downgrade the UL data rate, when the Network congestion, and selected data rate is more the GFBR.

Otherwise, the QoS Notification will be sent to the CN for the QoS Update. If the update request is authorized, then NG-RAN can send the selected data rate to UE to downgrade the UL data rate.

- For the Maximum value > MFBR, the NG-RAN can send the reselected data rate (the value is less than the MFBR) to UE to upgrade the UL data rate, when the Network congestion has been resolved.

It is not encouraged to upgrade the UL data rate with the value more than the MFBR, even the value before the congestion, because the Rate Control is proposed because of the network congestion scenario.

But the mechanism can be used to boost the UL data rate by selecting the UL data rate with the value more than the value before the congestion, or even more than the MFBR.

If the reselected value is more than the MFBR, the QoS Notification will be sent to the CN for the QoS Update. If the update request is authorized, then NG-RAN can send the selected data rate to UE to upgrade the UL data rate.

When the AQPs support for the GBR QoS flow, which indicated under the rate control, the GFBR and MFBR of the reselected QoS profile fulfilled can be used as above. The QNC procedure of the AQPs is the same as the exist mechanism.

Proposal 2: With regard to the GBR QoS flow, for the range of uplink bit rate for Rate control recommended by SA4, the Minimum value > GFBR and the Maximum value < MFBR is proposed. Otherwise QoS Notification should be sent to the CN for the QoS Update. Based on the update QoS authorized, then NG-RAN can send the selected data rate to UE to downgrade or upgrade the UL data rate.

Proposal 3: Both the AQPs and the rate control can be supported in the same time for the GBR QoS flow.

With regard to the non-GBR QoS flow, if the Indication rate control provided to NG-RAN, the NG-RAN can send the selected data rate to UE to downgrade the UL data rate when the Network congestion, considering the UE-AMBR, and the Session-AMBR if received.

The range/granularity of uplink bit rate for Rate control is recommended by SA4 for the non-GBR flow. The Maximum value of the range < Session-AMBR is proposed.

When the Reflective QoS support for the GBR QoS flow, which indicated under the rate control, the GFBR and MFBR of the reselected QoS profile fulfilled can be used as above. The QNC procedure of the AQPs is the same as the exist mechanism.

When the RQA is signalled for a QoS Flow, the uplink bit rate for Rate control cannot be applied, even the Indication of rate control provided to NG-RAN.

If both DL and UL bit rate for Rate control indicated to the NG-RAN, then the UL bit rate control can be achieved by the DL bit rate and the RQI marked.

Proposal 4: With regard to the non-GBR QoS flow, the range/granularity of uplink bit rate for Rate control is recommended by SA4 for the non-GBR flow. The Maximum value of the range < Session-AMBR is proposed.

Proposal 5: With regard to the non-GBR QoS flow, when the RQA is signalled for a QoS Flow, the uplink bit rate for Rate control cannot be applied, even the Indication of rate control provided to NG-RAN.

Proposal 6: With regard to the non-GBR QoS flow, when the RQA is signalled for a QoS Flow, if both DL and UL bit rate for Rate control indicated to the NG-RAN, then the UL bit rate control can be achieved by the DL bit rate and the RQI marked.

3. Conclusion

In this contribution, we discuss open issues of XR rate control, and propose the following:

Proposal 1: Indication for Rate Control is proposed to provide to the gNB to indicate that related QoS flow is subject to uplink rate control.

Proposal 2: With regard to the GBR QoS flow, for the range of uplink bit rate for Rate control recommended by SA4, the Minimum value > GFBR and the Maximum value < MFBR is proposed. Otherwise QoS Notification should be sent to the CN for the QoS Update. Based on the update QoS authorized, then NG-RAN can send the selected data rate to UE to downgrade or upgrade the UL data rate.

Proposal 3: Both the AQPs and the rate control can be supported in the same time for the GBR QoS flow.

Proposal 4: With regard to the non-GBR QoS flow, the range/granularity of uplink bit rate for Rate control is recommended by SA4 for the non-GBR flow. The Maximum value of the range < Session-AMBR is proposed.

Proposal 5: With regard to the non-GBR QoS flow, when the RQA is signalled for a QoS Flow, the uplink bit rate for Rate control cannot be applied, even the Indication of rate control provided to NG-RAN.

Proposal 6: With regard to the non-GBR QoS flow, when the RQA is signalled for a QoS Flow, if both DL and UL bit rate for Rate control indicated to the NG-RAN, then the UL bit rate control can be achieved by the DL bit rate and the RQI marked.