3GPP TSG-SA2 Meeting #169	S2-2505401

Fukuoka, Japan, May 19 – May 23, 2025 	(revision of S2-250xxxx)

Source:	CATT

Title:	Discussion on TEI20 WID of QoS Enhancement for MWAB

Document for:	Information

Agenda Item:	30.1

Work Item / Release:	TEI20_QoSforMWAB / Rel-20

Abstract of the contribution: this discussion paper is proposed to introduce TEI20 WID of QoS Enhancement for MWAB.

1. Introduction

This contribution provides discussions for the proposed TEI20 WID of QoS Enhancement for MWAB, including Stage 1 progress on Vehicle-Mounted Relays Phase 3 (VMR_Ph3), and analysis on gap and solution to support the SA1 requirement.

2. Stage 1 Progress on VMR_Ph3

SA1 WID on Vehicle-Mounted Relays Phase 3 (SP-241821) is approved in SA#106 meeting. And the WID captures the following service requirements in TS 22.261:

-	Subject to regulatory requirements and operator policy, the 5G network shall support a mechanism to determine suitable QoS parameters for traffic relayed via a mobile base station when the connection quality of the serving mobile base station relay changes, e.g. during mobility between terrestrial access network and satellite access network.

3. Support of MWAB mobility between TN access network and NTN access network

3.1	Existing Mechanisms

3.1.1	Satellite Backhaul Category

The satellite backhaul means the satellite is used as part of the backhaul between (R)AN and 5GC.

Satellite backhaul category refers to the type of the satellite (i.e. GEO, MEO, LEO or OTHERSAT, DYNAMIC_GEO, DYNAMIC_MEO, DYNAMIC_LEO and DYNAMIC_OTHERSAT) used in the backhaul.

The dynamic satellite backhaul category (i.e. DYNAMIC_GEO, DYNAMIC_MEO, DYNAMIC_LEO and DYNAMIC_OTHERSAT) refers that i.e. capabilities (latency and/or bandwidth) of the satellite backhaul change over time due to e.g. use of varying inter-satellite links as part of backhaul.

3.1.2	Policy control for satellite backhaul

3.1.2.1	Reporting satellite backhaul to PCF

Based on the following description in clause 5.43.4 in TS 23.501, during PDU Session Establishment procedure, the AMF is responsible to notify the SMF with the satellite backhaul is used. And then the SMF notifies such information to PCF.

If the AMF is aware that a satellite backhaul is used towards 5G AN, the AMF may report this to SMF as part of the PDU Session establishment procedure as described in clause 4.3.2 of TS 23.502 [3]. If AMF is aware that satellite backhaul category changes (e.g. at handover), the AMF reports the current Satellite backhaul category to SMF. The SMF reports it to the PCF when the "Satellite backhaul category change" PCRT was armed as specified in TS 23.503 [45], or notifies it to other NFs if requested as described in clause 5.2.8.3 of TS 23.502 [3]. When the backhaul network changes from a type of satellite to terrestrial network, the AMF reports that non-satellite backhaul network is used with the Satellite backhaul category.

When PCF receives the satellite backhaul, the usage of this parameter in PCF is specified in clause 6.1.3.5 in TS 23.503, as follows:

When the Satellite backhaul category change trigger is armed, the SMF reports to the PCF that the Satellite backhaul category change was met and the new Satellite backhaul category (including satellite backhaul is no longer used) when it becomes aware that there is a change of the backhaul which is used for the PDU Session between different types of satellite backhaul, or between satellite backhaul and a non-satellite backhaul. The SMF determines whether or not a satellite backhaul is used and whether there is a change of backhaul based on signalling from the AMF as specified in TS 23.501 [2].

NOTE 13:	As specified in clause 5.43.4 of TS 23.501 [2], Satellite backhaul category refers to the type of the satellite (or non-satellite) used in the backhaul. Only a single backhaul category can be indicated.

3.1.2.2	QoS monitoring when dynamic satellite backhaul is used

Based on the mechanism described in clause 2.2.1, if dynamic satellite backhaul is used to serve the PDU Session, the PCF behaviour is described in clause 6.1.3.6 in TS 23.503, as follows.

When SMF indicates that the dynamic satellite backhaul is used to serve the PDU Session, the PCF, based on local policy, may use QoS monitoring (as described in clause 6.1.3.21) to get reports for the packet delay (defined in clause 5.45.2 of TS 23.501 [2]). The PCF may take this reported packet delay information into account for the policy decision along with other criteria, such as the AF requested QoS requirements.

3.1.3	Summary of existing mechanism

Based on the existing mechanisms described in clause 2.2, PCF is the NF to use the satellite backhaul parameter which is provided by SMF. And SMF receives the parameter from AMF. AMF is responsible to determine whether and which satellite backhaul category is used to serve the PDU session based on configuration, as specified in clause 5.43.4 in TS 23.501, as follows.

NOTE:	It is assumed that the AMF can determine the Satellite backhaul category for the notification to the SMF based on local configuration, e.g. based on Global RAN Node IDs associated with satellite backhaul.

In general, the backhaul of gNB is fixed, so the local configuration in AMF is static information. The backhaul category change of the PDU Session is caused by UE mobility.

3.2	Gap analyse

In MWAB case, due to mobility of MWAB, the backhaul category of MWAB-gNB is not fixed which is different from gNB. So it is impossible to configure the backhaul category of MWAB-gNB in AMF in MWAB Broadcasted PLMN. In this case, the existing mechanism described in clauses 3.2 and 3.3 are not workable for MWAB case.

Furthermore, based on existing MWAB mechanism, the following scenarios may happen to transfer UE’s data via the BH PDU Session:

-	Scenario#1: There is only one BH PDU Session which is used to transfer all the data generated by the UEs connecting to the MWAB-gNB

-	Scenario#2: To guarantee the QoS of the emergency service, the emergency service data is transferred via the BH PDU Session with specific S-NSSAI. The non-emergency service data is transferred via another BH PDU Session.

-	Scenario#3: Dedicated BH PDU Sessions for specific S-NSSAI(s) in the MWAB Broadcasted PLMN.

For scenarios #2 and #3, the MWAB-gNB supports more than one backhaul paths which work independently. It is also not clear how to support these scenarios.

3.3	Solution

The following principles are proposed:

-	When the MWAB-UE has more than one BH PDU Session, these BH PDU Sessions use the same RAT Type.

-	No matter how many BH PDU Sessions are established by MWAB-UE, the backhaul category of MWAB-gNB is the RAT Type the MWAB-UE is camping on.

-	The RAT Type the MWAB-UE is camping on is determined by the serving AMF of MWAB-UE in BH PLMN based on the mechanism specified in clause 5.3.2.3 and/or local configuration, e.g. based on Global RAN Node IDs associated with satellite backhaul as specified in clause 5.43.4 in TS 23.501.

Based on the principles above, the candidate solutions to solve the issue described in clause 3.2 are shown in Table 1.

Table 1: solution description and impacts

| Solutions | Description | Impacts |
| --- | --- | --- |
| #1: Event exposure based solution | The AMF in MWAB Broadcasted PLMN subscribes to the RAT type change event report of the MWAB-UE from the BH PLMN. The AMF in BH PLMN is responsible to determine the RAT Type of MWAB-UE. When the change of RAT Type is detected, the AMF in BH PLMN sends the event report. Furthermore, based on the architectures for MWAB in Annex S in TS 23.501, the following cases may occur: - MWAB Broadcasted PLMN and the BH PLMN are same PLMN: the AMF in MWAB Broadcasted PLMN can subscribe the RAT Type event report from the AMF in BH PLMN directly. - MWAB Broadcasted PLMN and the BH PLMN are different PLMNs: the AMF in MWAB Broadcasted PLMN subscribes the RAT Type event report via NEF. In this case, the NEF also support the RAT Type event exposure. | MWAB-UE: Provide the identifier to MWAB-gNB MWAB-gNB: Provide the identifier of MWAB-UE to the AMF in MWAB Broadcasted PLMN. AMF: Support RAT Type event report. NEF: Support RAT Type event exposure. |
| #2: N2 based solution | The MWAB-UE notifies the RAT type to MWAB-gNB via internal interface. The MWAB-gNB sends the backhaul category which is the RAT Type of MWAB-UE to the AMF in the MWAB Broadcasted PLMN via N2 interface. | MWAB-UE: Provide the RAT Type to MWAB-gNB. MWAB-gNB: Provide the RAT Type of MWAB-UE via N2 interface to AMF in the MWAB Broadcasted PLMN. |

3.4	Evaluation and Proposal

Based on the impacts of the solutions in Table 1, the sol#2 has less impact than sol#1. But sol#2 cannot support dynamic satellite backhaul, because the MWAB-UE does not know the satellite RAT type is dynamic or not. So it is proposed to use sol#1 as the wayforward.

Proposal#1: when the MWAB mobility between terrestrial access network and satellite access network happens, to support the satellite backhaul reporting and QoS monitoring when dynamic satellite backhaul is used, the following principles are proposed

-	MWAB-UE provides the identifier (i.e. GPSI) to MWAB-gNB.

-	MWAB-gNB provides the identifier of the MWAB-UE via N2 message to the AMF in the MWAB Broadcasted PLMN.

-	The AMF in the MWAB Broadcasted PLMN subscribes the RAT Type change of MWAB-UE event from the serving AMF of MWAB-UE in BH PLMN directly or via NEF.

-	When the serving AMF of MWAB-UE detects the change of RAT Type of MWAB-UE, it sends event report including the RAT Type to the AMF in the MWAB Broadcasted PLMN directly or via NEF.

4. Support of Notification Control

4.1	Existing Mechanisms

The existing notification control mechanisms are as follows:

-	Notification control without alternative QoS profiles: based on the status of radio network (e.g. measurements like queuing delay or system load), when NG-RAN determines GFBR can no longer be guaranteed, the NG-RAN notifies such information to SMF.

-	Notification control with alternative QoS profiles: The SMF provides a prioritized list of alternative QoS profile(s) to NG-RAN. When the NG-RAN determines the GFBR can no longer be guaranteed, the NG-RAN also notifies the SMF with the alternative QoS profile can be fulfilled or the lowest alternative QoS profile cannot be fulfilled.

4.2	Gap analyse

In the existing notification control mechanism, when the NG-RAN determines GFBR can no longer be guaranteed, alternative QoS profile can be fulfilled or the lowest alternative QoS profile cannot be fulfilled, it only considers the status of radio resource (e.g. measurements like queuing delay or system load).

However, in the case of MWAB, besides the radio resource status of MWAB-gNB, the connection quality of the serving mobile base station relay is also impacted by the radio resource status of serving gNB of MWAB-UE. For example, the radio resource of MWAB-gNB can guarantee the GFBR of the QoS flow of UE connecting to MWAB-gNB, however caused by radio resource limitation of serving gNB of MWAB-UE, the GFBR above cannot be guaranteed. In this case, the alternative QoS does not work, because the MWAB-gNB cannot determine which alternative QoS profile can be supported or whether the lowest alternative QoS profile can be fulfilled or not, as it has no idea of the radio resource status of serving gNB of MWAB-UE.

4.3	Solution

When the MWAB-gNB determines that the PDB or the PER of the QoS profile cannot be fulfilled which is not caused by the radio resource status of the MWAB-gNB. To support the notification control, the following solution is proposed:

-	Notification control without alternative QoS profiles: the MWAB-gNB notifies the GFBR can no longer be guaranteed to serving SMF of UE.

-	Notification control with alternative QoS profiles:

-	The MWAB-gNB triggers the MWAB-UE to initiate the PDU Session Modification procedure by providing the prioritized list of alternative QoS profile(s).

-	The MWAB-UE sends the PDU Session Modification request including the prioritized list of alternative QoS profile(s) to the serving SMF in BH PLMN.

-	The SMF sends the prioritized list of alternative QoS profile(s) to the serving gNB in BH PLMN. The serving gNB accepts one of the alternative QoS profile or reject the request if the lowest alternative QoS profile cannot be fulfilled.

-	The SMF sends the PDU Session Modification response including the accepts one of the alternative QoS profile or rejection cause to MWAB-UE.

-	The MWAB-UE sends the accepts one of the alternative QoS profile or rejection cause to MWAB-gNB. The MWAB-gNB notifies the alternative QoS profile can be supported or the lowest alternative QoS profile cannot be fulfilled to the serving SMF of UE.

5. Summary

Based on discussion in clause 3 and clause 4, the proposals are as follows.

When the MWAB mobility between terrestrial access network and satellite access network happens, to support the satellite backhaul reporting and QoS monitoring when dynamic satellite backhaul is used, the following solution is proposed

-	MWAB-UE provides the identifier (i.e. GPSI) to MWAB-gNB.

-	MWAB-gNB provides the identifier of the MWAB-UE via N2 message to the AMF in the MWAB Broadcasted PLMN.

-	The AMF in the MWAB Broadcasted PLMN subscribes the RAT Type change of MWAB-UE event from the serving AMF of MWAB-UE in BH PLMN directly or via NEF.

-	When the serving AMF of MWAB-UE detects the change of RAT Type of MWAB-UE, it sends event report including the RAT Type to the AMF in the MWAB Broadcasted PLMN directly or via NEF.

When the MWAB-gNB determines that the PDB or the PER of the QoS profile cannot be fulfilled which is not caused by the radio resource status of the MWAB-gNB. To support the notification control, the following solution is proposed:

-	Notification control without alternative QoS profiles: the MWAB-gNB notifies the GFBR can no longer be guaranteed to serving SMF of UE.

-	Notification control with alternative QoS profiles:

-	The MWAB-gNB triggers the MWAB-UE to initiate the PDU Session Modification procedure by providing the prioritized list of alternative QoS profile(s).

-	The MWAB-UE sends the PDU Session Modification request including the prioritized list of alternative QoS profile(s) to the serving SMF in BH PLMN.

-	The SMF sends the prioritized list of alternative QoS profile(s) to the serving gNB in BH PLMN. The serving gNB accepts one of the alternative QoS profile or reject the request if the lowest alternative QoS profile cannot be fulfilled.

-	The SMF sends the PDU Session Modification response including the accepts one of the alternative QoS profile or rejection cause to MWAB-UE.

-	The MWAB-UE sends the accepts one of the alternative QoS profile or rejection cause to MWAB-gNB. The MWAB-gNB notifies the alternative QoS profile can be supported or the lowest alternative QoS profile cannot be fulfilled to the serving SMF of UE.