3GPP TSG-SA2 Meeting #169	S2-2505403

# Fukuoka, Japan, May 19 – 23, 2025          		          		           		(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR |  | rev |  | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS Enhancement for MWAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20, DUMMY |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA1 WID on Vehicle-Mounted Relays Phase 3 (SP-241821) is approved in SA#106 meeting. And the WID captures the following service requirements in TS 22.261: Subject to regulatory requirements and operator policy, the 5G network shall support a mechanism to determine suitable QoS parameters for traffic relayed via a mobile base station when the connection quality of the serving mobile base station relay changes, e.g. during mobility between terrestrial access network and satellite access network. This CR is proposed to support the requirement from the following aspects: - To support the MWAB-UE RAT type change event report when the MWAB moves between terrestrial access network and satellite access network; - To support notification control when the PDB or the PER of GBR QoS flow cannot be fulfilled caused by the resource limitation in serving gNB of MWAB-UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support the SA1 requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unsupported SA1 requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.x, 5.49.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change (all text new) * * *

5.49.x	Support of MWAB-UE RAT Type change event report

The MWAB-UE RAT Type represents the backhaul category of MWAB-gNB, when the MWAB moves between satellite and terrestrial network, the backhaul network of the MWAB-gNB also changes accordingly.

To support reporting of satellite backhaul to SMF specified in clause 5.43.4 and the QoS monitoring when dynamic satellite backhaul is used specified in clause 5.43.5, the AMF in MWAB Broadcasted PLMN subscribes the RAT Type change event report from the serving AMF of MWAB-UE in BH PLMN directly or via NEF by providing the identifier of MWAB-UE (GPSI) which is received from MWAB-gNB. The serving AMF of MWAB-UE determines the RAT Type as specified in clause 5.3.2.3. When the RAT Type change is detected, the serving AMF of MWAB-UE notifies the event reoport to the AMF in MWAB Broadcasted PLMN.

* * * Second Change * * *

5.49.y	Support of notification control

For a GBR QoS flow with notification control activated, the QoS profile cannot be fulfilled may be caused by the radio resource limitation in MWAB-gNB or in gNB connected by MWAB-UE. In the latter case, the following applies:

-	For notification control without alternative QoS profiles, the MWAB-gNB notifies the GFBR can no longer be guaranteed to serving SMF of UE.

-	For notification control with alternative QoS profiles, the MWAB-gNB provides the QoS profile of the QoS flow and prioritized list of alternative QoS profile(s) to MWAB-UE which initiates the PDU Session Modification procedure by providing the profile(s) to the BH network. During the procedure, the gNB connected by MWAB-UE determines to accept one of the alternative QoS profile or reject the request if the lowest alternative QoS profile cannot be fulfilled which is provided to the MWAB-UE. The MWAB-UE provides the information to the MWAB-gNB which is responsible to notify the alternative QoS profile can be supported or the lowest alternative QoS profile cannot be fulfilled to the serving SMF of UE

* * * End of Changes * * *