3GPP TSG-SA2 Meeting #169	S2-2505428

Fukuoka, Japan, May 19th – 23rd 2025

Source:	InterDigital Inc.

Title:	Key Issue #3 New Solution: UP path adjustment based on energy consumption information

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This paper proposes a solution for Key Issue #3.

# 1. Introduction

This paper proposes a new solution for Key Issue #3 - NF (re)selection and/or UP path adjustment based on energy related information.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-67 V0.1.0.

* * * * First Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| Y |  |  | Y |

* * * * Second Change * * * *

## 6.y	Solution #y: UP path adjustment based on energy consumption information

### 6.y.0	High-level solution principles

The PCF may be configured with network energy savings behaviours. When an AF influence on traffic routing is ongoing between an AF and the 5GC for a certain UE traffic, the PCF may request energy consumption information related to the UE’s PDU Session, application session, or traffic flow(s). The PCF may use the obtained energy consumption information to determine if certain thresholds related to energy are crossed. If thresholds are crossed, the PCF may adjust UE’s UP path, e.g., by updating routing policies for the UE’s PDU Session, application session, or traffic flow(s), for example update the DNAI list. The PCF sends the updated policies to the SMF. The SMF may be triggered to reconfigure routing information for the UE’s traffic (e.g., determining a new target DNAI, perform PSA UPF relocation, etc.)

### 6.y.1	Description

The solution applies to Key Issue #3 " NF (re)selection and/or UP path adjustment based on energy related information".

In this solution, it is proposed that the PCF is configured with energy consumption related thresholds associated with certain routes, e.g., DNAI. After a successful AF traffic routing influence procedure is performed between an AF and the 5GC, the PCF is triggered to request energy consumption information from the EIF. The energy consumption information from the EIF is related to the UE’s PDU Session, or application session, or traffic flow(s). The PCF then uses the obtained energy consumption information to determine if the DNAI is optimal based on threshold which are part of local policies. If the PCF determines that DNAI needs to be updated, it sends then new the DNAI to the SMF based on the existing procedure. The PCF then sends the updated rules to the SMF. The SMF may be triggered to reconfigure traffic routing information for the UE or perform PSA UPF relocation accordingly.

### 6.y.2	Procedures

Figure 6.X.3-1 below shows an AF session with required QoS (clause 4.15.6.6 of TS 23.502 [3]) with enhancements to support energy savings criteria for the session. The changes that are proposed by this solution are described below.

Figure 6.y.2-1: Procedure UP path adjustment based on Energy consumption information

- An AF influence on traffic routing procedure is successfully performed as described in clause 4.3.6.2 of TS 23.502 [3].

2a.	The PCF sends to the EIF a request to retrieve energy consumption information using the Neif_EventExposure_Subcribe service. The PCF includes PCF ID, Energy Consumption as Event ID, and parameters reflecting the granularity request by the PCF (e.g., DNN/S-NSSAI, Application ID, Flow description(s)). The PCF includes reporting parameters such as reporting threshold/period.

The NEF checks authorization for the AF request and may apply policies to verify that that feature based energy savings control can be authorized for the AF and the features allowed to be under energy related requirement, e.g. savings control, required energy efficiency. If the authorization is not granted, the NEF rejects the request otherwise proceeds with the following steps.

Note: The granularity requested by PCF to receive energy information from EIF is up to PCF implementation.

2b.	The PCF receives energy consumption information from the EIF for the requested granularity (e.g., per PDU Session, per application ID, per traffic flow).

3.	The PCF compares the energy consumption to a certain threshold (e.g., configured at the PCF), and determines if the DNAI is optimal based on threshold which are part of local policies.

4.	The PCF determines new policies related to the traffic flow or PDU Session. For example, the PCF updates the list of DNAI(s) in the traffic routing information of the traffic flow(s).

5a	The PCF sends a Npcf_SMPolicyControl_UpdateNotify request to the SMF with updated policy information about the PDU Session or traffic flow(s) (e.g., updated DNAI list).

5b.	The SMF acknowledges the PCF request.

6.	Using the updated policy information about the PDU Session or the traffic flow(s), the SMF may adjust the UP path the AF session. For example, the SMF may determine a new target DNAI for the PDU Session, update the UPF in the target DNAI, relocate PSA UPF.

### 6.y.3	Impacts on services, entities and interfaces

PCF:

-	Request energy consumption information from the EIF for UE’s PDU Session, an application ID or traffic flow(s).

-	Receive energy consumption information for the session and individual feature relative contribution to the total energy consumption cost.

- 	Update routing policies for the UE based on received energy consumption information.

* * * * End of Changes * * * *