3GPP TSG-WG SA2 Meeting #169	S2-2505429

Fukuoka, Japan, 19 – 23 May 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Authorization and Parameter Configuration of ISAC

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Authorization and Parameter Configuration of ISAC are proposed.

# 1. Introduction/Discussion

This solution proposes a solution to KI#2 and KI#6, for Sensing Entities’ Authorization and Parameter Configuration

Since there are radio resources and related data communication needed for the ISAC session. The sensing entities should have authorization.

For the ISAC parameters, most of they are supposed to be defined by RAN WGs. This solution only address some principles for either UE or RAN node as sensing entities.

FIRST CHANGE

## 6.0	Mapping of solutions to key issues

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| x |  | X |  |  |  | X |

Second CHANGE (all new)

# 6.x	Authorization and Parameters of ISAC

## 6.x.1	General Concepts

ISAC receives input from its requestor(s) and provides output to the consumer(s). This solution provides the authorization and parameters configuration for Sensing Entities.

## 6.x.2	Authorization of Sensing Entities

In 5GS, the parameters for ISAC service may be made available to the sensing entities in following ways:

-	pre-provisioned in the sensing entities; or

-	provided by Sensing Function (SF); or

-	provided or updated by the ISAC Application Server via PCF; or

-	provided or updated by the PCF.

The sensing entities shall apply any given parameter in the following priority order:

-	provided or updated by the Sensing Function, if available, otherwise;

-	provided or updated by the PCF, if available, otherwise;

-	provided or updated by the ISAC Application Server, if available, otherwise;

-	pre-configured in the sensing entities.

The authorization of sensing entities shall contain following options:

-	sensing entities authorized as sensing transmitters

-	sensing entities authorized as sensing receivers

-	sensing entities authorized as both sensing transmitters and receivers

## 6.x.3	Parameter Configurations of Sensing Entities

For the transportation of sensing data, its QoS parameters are generated by the PCF.

Sensing entities should provide "ISAC capability" to the core network, with following options:

-	UE, if involved, its capabilities are included in the registration phase and stored as part of UE context in AMF

-	RAN node, if involved, its capabilities are included in the capability exchange procedure.

Editors’ Note: Details of ISAC capabilities and the exchange procedures depend on RAN WGs

### 6.X.4	Impacts on services, entities and interfaces

Editors’ Note: The impacts on the core network functions and sensing entities are FFS

END OF CHANGES