3GPP TSG-SA2 Meeting #169	S2-2505431

Fukuoka, JP, 19th May 2025 – 23rd May 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Solutio to KI #1 New Sol ISAC System Architecture

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: The contribution proposes System Architecture to Support Sensing for Key Issue #1.

# Introduction

This contribution proposes system architecture and function enhancements required to provide end-to-end support for sensing.

# Discussion

Several applications could be supported using ISAC technologies. SA1, in Release 19, has studied thirty-two (32) use cases, while several functional and performance requirements have been identified in 3GPP TS 22.137. The considered use cases are grouped into the following three service types:

- Object Detection and Tracking: detect the presence of one or more target objects, providing information about the location, the trajectory, and the velocity of the respective object(s).

- Environment Monitoring: identify the characteristics of the environment and detect changes

- Human Motion Monitoring: Human healthcare assistance is another service that can be supported by ISAC technologies

The sensing architecture should support the requirements that have been defined in SA1 and should also be designed so as to be able to support all sensing modes.

In addition, the sensing services will be realized in various environments (e.g., public environments such as roads, indoor areas such as industrial premises), sensing different types of objects and associated information (e.g., vehicles, drones, humans etc.) that may be privacy sensitive. Hence, privacy is a key feature that should be taken into account.

# Proposal

A solution is proposed for KI#1 for incorporation in the TR 23.700-14.

FIRST CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #X | X |  |  |  |  |  |

SECOND CHANGE (all new)

## 6.X	Solution #X: ISAC System Architecture

### 6.X.1	Key Issue mapping

This solution aims to resolve Key Issue #1, " System Architecture to Support Sensing".

### 6.X.2	Description

Figure 6.x.2-1: High level core network view for supporting ISAC

The integration of sensing services into a communication system necessitates the introduction of a Sensing Function (SF), which determines and coordinates a sensing procedure, according to the sensing requirements, the network conditions, the capabilities of involved sensing entities (i.e. the sensing Tx and Rx) and the environment where he sensing takes place. The SF can be deployed as a dedicated Network Function (NF), since it is enabling a new functionality. Figure 6.x.2-1 shows a very high level view of the positioning of the SF in the network.

The sensing service consumer can be a third party (e.g., an external Application Function (AF) such as a factory digital twin, or an NF. A trusted AF transmits to the SF a sensing service request and receives from the SF a sensing service output.

In the case of untrusted AF the interaction is realized via the Network Exposure Function (NEF). The NEF receives a sensing service request from an (untrusted) AF sensing request and forwards to the selected SF, including in the request message the attributes received from the AF request and if needed converting certain received parameters into 5GC internal parameters (e.g. target sensing area). The NEF selects the SF that can serve a specific sensing service request, according to the requested sensing area and/or the sensing service request requirements. The Network Repository Function (NRF) stores the profile of each SF, indicating the supported sensing capabilities and features (e.g., supported sensing area, supported sensing services).

In a cellular ISAC network, the SF can receive multiple and heterogeneous sensing requests that indicate the type of the requested sensing service, the sensing area, the sensing Quality of Service (QoS) requirements, optionally information about the objects to be sensed etc. Each request may require its own configuration and different sensing modes can be used for a single sensing service. The SF determines the set of sensing operations to be performed,  discovers and selects  the sensing entities to be involved in a sensing session, according to the requirements in the sensing request and the available sensing capabilities in the sensing area of interest. For instance, different parts of the sensing area can be assigned to the different sensing entities (e.g., BSs or UEs). SF is then responsible for configuring the sensing entities by transmiting sensing session configuration to the involved sensing entities, starting the sensing operations, collecting the measurement information, calculate the final result and send the result to the corresponding sensing service consumer.

The processing of sensing data is expected to be an intensive periodic task, where sensing data from multiple sensing entities will be collected. And in most of the cases the sensing entities that need to serve a specific sensing service will be located in the same geographical area. The separation of the sensing control and processing logic may be beneficial for scalability purposes, the faster generation of sensing outputs and the more localised sensing data processing.

Hence, the SF can consist of two logical entities which may be collocated or placed at different physical entities: a) the Sensing Control Function (SCF) and b) the Sensing Processing Function (SPF).

-	The SCF is responsible for the reception of sensing requests as well as the control and configuration of sensing operations.

-	The SPF is responsible for collection and processing of sensing data received from one or more sensing entities (e.g., BS, UE) to generate the required sensing outputs.

Editor’s Note: whether the interface between the SCF and SPF is standardized is FFS.

### 6.X.3	Architecture reference model

#### 6.X.3.1	Reference point based representation of ISAC Architecture

Figure 6.x.3.1-1: Reference architecture for ISAC in reference point representation

Figure 6.x.3.1-1 above shows an architectural reference model for ISAC in reference point representation.

NOTE: the NSx interface is realised over the N2 and the NSy interface is realised over the N1

Editor’s Note: the transmission of sensing data from the sensing entities using the user plane interface is for FFS.

#### 6.X.3.2	Service based representation of ISAC Architecture

Figure 6.x.3.2-1: Reference architecture for ISAC in SBI representation

Figure 6.x.3.1-1 above shows an architectural reference model for 5GS ISAC in SBI representation.

The functionality of existing and new NFs is summarized as follows:

AF:

- A trusted AF transmits to the SF a sensing service request and receives from a SF a sensing service output, In the case of untrusted AF the NEF is involved.

NEF:

- receives a sensing service request from an (untrusted) AF sensing request and forwards to the selected SF, including in the request message the attributes received from the AF request and if needed converting certain received parameters into 5GC internal parameters (e.g. target sensing area)

- selects the SF that can serve a specific sensing service request, according to the requested sensing area and/or the sensing service request requirements.

NRF:

- stores the profile of each SF, indicating the supported sensing capabilities and features (e.g., supported sensing area, supported sensing services)

For scalability, for faster generation of sensing outputs and the more localised sensing data processing a separation of sensing control (SCF) and sensing data processing (SPF) logic can be assumed:

SCF:

-	receives sensing requests from a sensing consumer and provides the sensing outputs to the sensing consumer.

-	conducts authorization and privacy-related checks for each sensing service request

-	discovers and selects the sensing entities to be involved in the sensing session.

-	configures and control the sensing sessions that serve a specific sensing service request , according to the requirements received in the sensing service request, and the available sensing capabilities in the target sensing area.

-	transmit sensing session configuration to the involved sensing entities.

SPF:

-	collects and  processes the sensing data from sensing entities to generate the required sensing outputs.

### 6.X.4	Impacts on services, entities and interfaces

Editor’s Note:	Details of this clause is FFS

END OF CHANGES