SA WG2 Meeting ##169	S2-2505440

Fukuoka, Japan, 19th May 2025 – 23rd May 2025

# Title:	[Draft] Reply LS on Questions on stage 2 requirements for AIML_CN

# Response to:	LS on Questions on stage 2 requirements for AIML_CN (S2-2504575/ C4-250630)
LS on Clarification on SCP data collection (S2-2504509/ C4-251477)

# Release:	Rel-19

# Work Item:	AIML_CN

Source:	SA2

To:	CT4

Cc:	CT3

Contact Person:

# Name:	Dr. Thomas Belling

E-mail Address:	 thomas (dot) belling (at) nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR 1465 for TS 23.288, CR 5483 for TS 23.502

1. Overall Description:

SA2 thanks CT4 for their liaison statements. SA2 has agreed the attached CRs to address the comments of CT4. SA2 would also like to answer the questions raised by CT4 as follows:

A. LS C4-250630

1) Table 6.22.2-5 (Data collection from NRF/OAM) in TS 23.288 contains the following Note: " NOTE: If this information is not available in the OAM, it can be obtained from the NRF if available." CT4 observes that the above Note would result in overlapping functionalities in OAM and NRF APIs. Q1: Given that the metrics can be reported by OAM APIs, is it necessary for the NRF APIs to also support reporting the same metrics, and if so, can SA2 explain why?

SA2 Answer A1:

SA2 agrees that all the information listed in table 6.22.2-5 is available via OAM, see clause 5.10 in TS 28.552.  SA2 thus removed the NOTE.

2) if the answer to Q1 is yes, TS 23.502 suggests using the Nnrf_NFManagement_NFStatusSubscribe service operation for subscribing and reporting the aforementioned metrics: "- NF load status information, capacity and priority information of NFs and NF Services, etc as described in Table 6.x.2-2 and Table 6.x.2-5 of TS 23.288 [50]." The service operation description states: " Description: Consumer can subscribe to be notified of the following: - Newly registered NF along with its NF services. - Updated NF profile. - Deregistered NF." Q2: If the answer to Q1 is yes, this required functionality does not fit with the current scope of the Nnrf_NFManagement_NFStatusSubscribe service operation. CT4 kindly asks SA2 to clarify which service operation is suitable for this purpose?

SA2 Answer A2: SA2 agrees that the intention of the Nnrf_NFManagementStatusSubscribe service operation is a subscription for information available in NF profiles, but the information listed in table 6.22.2-5 is not part of any NF profile. SA2 thus removed the reference to 6.22.2-5 from the quoted bullet.

3) Table 6.22.2-1 (UE related Context Data collection) of TS 23.288 specifies: An NF instance may comprise multiple NF service instances supporting the same and/or different service names, e.g. for different network slices, possibly deployed in different Data Centers. Q3: Accordingly, should NF metrics related to signalling storm information be reported per Service Name (i.e. with the NF instance aggregating metrics of all NF service instances supporting the same service name) or per NF service instance?

SA2 Answer A3: SA2 clarified that Request type designates a NF service instance.

4) Clause 5.2.29.2.1 of TS 23.502 specifies: " Abnormal Signalling characteristics: An NF, NF set, Service Type, may be provided as target. A reporting threshold (in terms of absolute transaction or deviation from average, or in terms of a minimum percentage failed transactions) " Q4: Does “Service Type” refer to “service name”? If the answer is yes, should SCP metrics related to signalling storm information be reported per "Service Name" or "Service instance"? If the answer is no, what does "Service Type" refer to?

SA2 Answer A4: SA2 clarified in Clause 5.2.29.2.1 of TS 23.502 that the subscription can be per NF service name or per NF service instance and that the reported “Request type” designates a NF service instance.

B. LS C4-251477

TS 23.502 clause 5.2.29.2 defines the event(s) supported by Nscp_EventExposure. In this clause one event is defined, which is "Service Signalling characteristics". Furthermore, TS 23.288 clause 6.22.2 defines the list of information which may be reported to NWDAF as the consumer via SCP Event Exposure service. CT4 would appreciate SA2 insights on a few questions related to data collection from SCP:

Q1) Regarding the "UE related Context Data collection" specified in Table 6.22.2-1 of TS 23.288:

Requiring the SCP to provide information with UE granularity means that the NFs in the network would be required to be configured to use the custom header 3gpp-Sbi-Correlation-Info which is optional to support and may be subject to security/regulatory requirements (see TS 29.500). Furthermore, different NFs may use different UE identities, e.g. SUPI, GPSI, etc, for the same UE within this header Also the NFs of earlier releases that do not support this header do not provide UE information to the SCP, which would impact the accuracy of the statistics.

Considering these limitations, CT4 would like to ask SA2 to confirm whether the information with UE granularity is required to be obtained by the SCP for the event exposure purpose.

SA2 Answer B1:  SA2 agrees to remove requirements related to per-UE reporting from the SCP. An SCP can collect information on a per NF instance or NF service instance level. Related IEs have thus been transferred to a new Table 6.22.2-X,

Q2) According to TS 23.288 clause 6.22.2, the SCP is required to distinguish between different types of communications and signallings and provide information, for example, to collect the data of "Number of redundant signalling of NF" and "A posterior Request type of NF (0..max)" in Table 6.22.2-1, and "SCP Signalling statistics" in Table 6.22.2-3 which are depicted bellow. Currently there is no mechanism to enable SCP to differentiate between different types of communication SA2 refers to, e.g. whether the signalling is redundant or an "A posterior Request".

Therefore, CT4 would like to ask SA2 to consider reverting the requirements on "A posterior Request type of NF (0..max)" and "Number of redundant signalling of NF", unless SA2 could provide clarification on how these requirements can be implemented.

SA2 Answer B2:  SA2 agrees that this information cannot be observed by an SCP. The intention of Note 3 is rather to clarify that an SCP can perform retransmissions. Note 3 was clarified accordingly.

Q3) CT4 would also appreciate SA2 to clarify what "different types of signalling" corresponds to for "SCP Signalling statistics".

SA2 Answer B3:  The “different types of signalling” relate to addressed service. The wording in the table has been clarified accordingly.

Q4) "Heart-beat related information", "NF load status information", "Capacity and priority information" which are listed in TS 23.288 Table 6.22.2-2, may be provided by NRF. If required, SCP may get this information from NRF. Therefore, CT4 would like to ask for clarification on why the SCP is also required to provide this information.

SA2 Answer B4:  SA2 agrees to remove requirements to expose heart-beat related information from the SCP

2. Actions:

To CT4 group.

ACTION: 	SA2 kindly asks CT4 to take the provided answers into account.

# 3. Date of Next SA2 Meetings:

TSG SA WG2 Meeting #170	25th August – 29th August 2025	Goteborg, Sweden

TSG SA WG2 Meeting #171	13th October – 17th October 2025	Wuhan, China