3GPP TSG-WG SA2 Meeting #169	S2-2505519

Fukuoka, JP, 19th May – 23rd May, 2025	(revision of S2-2504764)

Source:	Huawei, HiSilicon, ZTE, SK Telecom

Title:	KI#2: New solution for AIML assisted user plane performance enhancement for normal traffic

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: It proposes a new solution of KI#2 on UC#2.

1. Introduction/Discussion

In Rel-20 FS_AIML_CN_Ph2 study item, it is proposed to study the analytics on user plane traffic patterns and how to use the analytics to improve user plane performance as shown below:

Key Issue #x: AI/ML-assisted user plane traffic pattern and behaviour analysis to support efficient performance of User Plane

- How to provide analytics (e.g., enhancing existing in TS 23.288 or introducing new) including the aspects of required inputs/outputs to improve performance of user plan.

- Which consumers can subscribe/request the analytics and how the consumers use the analytics to take actions to improve user plane performance.

- Whether and how to train ML models and perform the inference for the user plane analytics, considering solutions that minimize the load on Control Plane NFs.

It is beneficial for the network to know about the traffic pattern in advance to optimize user plane performance, but current mechanisms are hysteresis due to the dependent on transport layer and only applicable for special protocols (e.g. TSN). Considering the analytic capability of NWDAF, a new mechanism could be introduced to give Analytic output of traffic pattern for 5GC to enhance user plane performance.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-04.

# * * * * First change (All new text) * * * *

6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | x |  |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x |  |  |  |
| #X |  | x |  | x |

# * * * * Second change (All new text) * * * *

6.X	Solution X: NWDAF assisted traffic pattern generation for user plane performance optimization

6.X.1	High level principles

The high level principles of the solution are:

- The PCF subscribes and retrieves traffic pattern related analytics from the NWDAF and creates/updates PCC rule based on the analytic result.

- The NWDAF provides Analytic output of traffic pattern for target application(s) based on QoS monitoring result from UPF.

6.X.2	Description

This solution resolves Key Issue #X about how to analyse traffic pattern and update the QoS control of target application(s) with the assistance of NWDAF. This solution enhances the NWDAF to analysis traffic pattern based on QoS monitoring result and the PCF retrieves the traffic pattern for QoS control.

Based on target application(s) in the PDU Session policy control subscription information or by configuration, which requires NWDAF assisted traffic pattern generation, the PCF subscribes traffic pattern related analytic from NWDAF and updates PCC rule based on the traffic pattern prediction or statistics. The NWDAF can predict the bitrate related requirement of the application according to network status. For example, the bitrate will decrease based on high packet delay which may cause down speed of TCP sender. The NWDAF subscribes to SMF of the QoS monitoring result for traffic pattern analytic. Based on the Analytic output of traffic pattern from NWDAF, PCF could create/update PCC rule for target application(s) (e.g. mapping maximum burst size and maximum flow bitrate in the traffic pattern to MDBV and MBR as described in clause 5.27.3 of TS 23.501[2]).

6.X.3	Procedures

The procedure is described in Figure 6.X.3-1.

UPF

SMF

PCF

NWDAF

7. Nnwdaf_AnalyticsSubscription_Subscribe (analytics ID = user plane traffic pattern, SUPI, DNN, S-NSSAI )

RAN

UE

8. Nsmf_EventExposure_Subscribe (Event ID = QoS Monitoring)

9. N4 session update (SRR with notification target address = NWDAF address)

10. N2 signal (QoS Monitoring request)

11. Nupf_EventExposure_Notify (QoS Monitoring result)

12. Nnwdaf_AnalyticsSubscription_Notify (analytics ID = user plane traffic pattern, traffic pattern of the target application)

13. update PCC rule considering the traffic pattern

14. QoS rule, QoS profile, QER update

UDR

1. Existing PDU session establishment procedure

3. Npcf_SMPolicyControl_Create response (set Start/Stop of application traffic detection PCRT for target application)

4. N4session update (reporting trigger: start/stop of the target application traffic)

5. N4session update (report for start/stop of the target application traffic)

6. Npcf_SMPolicyControl_Update request (report for start/stop of the target application traffic)

2. Nudr_DM_Notify (PDU Session policy control subscription information including target application requiring NWDAF assisted traffic pattern generation)

Figure 6.X.3-1: procedure for NWDAF assisted traffic pattern generation

1.	UE PDU session is established via existing PDU session establishment procedure.

2-3. The PCF retrieves the PDU Session policy control subscription information from the UDR and may get the target application(s) identified by Application ID(s) which requires NWDAF assisted traffic pattern generation. If so, the PCF provides PCC rule(s) for these Application ID(s) (e.g., with the same parameters as used for the match-all PCC rule, apart from the precedence) and sets Start/Stop of application traffic detection PCRT and QoS Monitoring PCRT as defined in clause 6.1.3.5 of TS 23.503 [4] in the SMF. As an alternative, PCF may make such PCC decision based on configuration.

4.	Based on these PCC rule(s) received and the PCRT from PCF, SMF configures UPF to report the start of traffic from target application(s) as described in clause 6.2.2.2 of TS 23.503 [4].

7.	when the PCF is notified about the start of target application(s) detection, PCF subscribes to NWDAF for the user plane traffic pattern of the target application(s). The PCF also provides DNN, S-NSSAI of the PDU session and SUPI to the NWDAF for SMF discovery.

8.	After subscribed by PCF, the NWDAF discovers the SMF serving the PDU session as described in clause 6.2.8 of TS 23.288 [5]. The NWDAF subscribes to the SMF for QoS monitoring result of the target application(s) (e.g. packet delay, data rate, congestion information). The input for the analytic is described in Table 6.X.3-1.

9-10.	SMF starts QoS monitoring as described in clause 5.45 of TS 23.501 [2].

11.	The UPF notifies to NWDAF the QoS monitoring result as described in clause 5.8.2.18 of TS 23.501 [2].

12.	NWDAF provides the Analytic output as described in Table 6.X.3-2 of user plane traffic pattern for the target application(s) based on QoS monitoring result and notifies to PCF.

13.	PCF creates/updates PCC rule for target application(s) based on the traffic pattern provided from NWDAF (e.g. mapping maximum burst size and maximum bitrate in the traffic pattern to MDBV and MBR as described in clause 5.27.3 of TS 23.501 [2]).

14.	SMF updates QoS rule, QoS profile and QER on UE, RAN, UPF respectively based on the updated PCC rule via existing QoS framework.

Table 6.X.3-1: Input data from UPF related to user plane traffic pattern analytic

| Information | Source | Description |
| --- | --- | --- |
| Packet Filter Set | UPF | Identifies the traffic flow of the UE communicating with target application(s), as defined in clause 5.7.6 of TS 23.501[2]. |
| UL/DL/roundtrip packet delay | UPF | UL/DL/roundtrip packet delay generated by QoS monitoring as defined in clause 5.45 of TS 23.501[2]. |
| Congestion information | UPF | congestion information (i.e. a percentage of congestion level) generated by QoS monitoring as defined in clause 5.45 of TS 23.501[2]. |
| UL/DL data rate | UPF | UL/DL data rate generated by QoS monitoring as defined in clause 5.45 of TS 23.501[2]. |
| Reporting period | UPF | Measurement period of the QoS monitoring as defined in clause 5.8.2.18 of TS 23.501 [2]. This can be used to provide burst analysis. |
| Packet Interval | UPF | (Optional) The interval value (e.g., minimum, maximum, average and/or variance value) between packets with the traffic flow. This can be used to provide periodicity and burst analysis. |

Table 6.X.3-2: Output data for user plane traffic pattern prediction

| Information | Description |
| --- | --- |
| Packet Filter Set | Identified the service data flow of the target application(s), as defined in clause 5.7.6 of TS 23.501[2]). |
| Traffic pattern | Traffic pattern of the target application(s), as defined in of TS 23.503 [4] and TS 23.501 [2]. |
| > Maximum Bitrate | Requested Maximum Bitrate (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| > Guaranteed Bitrate | Requested Guaranteed Bitrate (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| > Maximum Burst Size | Maximum Burst Size as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| > 5GS delay | Requested 5GS delay (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| > Packet Error Rate | Requested Packet Error Rate (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| > Periodicity | (Optional) Periodicity as defined in clause 5.27 of TS 23.501 [2] of the traffic for target application(s) if applicable. |
| Start time and validity period of the traffic pattern | start time and validity period of the analysed traffic pattern. |

### 6.X.4	Impacts on services, entities and interfaces

PCF:

-	Subscribes to NWDAF for the Analytic output of traffic pattern for target application(s) requiring NWDAF assisted traffic pattern generation.

-	Updates PCC rule based on the traffic pattern provided from NWDAF as described in 6.X.2.

NWDAF:

-	Provides Analytic output of traffic pattern based on QoS monitoring result.

UDR:

-  Stores the target application(s) requiring NWDAF assisted traffic pattern generation in the PDU Session policy   control subscription information and provides to PCF.

UPF:

-  (Optional) Report Packet Interval to the NWDAF.

# * * * * End of changes * * * *