SA 2WG2 Meeting #169	S2-2505520

# May 19 – 23, 2025, Fukuoka, Japan               	   (Revision of S2-2504817)

Source: 	NTT DOCOMO

Title: 	KI#2: Solution for user plane analytics via efficient UP data collection

Document for: 	Approval

Agenda Item: 	20.3.1 (FS_AIML_CN_Ph2)

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a Solution for the user plane analytics in the context of KI#2 of FS_AIML_CN_Ph2.

# 1	Discussion

According to the KI#2 of the FS_AIML_CN_Ph2 (TR 23.700-04), the following aspects need to be studied.

-	How to provide analytics (e.g. enhancing existing in TS 23.288 [5] or introducing new) including the aspects of required inputs/outputs to improve performance of user plan.

-	Which consumers can subscribe/request the analytics and how the consumers use the analytics to take actions to improve user plane performance.

-	Whether and how to train ML models and perform the inference for the user plane analytics, considering solutions that minimize the load on Control Plane NFs.

This paper proposes a solution for user-plan analytics based on data collection from the UPF that minimizes the load on CP NFs (i.e. the overhead of control plane signalling for user plane data collection from the UPFs).

# 2 Proposal

It is proposed to update the solution for TR 23.700-04 according to the following text.

# 6	Solutions

********** First Change **********

## 6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | x |  |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x |  |  |  |
| #x |  | x |  |  |

********** Second Change (all new text) **********

## 6.X	Solution #X: User plane analytics derivation with efficient UP data collection

### 6.X.1	High level principles

The high-level principles of the proposed solution are as follows:

- Training ML models and then performing ML model inference to monitor user plan traffic (e.g., to detect burstiness or traffic abnormalities) need huge amount of packet-level information from the user plane. Collecting and transferring the information from UPF(s) to the NWDAF may cause significant control plane signalling overhead.

- The existing mechanisms such as Processing & Formatting Instructions or bulk data transfer is not applicable when UPF is the data source. Accordingly, the main idea of the proposed solution is to extend the UPF event exposure services in a similar way that instead of sending raw UP information from the UPF to the NWDAF, the UPF may preprocess the data (in a manner which facilitates the ML model training/inference in the NWDAF) and sends the result of the preprocessing to reduce the signalling overhead.

- In this solution, to derive UP related analytics using UP data by the NWDAF, the NWDAF sends a data collection request to the UPF, which also may include data preprocessing instructions that specify how the information required for model training/inference should be obtained from the raw (packet-level) data in the UPF.

- The UPF performs the local data processing and reports the locally preprocessed data to the NWDAF based on the received instructions.

### 6.X.2	Description

This solution addresses the key issue#2.

In this release, NWDAF can derive analytics to optimize user plane (UP) performance using information provided by UPF such as UP traffic pattern, traffic information and application traffic measurement information. However, the detailed packet-level raw data of UP traffic for ML model training/inference is huge, so, as the key issue also described, a solution that minimizes the load on control plane (CP) NFs is required. To address this aspect of the issue, solutions would avoid sending large raw (packet-level) data from the UPF to the NWDAF.

One approach to reduce the signalling overhead of data transfer from UPF to NWDAF is to locally process the data in the UPF and send the pre-processed data to the NWDAF. Similar mechanisms are already specified in TS 23.288 including the Formatting/Processing instructions of DCCF and bulk data transfer for NWDAF, DCCF/MFAF, ADRF. However, they are not applicable for UP analytics due to the following reasons:

- Even if DCCF is used for data collection, the large volume of unprocessed data should be transferred from the UPF to the DCCF that cause significant signalling overhead.

- The capabilities are specified only for NWDAF, DCCF/MFAF, and ADRF. The UPF services do not support such capability to process data before exposing the consumer (e.g., NWDAF).

- The existing formatting/processing instruction in DCCF are generic but for the UP analytics, the data can be pre-processed in a way that facilitate ML model training and/or inference in the NWDAF, i.e., the preprocessing can be Analytics specific.

Based on these observations, this solution proposes an analytics derivation mechanism based on efficient data collection from the UPF; wherein, the NWDAF sends the data collection request to the UPF, which may also include local data preprocessing instructions that specify local data preprocessing requirements for the UPF to obtain the information required for the analytics. It should be noted that the local data preprocessing instructions may depends on the Analytics ID and corresponding performance metrics (e.g., for a certain level of model accuracy, aggregated information within a relatively large time-window may be sufficient).

The following list is an example of what may be included in the local data preprocessing instructions. Based on the analytics and considering factors such as the trade-off between signalling overhead vs. preprocessing load, etc., different kind of data preprocessing can be performed locally by the UPF following the preprocessing instructions from NWDAF. The NWDAF decide whether to provide instructions or not based on internal logic or operator policy.

- Data volume instructions: Instructions specifying the amount of data to be sent by UPF e.g. maximum amount of data, samples per data batch, total number of data batches, etc.

- Sampling instructions: Instructions how to sample the UPF data. Examples of sampling instructions include Sampling Method (Periodic sampling, Event-driven sampling, …), Sampling Interval, Sampling Ratio, etc.

- Statistical feature extraction instructions: Instructions how to statistically aggregate the UPF data. Examples of data aggregation methods include Feature type aggregation (Mean, median, variance, standard deviation, max, min, …), aggregation per QoS Flow or PDU level metrics (Average packet size, packet inter-arrival time, Flow duration, Packet loss rate, jitter, …), time window to compute the features

- Compression and/or encoding instructions: Instructions for setting up the Compression/Encoding algorithm settings of UPF data.

Editor’s Note: List of local data preprocessing instructions sent by the NWDAF to the UPF is FFS.

### 6.X.3	Procedures

The following figure shows the procedure of the proposed solution where the indirect (via SMF) subscription to the UPF events is used. The same mechanism of providing the local preprocessing instructions to the UPF is also applicable for direct subscription to the UPF event exposure and also when the SMF configures the UPF via PFCP reporting rules.

### Figure 6.X.3.1: User plane analytics derivation with UP data

1.	Analytics consumer requests or subscribes for a UP analytics ID to the NWDAF using either Nnwdaf_AnalyticsSubscription service or Nnwdaf_AnalyticsInfo.

2.	If the request is authorized and in order to provide the requested analytics, the NWDAF determine whether to collect data from UPF for derive the analytics. Based on the analytics ID, corresponding requested ML model performance metric, and NWDAF internal logic (considering the NWDAF load, …), the NWDAF may also prepare local data preprocessing instructions.

3.	If the NWDAF determined to collect the UP data in step 2, the NWDAF sends the Nsmf EventExposure_Subscribe with local data preprocessing instructions that indicates the information needed for the analytics e.g., time period and statistical characteristics.

4.	The SMF received the request from the NWDAF selects the PDU session(s) and UPF as data sources. The SMF sends the request to the UPF using Nupf EventExposure_Subscribe including the local data preprocessing instructions in step 3.

5.	The UPF collects the raw data and then processes the raw data based on the local preprocessing instructions received from the NWDAF to obtain the required information.

6.	The UPF sends the locally collected and preprocessed data to the NWDAF using Nupf_EventExposure_Notify.

7.	The NWDAF derives requested analytics using collected data.

8.	The NWDAF provides the analytics using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

### 6.1.4	Impacts on services, entities and interfaces

NWDAF

-	Generates the local preprocessing instructions based on analytics required from the analytics consumer.

-	Sends the local data preprocessing instructions to the SMF or UPF.

SMF

-	Sends the local data preprocessing instructions to the UPF.

UPF

-	Pre-processes the local data following the instructions from the data consumer.

-	Sends the pre-processed data to the SMF or data consumer.

********** End of Changes **********