3GPP TSG-SA WG2 Meeting #169	S2-2505523

Fukuoka, Japan, May 19 – 23, 2025													(revision of S2-2505141)

Source:	CATT

Title:	KI#2_new solution of user plane traffic pattern and behaviour detection and policy update based on NWDAF analysis

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a way for user plane traffic pattern and behaviour detection and policy update based on NWDAF analysis for KI#2

1. Introduction

In TR 23.700-04 clause 5.3 “Key Issue #2: AI/ML-assisted user plane traffic pattern and behaviour analysis to support efficient performance of User Plane”, it states:

Use of AI/ML can be studied to provide analytics on user plane traffic patterns and how to use the analytics to improve user plane performance.

In this key issue, the following aspects will be studied:

- How to provide analytics (e.g., enhancing existing in TS 23.288 [5] or introducing new) including the aspects of required inputs/outputs to improve performance of user plan.

- Which consumers can subscribe/request the analytics and how the consumers use the analytics to take actions to improve user plane performance.

- Whether and how to train ML models and perform the inference for the user plane analytics, considering solutions that minimize the load on Control Plane NFs.

As described in TR 23.700-04 clause 5.1.1 “Use Case #1: AI/ML-assisted user plane traffic pattern and behaviour analysis to support efficient performance of User Plane”, it states:

The UPF is responsible for managing substantial volumes of uplink and downlink traffic originating from UEs’ applications or application servers. It must be robust and resilient against abnormal data traffic, for examples, large amount of abnormal or malicious IP packets, flows, bursts, syn-ack signaling. Without actions to mitigate the impact of such potential abnormal or malicious traffic pattern in the user plane, the UPF may experience performance degradation which can lead to resource inefficiency. Use of AI/ML can be studied

- to provide analytics on user plane traffic patterns,

- to apply mitigation strategies using analytics, e.g., avoid abnormal or malicious traffic,

so as to improve efficient performance of user plane as well as UPF’s robustness.

Based on the above KI description and use case, it is proposed that the UPF can subscribe at NWDAF to be notified on traffic pattern and anomaly analytics information, and if specific traffic patterns or abnormal behaviours in the user plane are detected based on the analytics, the UPF can send an N4 Session Report to the SMF as described in clause 4.4.2 of TS 23.502. Based on the N4 Session Report, SMF may initiate SM policy Association Modification to the PCF. Then the PCF may update SM policy to the SMF, and SMF sends the N4 rules based the update SM policy to the UPF for improving the user plane efficiency and performance.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-04.

* * * Start of Change * * * *

6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | x |  |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x |  |  |  |
| #x |  | x | x |  |

* * * Next Change * * * *

6.x	Solution #X: User plane traffic pattern and anomaly detection and policy update based on NWDAF analysis

6.x.1	High-level principles

The solution proposes that the UPF can sends N4 Session Report to the SMF based on the subscribed traffic pattern and anomaly analytics information from the NWDAF, which then triggers SMF initiated SM Policy Association Modification procedures to update the SM policy and N4 rules (PDR, FAR), to improve efficiency and performance of user plane.

6.x.2 	Description

This solution addresses Key Issue #2 "AI/ML-assisted user plane traffic pattern and behaviour analysis to support efficient performance of User Plane" and Use Case #1.

In this solution, UPF subscribes to traffic pattern and anomaly analytics information from the NWDAF. If specific traffic patterns or abnormal behaviours in the user plane are detected based on the analytics, UPF performs user plane traffic handling optimization, e.g. data rate limitation, traffic filtering, etc., to improve user plane performance. If an "abnormal traffic detection" event reporting trigger is received from the SMF, UPF sends an N4 Session Report containing the traffic pattern and anomaly analytics information related to the identified abnormal traffic to SMF. Then SMF may decide to update N4 rules based on local policy (if dynamic PCC is not deployed) or updated SM policy after initiating SM policy Association Modification procedure towards the PCF (if dynamic PCC is deployed), and sends the updated N4 rules to the UPF for improving the user plane efficiency and performance.

6.x.3	Procedures

Figure 6.x.2-1: Procedure for NWDAF assisted user plane traffic pattern / anomaly detection and policy update

1.	The UPF subscribes to traffic pattern and anomaly analytics information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe service operation. The parameters that can be provided by the UPF are listed in clause 6.1.3 of TS 23.288 [5].When a subscription to analytics information is received, the NWDAF determines whether triggering new data collection is needed.

2.	The NWDAF collects input data and notifies the UPF with the traffic pattern and anomaly analytics information by invoking Nnwdaf_AnalyticsSubscription_Notify service operation, based on the request from the UPF. The input data from UPF can obtain packet information, including source address, destination address, port number, protocol type, size, direction, etc. The traffic pattern and anomaly analytics information contains packet information and traffic analysis results. The analysis results include traffic classification (e.g. by service type), abnormal detection information and traffic prediction information (e.g. data traffic volume, data traffic rate, data transmission time interval, peak time), etc.

Editor's note:	It is FFS how the NWDAF derives the analytics.

Editor's note:	It is FFS why the UPF is the analytics consumer and forwards related information to the SMF (that in turn forwards it to PCF), rather than SMF and/or PCF being analytics consumers.

3.	Based on the traffic pattern and anomaly analytics information, the UPF performs user plane traffic handling optimization, e.g. data rate limitation (i.e. throttling suspicious data flows), traffic filtering (i.e. dropping malicious packets), etc., to improve user plane performance.

NOTE:	How the UPF uses the traffic pattern and anomaly analytics information to optimize user plane traffic handling is up to implementation.

4a/4b.	UPF sends the N4 Session Report to SMF if the "abnormal traffic detection" event reporting trigger is received from the SMF, and the report contains traffic pattern and anomaly analytics information related to the identified abnormal traffic.

5a/5b.	If dynamic PCC is not deployed, the SMF may decide to update N4 rules, e.g. FAR (Forwarding Action Rule), towards the UPF to instruct the UPF to drop the identified abnormal packets, based on local policy and the received traffic pattern and anomaly analytics information. The SMF initiates N4 session modification towards the UPF to provide the update N4 rules.

If dynamic PCC is deployed, the following steps 6-8 are performed.

6a.	SMF invokes an Npcf_SMPolicyControl_Update request to the PCF, and the request contains traffic pattern and anomaly analytics information related to the identified abnormal traffic.

6b.	PCF decides to update the SM Policy, e.g. adding new traffic gating rules to discard the identified abnormal traffic, based on the received traffic pattern and anomaly analytics information from the SMF, and sends an Npcf_SMPolicyControl_Update response with updated policy information to the SMF.

7a/7b.	SMF sends the N4 session modification request message to the UPF to update N4 rules based on the received SM policy from the PCF.

8.	UPF handles the user plane traffic based on the updated N4 rules.

6.x.4	Impacts on services, entities and interfaces

UPF:

-	Subscribes to traffic pattern and anomaly analytics information from NWDAF.

-	Sends an N4 Session Report, which contains traffic pattern and anomaly analytics information related to the identified abnormal traffic, to SMF based on "abnormal traffic detection" event reporting trigger.

NWDAF:

-	Collects input data and generates traffic pattern and anomaly analytics information base on  the subscription request from the UPF.

SMF:

-	Sends "abnormal traffic detection" event reporting trigger to the UPF, and receives the traffic pattern and anomaly analytics information related to the identified abnormal traffic in N4 Session Report from the UPF.

-	Update N4 rules to the UPF based on the traffic pattern and anomaly analytics information related to the identified abnormal traffic.

PCF:

-	Updates one or more SM policies based on the traffic pattern and anomaly analytics information related to the identified abnormal traffic.

* * * End of Change * * * *